3GPP TSG-RAN WG3 Meeting #128	R3-253312

Malta, MT, May 19 – 23, 2025

Agenda item:	 10.2

Source:		Lenovo

Title:		(TP for SON BLCR for 38.423) Discussion on MRO for CHO with candidate SCG(s) and subsequent CPAC

Document for:		Discussion and Decision

1	Introduction

In RAN3#125, we agreed that:

MRO for CHO with Candidate SCG(s)

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

The RLF report includes: The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

MRO for S-CPAC:

Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.

In RAN3#125bis, we further agreed that:

MRO for CHO with Candidate SCG(s)

RLF report is enhanced to including

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

In RAN3#126, we agreed that:

MRO for CHO with Candidate SCG(s)

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

The failure types defined for CHO in TS38.300 and for CPAC defined in TS37.340 are used as baseline. Clarification or amendment could be made on top of that if needed.

In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization.

SCGFailureInfomation handling for case 1b)/2b)/3b)/9b) and case 7b)/8b):

The serving MN which receives SCGFailureInformation from the UE performs the initial analysis.

The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g for case 3b).

Whether the above mechanism works depends on RAN2 progress on SCGFailureInfomation.

In RAN3#127, we agreed that:

MRO for CHO with Candidate SCG(s)

In case of too late CHO execution, reusing Handover Report message from the last serving MN to the relevant candidate MN.

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

MRO for S-CPAC

The forwarding mechanism in BLCR for TS37.340 need to be refined.

In RAN3#127bis, we further agreed that:

MRO for CHO with Candidate SCG(s)

No additional information is needed in handover report message.

Check stage 3 TP on the Target cell CGI related in handover report message.

One SCGFailureInfomation is included in SCG Failure Indication message, no need aggregation.

Respective MN in stage2 is at least a candidate MN.

Information needed in the SCG Failure Indication message: The Xn UE AP ID in the receiving node.

Additional information reported from the UE in SCGFailureInformation: The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the SCG failure is encountered.

No support of near failure case in R19.

Whether Handover Report message is needed for case 7a?

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

MRO for S-CPAC

The description on S-CPAC information forwarding is technically correct. Cleanup of the text is needed.

Whether to enhance UHI in case of S-CPAC? The motivation and benefits need to be clarified.

In this paper, we would further discuss the details of MRO for subsequent CPAC and CHO with candidate SCG(s).

2	Discussion

  - 2.1 MRO for CHO with candidate SCG(s)

Whether Handover Report message is needed for case 7a?

For case 7a as summarized in [1], both CHO and CPAC execution conditions are met, but CHO execution to the target PCell fails. In this case, the failure is a type of “HOF”, RLF report should be sent to the last serving MN for MRO analysis, there is no specific need for the last serving MN to send the Handover Report message to the relevant candidate MN.

Proposal 1: For case 7a, there is no need for the last serving MN to send the Handover Report message to the relevant candidate MN.

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

For the issue about whether CPC failure type is needed in the SCG Failure Indication message in addition to SCGFailureInformation, since SCGFailureInformation from the UE includes necessary information, after the respective MN receives the SCG Failure Indication message, it can perform failure type analysis and do corresponding optimisation, no matter the SCG Failure Indication is sent from the candidate MN to the source MN, or from the source MN to the candidate MN. Therefore, it is not needed to include CPC failure type in the SCG Failure Indication message.

Proposal 2: No need to include CPC failure type in the SCG Failure Indication message.

For the issue about whether to include suitable PCell ID in the SCG Failure Indication message in addition to SCGFailureInformation, we can see that RAN2#129 meeting agreed to enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any. It means that a list of candidate PCells or candidate PSCells which fulfilled execution conditions would be reported by the UE in the SCGFailureInformation message. When the last serving MN receives the SCGFailureInformation message from UE, it can forward it to the respective MN (e.g. in which the suitable PCell belongs to) via the SCG Failure Indication message, then the respective MN can perform failure type analysis to decide the candidate PCell/PSCell which needs to be optimized among the list of candidate PCells/PSCells. We don’t see the need to include suitable PCell ID in the SCG Failure Indication message.

Proposal 3: No need to include suitable PCell ID in the SCG Failure Indication message.

  - 2.2 MRO for subsequent CPAC

Whether to enhance UHI in case of S-CPAC?

In R18, UHI mechanism for legacy CPAC procedure is specified as “When the target SN receives the SCG UHI from the MN via SN Addition Request message for CPC, the target SN updates the time UE stayed in cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target SN. The updated value of the time UE stayed in the latest PSCell is equal to the value received from the MN via the SN Addition Request message plus the time from receiving SN Addition Request message from the MN to receiving SN Reconfiguration Complete from the MN”.

Similar as legacy CPAC, UHI mechanism is beneficial to avoid PSCell change ping-pong issue in S-CPAC. However, UHI mechanism for legacy CPAC procedure can’t totally work well for a subsequent CPAC procedure, since target SN of following subsequent CPC can’t get the latest and valid SCG UHI. Take the target SN of first following subsequent CPC for an example, in preparation phase of the subsequent CPAC procedure, the target SN of first following subsequent CPC only gets UHI associated with the source SN of the initial CPAC (i.e. the initiating SN at S-CPAC preparation), but actually, before first following subsequent CPC phase, initial CPAC execution is successfully completed, but the target SN of first following subsequent CPC can’t know the UHI associated with the source SN of the first following subsequent CPC. Therefore, SCG UHI enhancement is needed for S-CPAC.

Proposal 4: SCG UHI enhancement is needed for S-CPAC.

To solve the issue, the latest SCG UHI should be informed to the new source SN of following subsequent CPC, so, it is needed to include the latest SCG UHI in SN Reconfiguration Complete message.

Proposal 5: Include SCG UHI in SN Reconfiguration Complete message.

3	Conclusion

In this paper, the details of MRO for CHO with candidate SCG(s) and subsequent CPAC are discussed. We have the following proposals:

MRO for CHO with candidate SCG(s)

Proposal 1: For case 7a, there is no need for the last serving MN to send the Handover Report message to the relevant candidate MN.

Proposal 2: No need to include CPC failure type in the SCG Failure Indication message.

Proposal 3: No need to include suitable PCell ID in the SCG Failure Indication message.

MRO for subsequent CPAC

Proposal 4: SCG UHI enhancement is needed for S-CPAC.

Proposal 5: Include SCG UHI in SN Reconfiguration Complete message.

Reference

[1] R3-251749, Failure scenarios on MRO for CHO with candidate SCGs, Samsung, Cybercore, Lenovo

Annex for TS38.423

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

### 8.3.2	S-NG-RAN node Reconfiguration Completion

#### 8.3.2.1	General

The purpose of the S-NG-RAN node Reconfiguration Completion procedure is to provide information to the S-NG-RAN node whether the requested configuration was successfully applied by the UE.

The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: S-NG-RAN node Reconfiguration Complete procedure, successful operation.

The M-NG-RAN node initiates the procedure by sending the S-NODE RECONFIGURATION COMPLETE message to the S-NG-RAN node.

The S-NODE RECONFIGURATION COMPLETE message may contain information that

-	either the UE has successfully applied the configuration requested by the S-NG-RAN node. The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

-	or the configuration requested by the S-NG-RAN node has been rejected. The M-NG-RAN node shall provide information with sufficient precision in the included Cause IE to enable the S-NG-RAN node to know the reason for an unsuccessful reconfiguration. The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

Upon reception of the S-NODE RECONFIGURATION COMPLETE message the S-NG-RAN node shall stop the timer TXnDCoverall if TXnDCoverall is running.

If the SK-counter IE is included in the S-NODE RECONFIGURATION COMPLETE message, the S-NG-RAN node shall, if supported, use it to choose S-KSN as specified in TS 33.501 [28]

If the SCG UE History Information IE is included in the S-NODE RECONFIGURATION COMPLETE message, the S-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

#### 8.3.2.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.4	S-NODE RECONFIGURATION COMPLETE

This message is sent by the M-NG-RAN node to the S-NG-RAN node to indicate whether the configuration requested by the S-NG-RAN node was applied by the UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Response Information | M |  |  |  | YES | ignore |
| >CHOICE Response Type | M |  |  |  | – |  |
| >>Configuration successfully applied |  |  |  |  |  |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10] or the RRCConnectionReconfigurationComplete message as defined in subclause 6.2.2 of TS 36.331 [14]. | – |  |
| >>>SK-counter | O |  | INTEGER (0..65535) |  | YES | ignore |
| >>Configuration rejected by the M-NG-RAN node |  |  |  |  |  |  |
| >>>Cause | M |  | 9.2.3.2 |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in as defined in subclause 11.2.2 of TS 38.331 [10]. | – |  |
| SCG UE History Information | O |  |  | 9.2.3.151 | YES | ignore |