3GPP TSG-RAN WG3 Meeting #128	R3-253349

Malta, MT, 19 - 23 May, 2025

Agenda item:	21.3

Source: 	Huawei

Title: 	(TP for XR BL CRs) Discussion on miscellaneous issues for XR

Document for:	Discussion and Decision

# 1. Introduction

There are some miscellaneous issues remained open for XR, such as whether to include the BSSize and TTNB in the PDU Session Container or the PDU Set Information Container, and whether to support non-homogeneous deployment for available data rate report. In this contribution, we will discuss these open issues and give our views on the potential enhancements and spec impacts.

# 2. Discussion

## 2.1 Dynamic characteristic information

- 2.1.1 Which container to include BSSize and TTNB

During the last meeting, RAN3 agreed to introduce BSSize and TTNB in NG-U/F1-U/Xn-U. The TP in R3-252488 [1] added the BSSize and TTNB into the PDU Session Container for NG-U and the TP in R3-252388 [2] added them into the NR RAN Container for Xn-U and F1-U. Both of the TPs are agreed. However, there are editor’s notes which states FFS on whether to capture the BSSize and TTNB in the PDU Set Information Container.

------ Excerption from R3-252488 ------ Editor’s Note: FFS on whether TTNB and/or BSSize related information may be removed in this clause and introduced into other frame (e.g. DL PDU SET INFORMATION).

------ Excerption from R3-252388 ------ Editor Note: FFS whether to capture the Burst Size and Time To Next Burst related information in the DL USER DATA frame.

As we clarified in the previous contribution in R3-251857 [3], it is aligned with SA4’s design not to include the BSSize and TTNB in the PDU Set Information Container as SA4 used different RTP extension headers to carry the BSSize/TTNB and the PDU Set information. Separating BSSize and TTNB from the PDU Set information can provide more flexibility to the gNB when utilizing the information. The gNB can benefit from the newly introduced dynamic characteristic information even if it does not support PDU Set handling.

Thus, RAN3 should confirm that the dynamic characteristic information, i.e., BSSize and TTNB, are not included in the PDU Set Information Container. For NG-U, the information should be introduced in the PDU Session Container. While for Xn-U and F1-U, the information should be introduced in the NR RAN Container. The TP for TS 38.415 in R3-252488 and the TP for TS 38.425 in R3-252388 should be adopted and the editor’s notes should be removed. Corresponding TPs are provided in annex A.

Proposal 1: RAN3 to agree adding BSSize and TTNB in the PDU Session Container for NG-U and in the NR RAN Container for Xn-U/F1-U, and remove the editor’s notes on FFS whether to move the information to the PDU Set Information Container.

- 2.1.2 Forwarding BSSize and TTNB in DC case

Moreover, some additional handling for the dynamic characteristic information should be considered in case of DC to allow both the MN and SN be aware of the information. Similar issue has been discussed for EoDB and it was agreed that the PDCP hosting node provides the EoDB to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the data burst, as implemented in the BL CR in R3-252533 [4] for TS 37.340.

------ Excerption from R3-252533 ------ When the PDCP hosting gNB receives the End of Data Burst Indication from the UPF, it always provides the End of Data Burst Indication to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the Data Burst.

The above solution can be applied to BSSize and TTNB as well. When the PDCP hosting node receives BSSize or TTNB from the UPF, it should forward the information to the corresponding node, e.g., the gNB-DU and the gNB for split, along with the forwarded packets. Specially, when the data burst is split between the MN and SN, the BSSize provided by the PDCP hosting node may be different from the one provided by the UPF. This can be left to the NW implementation considering how the data burst is split actually. TP for TS 37.340 is provided in annex A.

Proposal 2: The PDCP hosting node provides the BSSize and TTNB to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the Data Burst.

## 2.2 Available data rate report

According to SA2’s conclusion, RAN3 agreed to introduce thresholds and direction in NGAP/XnAP/F1AP/E1AP to trigger NG-RAN monitor available data rate via F1-U and NG-U for QoS flows. One remaining issue is whether and how to support non-homogeneous deployment for the feature.

One view is that the issue is similar to R18 PDU Set handling, i.e., the gNBs may have different capabilities on whether supporting available data rate report, and the CN needs to get aware of it by sending request to the NG-RAN during the PDU session related procedures and receiving feedback on whether the feature is supported.

However, according to TS 23.501 [6], available bitrate report is described in the clause of QoS monitoring, which means it follows the general mechanism as legacy QoS monitoring. And, as described in clause 5.45.1 in TS 23.501, the AMF can know whether NG-RANs in an area support QoS monitoring based on OAM configuration. In addition, the AMF can further inform the SMF about the support situation so that the SMF can decide whether QoS monitoring is possible or not.

------ Excerption from TS 23.501 ------ ## 5.45 QoS Monitoring ### 5.45.1 General ------ unrelated part skipped ------ Based on configuration, the AMF may know whether the QoS monitoring feature (i.e. each of Packet delay monitoring and/or Congestion monitoring) is supported by all NG RAN nodes in an area. The AMF shall, when configured, inform the SMF about the support of the QoS monitoring feature by sending the parameter NG RAN QoS monitoring capability during PDU Session establishment, PDU Session modification, Service request and UE mobility procedures. The SMF determines whether QoS monitoring is possible or not for the PDU Session based on whether the NG RAN QoS monitoring capability is received (or not) from the AMF and the QoS monitoring capability of the UPF. QoS monitoring is possible if the SMF has received the NG RAN QoS monitoring capability in the last message received from AMF (i.e. Nsmf_PDUSession_CreateSMContext or Nsmf_PDUSession_UpdateSMContext service operation) and the UPF also supports the corresponding QoS monitoring capability. Otherwise, QoS monitoring is not possible for the PDU Session for the respective QoS monitoring parameter(s). The SMF may need to notify the PCF when the support for QoS monitoring has changed as described in clause 6.1.3.5 of TS 23.503 [45] also indicating the affected QoS monitoring parameter(s). ------ unrelated part skipped ------ ### 5.45.6 Available bitrate monitoring The QoS Monitoring for available bitrate allows the measurement and report of the UL and/or DL available bitrate for a GBR QoS Flow without Alternative QoS Profiles at the NG-RAN. The available bitrate refers to the bitrate that NG-RAN is able to provide for a QoS Flow within a duration. The duration is based on RAN implementation. NOTE 1: How the NG-RAN measures the available bitrate is NG-RAN implementation specific. NOTE 2: The available bitrate can be between the GFBR and the MFBR. The available bitrate can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without Alternative QoS Profiles. For the reporting of the available bitrate, periodic reporting is not applied and only threshold-based reporting applies. Based on the PCC rule containing available bitrate as QoS monitoring parameter and Reporting threshold(s) per direction in the QoS Monitoring policy (as described in clause 6.1.3.27.1 of TS 23.503 [45]), the SMF configures the NG-RAN to measure and report the available bitrate by sending QoS monitoring request including the threshold(s). Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide matches or exceeds indicated Reporting threshold(s), the NG-RAN reports the UL and/or DL available bitrate information of the highest fulfilled threshold to the PSA UPF via the GTP-U header of UL packets. NOTE 3: The NG-RAN is expected to immediately report the UL and/or DL available bitrate when it matches or exceeds a lower threshold value and report the UL and/or DL available bitrate when it matches or exceeds a higher threshold value with some hysteresis. The SMF may configure the PSA UPF to perform QoS monitoring for available bitrate as described in clause 5.8.2.18. For the reporting of the available bitrate via the PSA UPF, only event-based reporting applies in the PSA UPF. The Reporting threshold(s) in the PCC rule are not sent to UPF. In UPF, the event for the reporting is the reception of available bitrate information from NG-RAN. According to the QoS Monitoring request for the available bitrate from the SMF, the PSA UPF monitors and exposes the available bitrate information reported from NG-RAN to the target NF as instructed. During handover, the QoS Monitoring configuration is provided to the Target NG-RAN per QoS Flow in the QoS Monitoring configuration.

Since available data rate report is considered as a part of QoS monitoring, it should follow the same way to get the information about whether the feature is supported at NG-RAN. There is no need to introduce an additional indication during the NGAP PDU session related procedures.

Proposal 3: RAN3 to not specify signalling in NGAP to indicate whether the NG-RAN supports available data rate report.

## 2.3 RAN2 RLC enhancements

During the R19 XR discussions, RAN2 has introduced some enhancements for the RLC AM mode, including to avoid unnecessary retransmission and to perform autonomous retransmission. In this section, we would like to investigate  these enhancements from RAN3 perspective to check the potential impacts to RAN3.

### 2.3.1 Avoid unnecessary retransmission

First, to avoid unnecessary retransmission for the packets with stringent delay budget, RAN2 agreed to stop transmission and retransmission for an SDU when the TX RLC entity receives a discard indication of the SDU from the PDCP. The related agreement is excerpted below.

------ Excerption from R2-129 report ------ - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment.

In the latest RLC running CR in R2-2502162 [7], the above enhancement was implemented as below.

------ Excerption from R2-2502162 ------ If stopReTxObsoleteSDU is set to enabled, when receiving a discard indication for an RLC SDU with SN = x from the upper layer (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment for transmission or retransmission. Editor’s Note: FFS on the term, whether it should be “obsolete”, or “outdated”, or “discard”. Same as below.

It can be seen that the TX RLC entity perform the stop of transmission/retransmission based on the reception of the discard indication, which is sent from the PDCP layer when the PDCP discard timer expires for the corresponding SDU, as described in TS 38.323 [8]. There is no new method introduced to inform the RLC entity about the expiry of the SDU.

------ Excerption from TS 38.323 ------ ## 5.3 SDU discard When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall: - if pdu-SetDiscard is configured: - discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs; NOTE 1: PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set. - else: - discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

For DL transmission, the gNB-DU should stop the transmission and retransmission based on the indication from the gNB-CU since the PDCP entity on the gNB-CU is aware of whether the packet is outdated or not. Currently, there is already F1-U indications indicating the PDCP SN of the discard packets, which are captured in TS 38.425 [9]. The related specification is copied below.

------ Excerption from TS 38.425 ------ #### 5.5.2.1 DL USER DATA (PDU Type 0) This frame format is defined e.g. to allow the corresponding node to detect lost NR-U packets and may be associated with the transfer of a Downlink PDCP PDU. The following shows the respective DL USER DATA frame. NOTE 1: All information elements defined in Figure 5.5.2.1-1 are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP. Bits; Number of Octets. 7; 6; 5; 4; 3; 2; 1; 0. PDU Type (=0); Spare; DL Discard Blocks; DL Flush; Report polling; 1. Spare; Request OutofSeq Report; Report Delivered; User data existence flag; Assistance Info. Report Polling Flag; Retransmission flag; 1. NR-U Sequence Number; 3. DL discard NR PDCP PDU SN; 0 or 3. DL discard Number of blocks; 0 or 1. DL discard NR PDCP PDU SN start (first block); 0 or 3. Discarded Block size (first block); 0 or 1. …. DL discard NR PDCP PDU SN start (last block); 0 or 3. Discarded Block size (last block); 0 or 1. DL report NR PDCP PDU SN; 0 or 3. Padding; 0-3. Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format ------ unchanged part skipped ------ #### 5.5.3.8 DL discard NR PDCP PDU SN Description: This parameter indicates the downlink NR discard PDCP PDU sequence number up to and including which all the NR PDCP PDUs should be discarded. Value range: {0..218-1}. Field length: 3 octets. #### 5.5.3.9 DL Discard Blocks Description: This parameter indicates the presence of DL discard Number of blocks, DL discard NR PDCP PDU SN start and Discarded Block size. Value range: {0= DL discard Number of blocks, DL discard NR PDCP PDU SN start and Discarded Block size not present, 1= DL discard Number of blocks, DL discard NR PDCP PDU SN start and Discarded Block size present}. Field length: 1 bit. #### 5.5.3.10 DL discard NR PDCP PDU SN start Description: This parameter indicates the starting SN of a downlink NR PDCP PDU block to be discarded. Value range: {0..218-1}. Field length: 3 octets. #### 5.5.3.11 DL discard Number of blocks Description: This parameter indicates the number of NR PDCP PDU blocks to be discarded. Value range: {1..244}. Field length: 1 octet. #### 5.5.3.12 Discarded Block size Description: This parameter indicates the number of NR PDCP PDUs counted from the starting SN to be discarded. Value range: {1..255}. Field length: 1 octet.

The above indications work as the “discard indication” mentioned in the RAN2 specifications for DL transmission in CU-DU split architecture. Considering the R19 RLC enhancement to avoid unnecessary retransmission, the current F1-U indications can be reused and they are sufficient for the RLC layer to perform the stop the unnecessary (re)transmission.

Proposal 4: RAN3 to reuse the discard indication in F1-U to stop the transmission or retransmission for the outdated SDUs.

### 2.3.2 Autonomous retransmission

RAN2 also discussed in Rel-19 on how to perform RLC autonomous retransmission based on the remaining time of the packets to improve the reliability and guarantee the stringent delay budget. Although RAN2 only focus on the UL transmission, we think RAN3 can look at how to support the symmetrical mechanism for DL.

As agreed by RAN2, the autonomous retransmission is triggered when the remaining time of an RLC SDU falls below a specified threshold, and the remaining time of the SDU is determined based on its PDCP discard timer.

------ Excerption from R2-129 report ------ - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC

------ Excerption from R2-129bis report ------ - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission

For UL transmission, the PDCP layer will send an indication to the RLC layer to indicate that the remaining time of the SDU falls below the threshold specified for autonomous retransmission to trigger the RLC layer for autonomous retransmission.

For DL transmission, RAN3 can support the same method by introducing an indication from the gNB-CU to the gNB-DU to support DL autonomous retransmission in CU-DU split architecture. For example, the gNB-CU can indicate the PDCP SNs of the SDUs whose remaining time are below the threshold in the GTP-U header of the DL packets. Upon receiving the indication, the gNB-DU can perform autonomous retransmission accordingly for the corresponding SDUs.

Proposal 5: RAN3 to introduce indication from the gNB-CU to the gNB-DU to indicate the SDUs with remaining time less than the threshold to support DL autonomous retransmission.

# 3. Conclusion

In this contribution, we analyzed the specification impacts of miscellaneous issues for XR. The following observations and proposals were made:

Dynamic characteristic information:

Proposal 1: RAN3 to agree adding BSSize and TTNB in the PDU Session Container for NG-U and in the NR RAN Container for Xn-U/F1-U, and remove the editor’s notes on FFS whether to move the information to the PDU Set Information Container.

Proposal 2: The PDCP hosting node provides the BSSize and TTNB to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the Data Burst.

Available data rate report:

Proposal 3: RAN3 to not specify signalling in NGAP to indicate whether the NG-RAN supports available data rate report.

RAN2 RLC enhancements:

Proposal 4: RAN3 to reuse the discard indication in F1-U to stop the transmission or retransmission for the outdated SDUs.

Proposal 5: RAN3 to introduce indication from the gNB-CU to the gNB-DU to indicate the SDUs with remaining time less than the threshold to support DL autonomous retransmission.

# 4. Reference

[1]. R3-252488, China Telecom, Nokia, Nokia Shanghai Bell, ZTE, Huawei, CMCC, Ericsson, Samsung, (TP to TS 38.415) enhancement for BSSize, TTNB and available data rate.

[2]. R3-252388, China Telecom, Nokia, Nokia Shanghai Bell, ZTE, Huawei, (TP to TS 38.425) enhancement for BSSize, TTNB and available data rate.

[3]. R3-251857, Huawei, (TP for XR BL CR) Discussion on the miscellaneous issues for XR.

[4]. R3-252533, ZTE Corporation, Nokia, Nokia Shanghai Bell, Qualcomm Inc, Lenovo, Huawei, Ericsson, CATT, CMCC, Samsung, Xiaomi, NEC, China Telecom, (BL CR to 37.340) Support of XR enhancements.

[5]. S2-2504417, InterDigital, Available Bitrate Reporting when the Serving RAN Node Changes.

[6]. 3GPP TS 23.501, "System architecture for the 5G System (5GS); Stage 2".

[7]. R2-2502162, vivo, Running RLC CR for XR.

[8]. 3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) protocol specification".

[9]. 3GPP TS 38.425: "NR user plane protocol".

# Annex A - TPs for BSSize and TTNB

## Annex A.1 – TP for BL CR for TS 38.415

CHANGE STARTS

#### 5.5.3.ca	BSSI (Burst Size Indicator)

Description: This parameter indicates the presence of Burst Size (BSSize).

Value range: {0= BSSize not present, 1= BSSize present}.

Field length: 1 bit.

#### 5.5.3.db	Burst Size (BSSize)

Description: This parameter indicates the total size of the burst.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.e	TTNBI (TTNB Indicator)

Description: This parameter indicates the presence of Time To Next Burst (TTNB).

Value range: {0= TTNB not present, 1= TTNB present}.

Field length: 1 bit.

#### 5.5.5.f	Time To Next Burst (TTNB)

Description: This parameter indicates the the approximate time in tenth of milliseconds to the next burst.

Value range: {0..216-1}.

Field length: 2 octets.

CHANGE ENDS

### Annex A.2 – TP for BL CR for TS 38.425

CHANGE STARTS

#### 5.5.3.c	BSSI (Burst Size Indicator)

Description: This parameter indicates the presence of Burst Size (BSSize).

Value range: {0= BSSize not present, 1= BSSize present}.

Field length: 1 bit.

#### 5.5.3.d	Burst Size (BSSize)

Description: This parameter indicates the total size of the burst.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.e	TTNBI (TTNB Indicator)

Description: This parameter indicates the presence of Time To Next Burst (TTNB).

Value range: {0= TTNB not present, 1= TTNB present}.

Field length: 1 bit.

#### 5.5.3.f	Time To Next Burst (TTNB)

Description: This parameter indicates the the approximate time in tenth of milliseconds to the next burst.

Value range: {0..216-1}.

Field length: 2 octets.

CHANGE ENDS

### Annex A.3 – TP for BL CR for TS 37.340

CHANGE STARTS

## 13.x	eXtended Reality Services

### 13.x.1	Overview

The eXtended Reality Services as described in TS 38.300 [3] is extended to address the NR-DC operation.

### 13.x.2	Awareness

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the PDU Set QoS Parameters as part of the QoS profile to the SN to enable PDU Set based QoS handling.

The UE may report uplink assistance information (jitter range, burst arrival time, UL data burst periodicity) per QoS flow via UE Assistance Information to the MN or the SN as configured. If the SN receives the burst arrival time, the SN may use it by considering the SFN offset of the MN.

When the PDCP hosting gNB receives the End of Data Burst Indication from the UPF, it always provides the End of Data Burst Indication to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the Data Burst.

When the PDCP hosting gNB receives the Data Burst Size Indication or the Time To Next Burst Indication from the UPF, it always provides the Data Burst Size Indication or the Time To Next Burst Indication to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the Data Burst.

CHANGE ENDS