3GPP TSG-RAN WG3 Meeting #128	R3-253370

Malta, MT, 19th – 23rd May 2025

Title:	Discussion on AIML based Coverage and Capacity Optimization

Source:				China Telecom

Agenda item:		11.3

Document for:		Discussion and Decision

1. Introduction

In RAN3#127bis meeting, further agreements were achieved as below:

The maximum value of the Time interval for predicted CCO issue and future CCO state is 60s.

Legacy UE performance measurement metrics can be reused for CCO.

However, the following issues are still FFS:

For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state?

- Option1: no need

- Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

- Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

Whether/what additional UE performance measurement metrics is needed?

Timing information for predicted CCO issue is NOT needed to exchange over Xn? Whether the predicted CCO issue and /or future CCO state can be updated over Xn?

In this contribution, we further discuss the abovementioned issues.

2. Discussion

2.1 Future CCO state over F1 at receiving gNB

According to the latest agreed TP, the CU can send to the DU a Predicted CCO Assistance Information which includes the Predicted CCO issue, the Predicted Affected Cells and Beams, and the Time for Predicted CCO Issue. The DU can send to the CU a Future Coverage Modification Notification which includes the Future cell Coverage State, Future Coverage Modification Cause, and Time for Future Coverage Modification Notification, etc. The Predicted CCO issue, Predicted Affected Cells and Beams, Future Cell Coverage State and Time for Future Coverage State are also sent via Xn from CU1 (gNB1) to CU2 (gNB2).

In legacy CCO solution, DU2 derives the CCO state based on UE measurements. CU2 does not send to DU2 the CCO state generated by DU1. However, for a predicted CCO issue, DU2 cannot deduce it from measurements because DU1 has not applied the future CCO state. Therefore, to assist DU2 to generate future CCO state, CU2 needs to send to DU2 the information from CU1, there can be two options:

- Opton1: CU2 directly forwards the received future CCO state of neighbor cells to DU2.

- Option2: CU2 generates a recommended future CCO state and send to DU2.

Considering CU is more knowledgeable on the neighbour node status, CU2 can generate the recommended future CCO state by AI/ML-based CCO model based on the received future CCO state and predicted CCO issue, also together with other information. Therefore, we think CU2 can generate a recommended future CCO state and send it to DU2 as assistance information.

Proposal 1: For the receiving side, gNB-CU generates a recommend future CCO state and sends to gNB-DU as assistance information to derive future CCO state.

2.2 Timing information for AI/ML-based CCO

RAN3 agreed to transfer the Time for Future Coverage Modification IE (i.e. time information for future CCO state) within the NG-RAN CONFIGURATION UPDATE message. However, whether to send time information for the coverage modification cause which reflects the predicted CCO issue over Xn needs further discussion.

From our side, providing time information for predicted CCO issues over Xn is essential for proactive network management. For example, consider a scenario where gNB1 predicts a capacity overload issue at 5 PM, with offloading to gNB2 starting at 4:30 PM. Knowing 5 PM, gNB2 can ensure it has sufficient resources ready at 5 PM to handle the offloaded users, especially if the offloading process takes time to complete and some strain might still occur at 5 PM if not fully mitigated. If gNB2 only knows the future CCO state will be applied at 4:30 PM, it might assume that the issue is fully handled by then, but if the offloading isn't complete and the issue still occurs at 5 PM, gNB2 might be unprepared for residual effects, risking degraded service.

Proposal 2: RAN3 to agree that timing information for the predicted CCO issue can be transferred over Xn.

Proposal 3: Introduce the Time for coverage modification cause IE within the NG-RAN CONFIGURATION UPDATE message.

3. Conclusion

In this contribution, we further discuss AI/ML based Coverage and Capacity Optimization (CCO). We have the following proposal:

Proposal 1: For the receiving side, gNB-CU generates a recommend future CCO state and sends to gNB-DU as assistance information to derive future CCO state.

Proposal 2: RAN3 to agree that timing information for the predicted CCO issue can be transferred over Xn.

Proposal 3: Introduce the Time for coverage modification cause IE within the NG-RAN CONFIGURATION UPDATE message.

4. Reference

- 3GPP RAN3#125b Chair’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chair’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#127 Chair’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- 3GPP RAN3#127bis Chair’s notes (RAN3_127bis_agenda_20250411_1400_EOM.doc)