3GPP TSG-RAN3 Meeting #128	R3-253376

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	16.2

Source:	Xiaomi

Title:	(TP for TS 38.300 and TS 38.413) Device context handling

Document for:	Discussion

1	Introduction

In this paper, we would like to discuss how to handle the device context in the gNB for command after inventory, based on the related progress made in RAN3 at the last meeting and RAN2 agreements.

About Command after inventory - Introduce a follow-on command indication in Inventory Request Transfer IE, in case of Command after inventory. - For command related NGAP messages, the Correlation Identifier IE is the same one as in the related Inventory procedure. - WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure. - WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages. - Include the AIOTF Identifier IE outside of the Command related Transfer IEs in all the command related messages. - The Command Request Transfer IE, also includes the following: - An A-IoT NAS PDU - Command Assistance Information: (Estimate of Expected D2R Message Size) - It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…) - Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory. - In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device. - The Device Association is included in the Command Request Transfer IE and Command Response Transfer IE. It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

RAN2 has the following agreements related AS ID

Agreements

1	AS ID is applied for Inventory + command case;

2	AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed.

3	For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits.

4	Do not specify the reader behaviour on how exactly the ASID is generated.

5	The device releases the AS ID upon power off (no stage 3 specification impact);

6	The device only keeps one AS ID at a time.

7	For CFRA, command message is used for AS ID assignment

8	For CBRA, Msg 2 is used for AS ID assignment

9	The device releases the AS ID at least:

- upon receiving Paging with new transaction id for that device, i.e. different session/service

- when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA)

- FFS other cases for release ASID to avoid keeping it indefinitely.

2	Discussion

### Overall call flow for inventory and command operation

The following is the figure shown how the gNB handle the device context in case of inventory and command operation.

### Content of the Device context

At step 1, if there’s follow-on command in the inventory request, after the gNB performs the inventory procedure and establish communication with an A-IoT device, it will allocate a RAN NGAP device ID for the device and include the RAN NGAP device ID in the inventory report associated with the A-IoT NAS. At step 3, the A-IoT device may be assigned with an AS ID, which will be used for the R2D reception and D2R scheduling between the gNB reader and the A-IoT device, the gNB needs to maintain the relations between the RAN NGAP device ID and the associated AS ID, in addition, as the RAN NGAP device ID is per session, the gNB should also maintain the correlation ID in the inventory request as the A-IoT device context.

Based on the above, the device context for a specific device during an A-IoT session includes the following:

- Correlation ID, used to identify the A-IoT session

- RAN NGAP device ID, used for the communication over NG between gNB and CN for the A-IoT device

- AS ID, used for the communication over A-IoT radio between gNB and the A-IoT device

Proposal 1, gNB should maintain the following device context for a specific A-IoT device in case of inventory and command operation

- correlation ID in the inventory request

- RAN NGAP device ID

- AS ID.

### Device context release handling

if there’s no further communication is needed (e.g., A-IoT session is end), the device context can be released, in our view, the command procedure is device associated, the further communication can be per device and per session, so both per device and per session context release should be supported.

To support per session device context release, an A-IoT Session Release procedure can be introduced to release the all the device context(s) when the A-IoT session is finished.

To support per device context release, the following options can be considered

- option A, introduce list of RAN NGAP device ID in the A-IoT Session Release procedure

- option B, introduce a release flag in the command request, to indicate that there’s no more communication after this command procedure.

Option A is preferred as it’s cleaner.

Proposal 2, RAN3 agrees to support both per session and per device context release.

Proposal 3, RAN3 agrees to introduce an A-IoT Session Release procedure to release either all the device context or the indicated device context.

### Device context handling due to A-IoT radio failures

In some cases, the gNB cannot communicate with the A-IoT device any more, e.g., due to radio link failure, or the AS ID is released at device side due to power off, gNB will not maintain the device context, it should indicate the device context is released at gNB side to the A-IoT CN, if A-IoT CN wants to have further communication, it will re-trigger the inventory procedure.

Proposal 4, RAN3 agrees to introduce a A-IoT device context release indication procedure to indicate A-IoT CN the device context is released at gNB side due to some abnormal reasons.

3	Conclusion

In this contribution, we discussed device context handling and have the following observations and proposals.

Proposal 1, gNB should maintain the following device context for a specific A-IoT device in case of inventory and command operation

- correlation ID in the inventory request

- RAN NGAP device ID

- AS ID.

Proposal 2, RAN3 agrees to support both per session and per device context release.

Proposal 3, RAN3 agrees to introduce an A-IoT Session Release procedure to release either all the device context or the indicated device context.

Proposal 4, RAN3 agrees to introduce a A-IoT device context release indication procedure to indicate A-IoT CN the device context is released at gNB side due to some abnormal reasons.

4	Reference

[1]	 RP-250455              Rel-19 Ambient IoT WI specifications and other issues  Huawei, HiSilicon

5	Annex TP for TS 38.300 (device context handling)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

16.xx.x6	Device Context Handling

When an A-IoT-enabled gNB successfully inventories an A-IoT device, and a follow-on command indication is present in the inventory request, it establishes the device context, which includes the correlation ID, the RAN NGAP UE ID, and the AS ID for the A-IoT device. This device context facilitates further command operation between the A-IoT CN, A-IoT-enabled gNB, and the A-IoT device.

If the A-IoT session is end or there’s no more command to the A-IoT device(s), the A-IoT CN sends the A-IoT Session Release message to the A-IoT enabled gNB to stop A-IoT operation and release the device context(s).

If there’s some abnormal failures at radio interface, the A-IoT enabled gNB releases the device context and indicate to the A-IoT CN.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

6	Annex TP for TS 38.413 (device context handling)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |
| Broadcast Session Transport | BROADCAST SESSION TRANSPORT REQUEST | BROADCAST SESSION TRANSPORT RESPONSE | BROADCAST SESSION TRANSPORT FAILURE |
| Inventory Request | INVENTORY REQUEST | INVENTORY RESPONSE | INVENTORY FAILURE |
| Command Request | COMMAND REQUEST | COMMAND RESPONSE | COMMAND FAILURE |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| RAN Paging Request | RAN PAGING REQUEST |
| Inventory Report | INVENTORY REPORT |
| A-IoT Session Release | AIOT SESSION RELEASE |
| Device Context Release Indication | DEVICE CONTEXT RELEASE INDICATION |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

8.xx.4	A-IoT Session Release

8.xx.4.1	General

The purpose of the A-IoT Session Release procedure is to request the NG-RAN node to stop A-IoT operation and release all the device context for a A-IoT session. This procedure applies only if the NG-RAN node is a gNB.

8.xx.4.2	Successful Operation

Figure 8.xx.2.2-1: A-IoT Session Release

The A-IoT CN node initiates the procedure by sending an AIOT SESSION RELESAE message for the A-IoT session.

8.xx.4.3	Abnormal Conditions

Void.

8.xx.5	Device Context Release Indication

8.xx.5.1	General

The purpose of the Device Context Release Indication procedure is to enable the NG-RAN node to indicate the A-IoT CN node the A-IoT device context(s) are released for a A-IoT session. This procedure applies only if the NG-RAN node is a gNB.

8.xx.5.2	Successful Operation

Figure 8.xx.2.2-1: Device Context Release Indication

The NG-RAN node initiates the procedure by sending an DEVICE CONTEXT RELEASE INDICATION message for the A-IoT session.

8.xx.5.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.2.x.8	AIOT SESSION RELESAE

This message is sent by the A-IoT CN node to the gNB to indicate the A-IoT session ends.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Requested Device |  | 0..1 |  |  | YES | reject |
| >Requested Device Item |  | 1..<maxnoofDevices> |  |  |  |  |
| >>RAN NGAP Device ID | M |  | 9.3.3.yy1 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDevices | Maximum no. of Devices within a gNB. Value is 65535. |

9.2.x.9	DEVICE CONTEXT RELESAE INDICATION

This message is sent by the gNB to the A-IoT CN node to indicate the release of the A-IoT device context.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Released Device |  | 0..1 |  |  | YES | reject |
| >Released Device Item |  | 1..<maxnoofDevices> |  |  |  |  |
| >>RAN NGAP Device ID | M |  | 9.3.3.yy1 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDevices | Maximum no. of Devices within a gNB. Value is 65535. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>