3GPP TSG-RAN WG3 #128	          R3-253392

Malta, MT, 19th – 23th May  2025

Agenda item:	14.3

Source:	Nokia, Nokia Shanghai Bell, Qualcomm, Xiaomi, China Telecom, LG Electronics

Title:	(TP to BL CR for TS 38.413) Enhancement to support hard FLSO

Document for:	Discussion and Decision

# 1	Introduction

Last RAN3 meeting agreed the Stage-2/3 TP for the feeder link switch over, with EN to allow further check. This contribution discusses the remaining issues.

# 2	Feeder Link Switch Over

For Feeder Link Switch Over, last RAN3 meeting agreed following:

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

An EN is also added in the Stage-2/3 TP to allow companies have a further check.

In our understanding, this enhancement is beneficial to reduce packet loss during FLSO, especially since the NG-U does not support retransmission. In case Hard FLSO is performed, the gNB only connect with the new NTN GW after disconnect from the old NTN GW. This actually includes multiple steps, e.g. disconnect from old NTN GW, adjust antenna to new NTN GW, setup layer 1/2 connection, setup IPSec tunnel, setup SCTP, initiate NGAP RAN Configuration Update procedure via new SCTP, etc. This takes time. During this period, since 5GC does not know the hard FLSO until gNB informs 5GC, the 5GC may continue sending the DL NG-C/U packet to the old NTN GW. The old NTN GW has no way to deliver it to the gNB or forward it to the new NTN GW. This causes packet loss.

Observation 1-1: without knowing the hard FLSO, the 5GC (AMF/UPF) continue sending the DL NG-C/U to the gNB via old NTN GW, and will be discarded at the old NTN GW.

Please note the issue does not exist in Transparent architecture. In transparent architecture, the gNB is on the ground, and is aware of the FLSO. The DL data is always sent to the gNB irrespective of the FLSO. For a UE within the gNB’s coverage, the FLSO only affect which NTN GW to be used by the gNB to send the DL data to the satellite. When a hard FLSO is to be performed, the gNB just stop the DL transmission and buffer the DL data. After the hard FLSO is completed, the gNB forward the buffered DL data to the satellite via the new NTN GW. So there is no packet loss during the hard FLSO in Transparent architecture.

It is beneficial to inform the 5GC before the hard FLSO, so the 5GC can suspend the DL NG-C/U. This can be supported by the RAN Configuration Update procedure or other NGAP procedure, which can be further discussed.

Without this enhancement for Regenerative architecture, the packet loss can happen for FLSO in Regenerative architecture but not in Transparent architecture.

In last meeting, company mentioned another OAM-based approach that AMF may be configured with the information for hard FLSO, thus not requiring the satellite/gNB to inform AMF when the hard FLSO is performed. However, we think this proposal has issues for following reasons. 1) current AMF can be configured with the TAIs of the gNB, but not the information for hard FLSO that may not cause the change of gNB’s TAI. 2) the actual FLSO is performed in considering multiple criteria, e.g. quality of current radio connection via the old NTN GW, the visibility/availability of the new NTN GW, etc. The radio connection with the NTN GW is also affected by other criteria, e.g. the weather (cloud, rain, etc). For example, in case heavy rain happens in the geo-area of the old NTN GW, the FLSO may be performed earlier due to the bad radio connection with the old NTN GW. So even in case enhancement is introduced to configure the AMF with the timing window information for the hard FLSO (e.g. hard FLSO happen between 10:00 – 10:05), the AMF cannot know when the hard FLSO is actually performed (e.g. at 10:03). Since AMF cannot know the actual time for hard FLSO, the AMF may only be able to stop DL NG at the beginning of the timing window, which may be well before the hard FLSO. This causes more issues, e.g. long interruption. So we think this option is not good and should not be adopted.

Observation 1-2: Configurating the AMF with the timing window information for hard FLSO may cause more issues, and does not help.

In summary, to have similar performance in both Transparent architecture and Regenerative architecture, we propose to agree the enhancement agreed in last meeting and remove related Ens from Stage-2/3 BL CRs.

Observation 1-3: it is beneficial to inform the 5GC before the hard FLSO, so the 5GC can suspend the DL NG-C/U.

Proposal 1: Remove the ENs related to FLSO from Stage-2/3 BL CRs.

Draft TS 38.300 TP can be found in ([3])

Draft TS 38.413 TP can be found in Annex A – TP for TS 38.413

In last meeting, it was commented whether this may be also used for NG suspend/resume. By suspending the DL NG-C/U transmission, the NG connection is implicitly suspended, since there will no NG-C/U exchange between the gNB and the CN. We agree this enhancement can also be used for NG suspend/resume.

Proposal 2: this enhancement for FLSO can also be used to suspend the NG connection.

# 3	Conclusion

In this contribution, we discussed the issues to support regenerative architecture. Our proposals are:

Proposal 1: Remove the ENs related to FLSO from Stage-2/3 BL CRs.

Draft TS 38.300 TP can be found in ([3])

Draft TS 38.413 TP can be found in Annex A – TP for TS 38.413

Proposal 2: this enhancement for FLSO can also be used to suspend the NG connection.

# References

- R3-252523	(BL CR to 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN

- R3-252525	(BL CR to 38.413) Support for Regenerative Payload and MBS broadcast in NR NTN

- R3-253265	(TP to TS 38.300 BL CR) Support for Regenerative Payload and MBS broadcast in NR NTN

# Annex A – TP for TS 38.413

-----------------Start of the Changes-------------------

### 8.7.2	RAN Configuration Update

#### 8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

#### 8.7.2.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the Downlink NG Transmission Control IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, suspend or resume the downlink NG transmission accordingly.

-----------------Next Changes-------------------

### 8.7.x	NG Removal

#### 8.7.x.1	General

The purpose of the NG Removal procedure is to remove the interface instance between the NG-RAN node and the AMF in a controlled manner. If successful, this procedure erases any existing application level configuration data in the two nodes.

The procedure uses non-UE associated signaling.

#### 8.7.x.2	Successful Operation

Figure 8.7.x.2-1: NG Removal, successful operation

The NG-RAN node initiates the procedure by sending the NG REMOVAL REQUEST message to the AMF. Upon reception of the NG REMOVAL REQUEST message, the AMF shall reply with the NG REMOVAL RESPONSE message. After receiving the NG REMOVAL RESPONSE message, the NG-RAN node shall initiate removal of the TNL association towards the AMF and may remove all resources associated with that interface instance. The AMF may then remove all resources associated with the NG-RAN node.

#### 8.7.x.3	Unsuccessful Operation

Figure 8.7.x.3-1: NG Removal, unsuccessful operation

If the AMF cannot accept to remove the interface instance with NG-RAN node, it shall respond with an NG REMOVAL FAILURE message with an appropriate cause value.

#### 8.7.x.4	Abnormal Conditions

Void.

-----------------Next Change-------------------

### 8.17.1	Broadcast Session Setup

#### 8.17.1.1	General

The purpose of the Broadcast Session Setup procedure is to request the NG-RAN node to setup MBS session resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.1.2	Successful Operation

Figure 8.17.1.2-1: Broadcast Session Setup, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION SETUP REQUEST message to the NG-RAN node. If the NG-RAN node accepts all the MBS QoS flows in the MBS session at least in one of its cells, the NG-RAN node responds with the BROADCAST SESSION SETUP RESPONSE message.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44], and TS 38.300 [8] for NTN operation.

If the MBS Session FSA ID List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine cells/frequencies within the MBS service area to broadcast MBS session data as specified in TS 23.247 [44].

If the Supported UE Type List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the Associated Session ID IE is included in the BROADCAST SESSION SETUP REQUEST message the NG-RAN node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.300 [8].

-----------------Next Change-------------------

### 8.17.2	Broadcast Session Modification

#### 8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area as needed and send the BROADCAST SESSION MODIFICATION RESPONSE message. For NTN operation, the NG-RAN node shall, if supported, update the intended service area information as specified in TS 38.300 [8].

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the Supported UE Type List IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the MBS NG-U Failure Indication IE is included in the BROADCAST SESSION MODIFICATION REQUEST message within the MBS Session Setup or Modification Request Transfer IE and set to "N3mb path failure", the NG-RAN node may provide new NG-U transport layer information to replace the failed one(s) or switch the data delivery to another 5GC as per the Broadcast MBS session restoration procedure for N3mb Path Failure specified in TS 23.527 [57].

-----------------Next Change-------------------

#### 9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node ® AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Downlink NG Transmission Control | O |  | ENUMERATED (suspend, resume, …) | Indicates the CN to suspend or resume the downlink NG Transmission | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

-----------------End of the Changes-------------------