3GPP TSG-RAN WG3 #128	          R3-253396

Malta, MT, 19th – 23th May  2025

Agenda item:	21.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the remaining issues of Rel-19 XR

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses the remaining issues for Rel-19 XR enhancements:

- UL Rate Control, e.g. impact on F1AP, FFS on whether CN may inform gNB the recommended UL rate for specific QoS flows.

- FFS on specific container/frame to be used for BSSize and TTNB. FFS on impact to TS 38.420/470.

- FFS on non-homogeneous deployment for support of exposure of available data rate.

- FFS on whether the gNB-CU sends an indication to the gNB-DU to stop the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

# 2	Specify uplink congestion signalling

F1 Impact on UL Congest Control

The previous RAN2 LS listed two options:

- Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

In last RAN2 meeting, RAN2 agreed

Agreements on XR rate control 5. From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. 6. FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

The impact to F1AP is whether the indication to gNB-DU is per DRB or per QoS flow. The latest RAN2 agreement requires this indication to be per QoS flow, so the gNB-DU can add the QFI information in the MAC CE. It is up to the implementation of the gNB-DU on how to determine the recommended data rate. For example, one gNB-DU may be implemented to split the available data rate equally for the QoS flows sharing the same DRB, or split proportionality according to the GFBR, or consider the current UL data rate of a QoS flow, etc. There is no need to specify the detail.

Proposal 1-1: add the per-QoS flow indication in F1AP to inform the gNB-DU that the indicated QoS flow is subject to UL congestion control.

Recommended UL data rate

In last meeting, Contribution ([2]) proposes to discuss whether CN may inform gNB the recommended UL rate for specific QoS flows, with following reason:

If the Rate control can be decide by gNB, gNB has no idea whether the adjusted data rate fulfills the application requirement. In other word, whether the UE application layer satisfy the new data rate can not be detected before this update.

In another way, gNB can also ask UEs to give some assistance information on the rate control. But UE has no idea on how heavy the workload at gNB side. It is impossible for multiple UEs sync up with each other and provide assistance info on rate control info which may works at gNB side. Hence, the most straight forward way to solve this issue is to let CN provide some recommended UL rate info which may let both gNB and UE satisfy.

So the main benefit is the gNB may only need to inform UE when a specific rate can be supported, rather any value of the data rate.

Contribution ([2]) proposes following IE to be provided from CN to gNB for a specific QoS flow.

#### 9.2.3.x	Recommended Uplink Rate Control Information List

This IE defines the Recommended Uplink Rate Control Information to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended Uplink Rate Control Item |  | 1..<maxnoofRecULRateControl> |  |  |
| >Recommended Uplink Rate Control Index | M |  | INTEGER(1..FFS, ...) | Indicate the index of the item within the Recommended Uplink Rate Control Information List IE. |
| >Recommended Uplink Rate Control | O |  | Bit Rate 9.3.1.4 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRecULRateControl | Maximum no. of alternative sets of Uplink Rate Control Information allowed. Value is FFS. |

RAN3 need to check with SA2/SA4 on the feasibility. We suggest send a LS to SA2/SA4 asking whether the CN can provide such information.

Proposal 1-2: RAN3 send a LS to SA2/SA4 regarding whether the CN can provide the Recommended UL Data Rate to gNB, so the gNB only need to inform UE when a specific UL data rate can be supported.

Draft LS can be found in Annex A – LS to SA2/SA4 on Recommended UL Data Rate

# 3	BSSize and TTNB

According to TS 26.522, SA4 defines BSSize and TTNB in a new RTP header Extension, i.e. RTP header extension for dynamically changing traffic characteristics (TS 26.522, section 4.5), which is different to the RTP Header extension for PDU Set Marking, i.e. RTP Header Extension for PDU Set Marking (TS 26.522, section 4.2). So it is suggested to define BSSize and TTNB out of the existing PDU Set Information user plane protocol, i.e. Add BSSize and TTNB into the DL PDU SESSION INFORMATION frame defined in TS 38.415 and the DL USER DATA frame defined in TS 38.425.

Proposal 2: Remove the EN from TS 38.415 BL CR, i.e.

- For TS 38.415 TP, remove following EN

Editor’s Note: FFS on whether TTNB and/or BSSize related information may be removed in this clause and introduced into other frame(e.g. DL PDU SET INFORMATION).

- For TS 38.425 TP, remove following EN

Editor Note: FFS whether to capture the Burst Size and Time To Next Burst related information in the DL USER DATA frame.

TP to TS38.415 and TS38.425 BL CR can be found in ([7]).

# 4	Support of exposure of available data rate

Non-homogeneous deployment

This was briefly discussed in last RAN3 meeting. It was commented whether need to consider non-homogeneous deployment where the gNBs in an area may have different capabilities, more specifically, whether the gNB need to inform CN on its capability to support the exposure of available data rate. The non-homogeneous deployment was supported in Rel-18 XR, e.g. TS 38.300 Section 16.15.5 “Non-Homogeneous support of PDU set based handling in NG-RAN”. In Rel-18, NGAP signaling was agreed to enable the gNB inform CN on whether PDU set based handling is supported in the gNB.

However, SA2 TS 23.501 seems not requiring the signaling-based approach for the non-homogeneous deployment per following text from TS 23.501:

Based on configuration, the AMF may know whether the QoS monitoring feature (i.e. each of Packet delay monitoring and/or Congestion monitoring) is supported by all NG RAN nodes in an area. The AMF shall, when configured, inform the SMF about the support of the QoS monitoring feature by sending the parameter NG RAN QoS monitoring capability during PDU Session establishment, PDU Session modification, Service request and UE mobility procedures.

During the offline discussion, it was also commented that the CN may need to know whether the exposure of available data rate is “active” in the gNB after the CN has requested the gNB to exposure the available data rate. But we are not sure about this scenario. According to SA2 TS 23.501, SA2 did not consider this scenario. It is unclear on why the status in gNB is “not active” for one or some QoS flows when the gNB support the exposure of available data rate. Unless it is required by SA2, we do not see any NGAP enhancement is needed.

Proposal 3-1: No NGAP signaling enhancement is needed for the support of non-homogeneous deployment.

Encoding of available bit rate

In current NGAP/XnAP/E1AP/F1AP, the Bit Rate IE is defined as INTEGER (0..4,000,000,000,000,...), with unit bit/second. This supports up to 4T bps. The available bit rate and threshold need to be aligned with the Bit Rate IE.

A small optimization may use unit kbps, rather bps. This requires a 32-bit integer to report up to 4G Kbps (or 4T bps). This also aligns with the following Reporting Threshold defined in TS 29.244 ([8]):

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 348 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  | ULDR | DLDR | ULCI | DLCI |  |
|  | m to (m+1) | DL Congestion information threshold |  |  |  |  |  |  |  |  |
|  | p to (p+1) | UL Congestion information threshold |  |  |  |  |  |  |  |  |
|  | q to (q+4) | DL Data Rate threshold |  |  |  |  |  |  |  |  |
|  | r to (r+4) | UL Data Rate threshold |  |  |  |  |  |  |  |  |
|  | s to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.240-1: Reporting Thresholds

The Downlink Data Rate threshold field and the Uplink Data Rate threshold field shall be encoded as kilobits per second (1 kbps = 1000 bps) in binary value (rounded upwards).

Proposal 3-2: define available bit rate and threshold as a 32-bit integer with unit kbps.

In current system for Alternative QoS, the maxnoofQoSparaSets is set to value “8”. We suggest use the same value “8” for maxnoofThresholds. There is no much benefit to provide many thresholds considering the actual scenario.

Proposal 3-3: use value “8” for maxnoofThresholds.

TP to TS38.415 and TS38.425 BL CR can be found in ([7]).

TP to TS38.413 TP can be found in ([7]).  If agreed, similar TP is needed for other BL CRs.

# 5	Avoid unnecessary (re-)transmission

Contribution ([5]) propose to discuss how to support unnecessary RLC retransmission avoidance. The reason is as below:

RAN2 agreements:

Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly

For downlink, the PDCP is located in the gNB-CU while the TX RLC entity is located in gNB-DU. To support RAN2’s agreement, some coordination between the gNB-CU and the gNB-DU is needed. The gNB-CU needs to send a discard indication of a PDCP SDU to the gNB-DU so that the gNB-DU can stop the transmission and retransmission of the SDU or the segment of the SDU. When the PDCP timer expires for a PDCP SDU, the gNB-CU sends the indication to the gNB-DU to stop the transmission and retransmission of the RLC SDU or the segment of the RLC SDU which corresponds to the PDCP SDU.

In current F1-U ([6]), the node hosting the NR PDCP entity (gNB-CU-UP) can indicate to the corresponding node (gNB-DU) to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs. The discard was introduced for PDCP duplication, i.e. to discard those duplicated DL data buffered in the gNB-DU, after the UE has received them from the other leg. The PDCP PDUs to be discarded may already be sent to lower layer for (re-)transmission. It seems the Unnecessary retransmissions avoidance can be supported by current discard mechanism in F1-U. At current stage, no enhancements are needed. Depending on RAN2 progress, RAN3 can discuss whether further enhancements is needed.

Observation: current discard mechanism in F1-U can be used to stop the unnecessary (re-)transmission.

Proposal 4: RAN3 can wait for RAN2 progress to decide potential enhancement.

# 6	Alternative QoS with PDU Set QoS parameters

Last RAN3 meeting agreed TS 37.340 TP to clarify that the notification control in MR-DC is updated to cover the PSER/PSDB (TP is copied below)

### 10.13.2	MR-DC with 5GC

The Notification Control Indication procedure may be initiated either by the MN or by the SN and is used to indicate that GFBR/PSER/PSDB for one or several QoS flows cannot be fulfilled any more or can be fulfilled again by the reporting node.

However, current TS 38.423 (coped as below) only support the notification control when GFBR cannot be fulfilled or can be fulfilled, but not for PDB/PER.

### 8.3.10 Notification Control Indication

8.3.10.1 General

The purpose of the Notification Control indication procedure is to provide information that for already established GBR QoS flow(s) for which notification control has been requested, the NG-RAN node involved in Dual Connectivity cannot fulfil the GFBR anymore or that it can fulfil the GFBR again.

…

This procedure is triggered to notify the S-NG-RAN node for SN-terminated bearers, that resources requested from the M-NG-RAN node can either not fulfil the GFBR anymore or that the GFBR can be fulfilled again, as specified in TS 7.340 [8].

It would be very difficult to measure whether PSER/PSDB can be fulfilled or not fulfilled in MR-DC, especially for split bearer. So we propose to align with current Xn Notification Control, and not support the monitoring of PSER/PSDB in MR-DC.

Proposal 5: remove the PSDB/PSER in the notification control for MR-DC.

TP to BL CR for TS 37.340 can be found in Annex B – TP to BL CR for TS 37.340

# 7	Conclusion

In this contribution, we have analyzed the new RAN3 related objectives. Our proposals are:

For Specify uplink congestion signalling:

Proposal 1-1: add the per-QoS flow indication in F1AP to inform the gNB-DU that the indicated QoS flow is subject to UL congestion control.

Proposal 1-2: RAN3 send a LS to SA2/SA4 regarding whether the CN can provide the Recommended UL Data Rate to gNB, so the gNB only need to inform UE when a specific UL data rate can be supported.

Draft LS can be found in Annex A – LS to SA2/SA4 on Recommended UL Data Rate

For QoS Handling when Traffic Characteristics Change Dynamical:

Proposal 2: Remove the EN from TS 38.415 BL CR, i.e.

- For TS 38.415 TP, remove following EN

Editor’s Note: FFS on whether TTNB and/or BSSize related information may be removed in this clause and introduced into other frame(e.g. DL PDU SET INFORMATION).

- For TS 38.425 TP, remove following EN

Editor Note: FFS whether to capture the Burst Size and Time To Next Burst related information in the DL USER DATA frame.

TP to TS38.415 and TS38.425 BL CR can be found in ([7]).

For Support of exposure of available data rate:

Proposal 3-1: No NGAP signaling enhancement is needed for the support of non-homogeneous deployment.

Proposal 3-2: define available bit rate and threshold as a 32-bit integer with unit kbps.

Proposal 3-3: use value “8” for maxnoofThresholds.

TP to TS38.415 and TS38.425 BL CR can be found in ([7]).

TP to TS38.413 TP can be found in ([7]).  If agreed, similar TP is needed for other BL CRs.

For Avoid unnecessary (re-)transmission:

Observation: current discard mechanism in F1-U can be used to stop the unnecessary (re-)transmission.

Proposal 4: RAN3 can wait for RAN2 progress to decide potential enhancement.

For Alternative QoS with PDU Set QoS parameters:

Proposal 5: remove the PSDB/PSER in the notification control for MR-DC.

TP to BL CR for TS 37.340 can be found in Annex B – TP to BL CR for TS 37.340

# References

- R3-251513	LS on uplink rate control  RAN2

- R3-252160, Discussion on XR Enhancement with draft reply LS

- R3-251736	(TP to BL CR for TS 38.413) enhancement for available data rate report

- R3-251738	(TP to BL CR for TS 38.300) Support for Burst Size and TTNB

- R3-251955 Discussion on bit rate control and unnecessary retransmission

- TS 38.425 	NG-RAN; NR user plane protocol

- R3-253397, (TP to BL CR for TS 38.415 and TS 38.425) Update for BSSize/TTNB and available data rate exposure

- TS 29.244, Interface between the Control Plane and the User Plane nodes

# Annex A – LS to SA2/SA4 on Recommended UL Data Rate

3GPP TSG-RAN WG3 #128	          R3-25xxx2

Malta, MT, 19th – 23th May  2025

Title:	[DRAFT] LS on Recommended UL Data Rate

Response to:	-

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	Nokia [to become RAN3]

To:	SA2, SA4

Cc:	RAN2

Contact Person:

#### Name:	Steven Xu

E-mail Address:	steven.1.xu@nokia-sbell.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 is discussing the UL Rate Control, i.e. in case the UL is congested, the gNB informs UE for the recommended UL data rate. To avoid the gNB informs the UE for any data rate value, RAN3 would like to hear SA2/SA4 opinion on whether the CN can provide the gNB on the recommended UL data rate(s), so the gNB only need to inform UE when a specific UL data rate is available.

2. Actions:

To SA2 and SA4 group.

ACTION: 	RAN3 asks SA2 and SA4 to provide feedback on when the CN can provide the gNB on the recommended UL data rate(s).

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #129	25-29 August 2025				India

TSG-RAN3 Meeting #129bis	13-27	October 2025			Prague , CZ

# Annex B – TP to BL CR for TS 37.340

### 10.13.2	MR-DC with 5GC

The Notification Control Indication procedure may be initiated either by the MN or by the SN and is used to indicate that GFBR for one or several QoS flows cannot be fulfilled any more or can be fulfilled again by the reporting node.

Figure 10.13.2-1: Notification Control Indication procedure

Figure 10.13.2-1 shows an example signalling flow for the Notification Control Indication procedure.

1.	The MN may, for an SN terminated bearer, indicate, that the GFBR requested from the MN cannot be fulfilled anymore.
In case the SN terminated bearer is configured as a split bearer, the SN may decide to increase the share provided by the SN or it may decide to notify the MN that resources requested for the SN terminated bearer cannot fulfill the GFBR any more.

2.	Continuing the example message flow from step 1, the SN informs the MN that the GFBR for an SN terminated bearer cannot be fulfilled any more.

3.	The MN decides to inform the 5GC that NG-RAN cannot fulfill the GFBR for a GBR QoS flow any more.