3GPP TSG-RAN WG3 #128								R3-253415

Malta, MT, 19th – 23rd May 2025

Agenda Item:	12.2

Source:	China Telecom

Title: 	RAN2 Impact of WAB

Document for:	Discussion

Introduction

The new WI on additional topological enhancements for NR was approved in RAN#105 meeting [1]. And the following functional aspects of WAB-nodes need to be discussed in WI Phase:

- NG interface aspects.

- WAB authorization.

- Mobility handling.

- Xn interface aspects.

Furthermore, according to the note in the WID, RAN2 impact should be identified as early as possible and kept to a minimum. During the SI phase, discussions on RAN2 impact were limited, as it was assumed that the WAB would not affect the Uu interface. Since the WAB entity consists of two components, namely WAB-MT and WAB-gNB—similar to IAB and NCR—it is necessary to define the functionalities of WAB-MT and the behaviour of WAB-gNB during mobility procedures, at least in Stage 2. This contribution primarily focuses on the RAN2 impact and functional aspects of WAB.

Discussion

RRC States

According to agreement in RAN3#125bis meeting, WAB-MT supports at least a subset of functionalities:

The WAB-MT supports at least a subset of UE functionalities.

Therefore, it is necessary to determine whether all RRC states are supported in WAB-MT. The behaviors and functionalities of NCR-MT and IAB-MT are documented in existing Stage 2 and Stage 3 specifications. As stated in TS38.300, NCR-MT supports all RRC states, as well as the behavior of NCR-Fwd during state transitions.

Similarly, WAB-MT must support all existing RRC states, namely Connected, Idle, and Inactive. Additionally, WAB-MT must support all SRBs, as it may be configured with either a single carrier or MR-DC by BH gNB(s). Based on this analysis, it is concluded that WAB-MT must support all RRC states and all types of SRBs.

When the NCR-MT is in RRC_CONNECTED state, the NCR-Fwd may amplify-and-forward RF signals based on the side control information received from the gNB. The NCR-MT does not support RRM measurements in RRC_CONNECTED.

When the NCR-MT transitions from RRC_CONNECTED state to RRC_INACTIVE state, the NCR-Fwd may continue to amplify-and-forward RF signals in accordance with the last side control information received from the gNB. When the NCR-MT is in RRC_INACTIVE state, the NCR-Fwd ceases to amplify-and-forward RF signals if no suitable cell is detected, or if the NCR-MT selects a different cell than the last serving cell on which side control configuration was received.

When an NCR-MT in RRC_INACTIVE state determines degradation of the NCR-Fwd backhaul link beam, then the NCR-Fwd should cease amplifying-and-forwarding RF signals, and the NCR-MT should attempt to resume its RRC connection (with cause value mo-Signalling). The criteria to evaluate backhaul beam degradation are left to the NCR-node implementation.

When the NCR-MT transitions from RRC_CONNECTED state to RRC_IDLE, the NCR-Fwd ceases any amplifying-and-forwarding of RF signals. How an NCR-MT transitions back from RRC_IDLE state to RRC_CONNECTED state is left to NCR-node or network implementation.

Proposal 1: WAB-MT supports all RRC states and all types of SRBs.

In the Connected state, WAB-gNB can send and receive signals to and from the UE and establish Xn and NG interfaces with BH gNB/5GC. However, the behaviour of WAB-gNB in the Idle and Inactive states has not been previously discussed in RAN3 and RAN2 meetings. When WAB-MT transitions from the Connected state to the Idle state, WAB-gNB ceases signal transmission and reception with the UE, removes Xn/NG interfaces, and redirects all UEs to other NG-RAN or E-UTRAN nodes.

For the Inactive state, whether WAB-gNB ceases signal transmission and reception requires further study in RAN2. From a RAN3 perspective, the Xn/NG interfaces could be maintained, as the arrival of Xn-C/NG-C signalling or Xn-U/NG-U user plane data may trigger WAB-MT to transition from the Inactive to the Connected state. In summary, the behaviour of WAB-gNB in the Idle and Inactive states of WAB-MT requires further discussion in RAN2.

Proposal 2: The behaviour of WAB-gNB in the Idle and Inactive states of WAB-MT requires further discussion in RAN2.

RLF and BFD/BFR

According to the NOTE6 in objective part of WID, the Mobility of the WAB-MTs reuses the legacy UE mobility procedures. This NOTE also implies that the WAB-MT shall supports RLF detection, Beam Failure Detection (BFD) and Beam Failure Recovery (BFR) procedure as normal UE.

NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures.

According to NOTE 6 in the objective section of the WID, the mobility of WAB-MTs follows legacy UE mobility procedures. This note also implies that WAB-MT must support Radio Link Failure (RLF) detection, Beam Failure Detection (BFD), and the Beam Failure Recovery (BFR) procedure, similar to a standard UE.

When RLF is detected during a mobility procedure, WAB-MT performs the RRC re-establishment procedure as a legacy UE. However, during this procedure, the behavior of WAB-MT requires further discussion in RAN2, as WAB-gNB disconnects from the BH gNB before the re-establishment process is completed. Similarly, for NCR-MT, the behavior of NCR-Fwd must also be specified in the Stage 2 specification.

Proposal 3: The behavior of WAB-gNB during RLF or Beam Failure must be discussed in the RAN2 WI phase.

Access Control

To prevent a WAB-MT from attempting to access the cell of a WAB-gNB, a mechanism must be specified to avoid multi-hop scenarios. One potential solution is to introduce a new indicator in SIB1 to signal WAB support. The WAB-MT should treat a cell broadcasting this new indicator as a barred cell, while legacy UEs should ignore it.

Additionally, since IAB and NCR are alternative methods for extending coverage and enhancing wireless topology, RAN2 must determine whether WAB should support NCR and IAB. In our view, WAB should not support NCR and IAB to prevent multi-hop scenarios, and a Stage 2 specification detailing this conclusion is necessary.

Proposal 4: The decision on whether WAB should support NCR and IAB should be made by RAN2.

Furthermore, several aspects related to IAB-MT and NCR-MT have been discussed within the scope of RAN2. Accordingly, RAN2 must also determine whether these aspects are applicable to WAB-MT:

- Cell status and cell reservations: Both IAB-MT and NCR-MT ignore the existing cell status and reservations in SIB1, e.g, cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse, intraFreqReselection, and cellReservedForOtherUse.

- Unified Access Control: Unified Access Control does not apply to IAB-MTs and NCR-MTs

- additional allowed cell list and/or forbidden cell list: NCR-MT may be configured with additional allowed cell list and/or forbidden cell list. And The NCR-MT shall consider only the allowed cell list for reselection candidate cells.

Proposal 5: RAN2 should evaluate the necessity of including cell status and cell reservations, unified access control, and additional allowed or forbidden cell lists for WAB.

Proposal 6: RAN3 agree to send a LS to RAN2 to start their work immediately.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: WAB-MT supports all RRC states and all types of SRBs.

Proposal 2: The behaviour of WAB-gNB in the Idle and Inactive states of WAB-MT requires further discussion in RAN2.

Proposal 3: The behavior of WAB-gNB during RLF or Beam Failure must be discussed in the RAN2 WI phase.

Proposal 4: The decision on whether WAB should support NCR and IAB should be made by RAN2.

Proposal 5: RAN2 should evaluate the necessity of including cell status and cell reservations, unified access control, and additional allowed or forbidden cell lists for WAB.

Proposal 6: RAN3 agree to send a LS to RAN2 to start their work immediately.

Reference

- RP-242395，New WID on additional topological enhancements for NR，NTT DOCOMO, INC., AT&T

- S2-2409245，Mobility of UEs served by MWAB,Huawei, HiSilicon, Nokia, Ericsson, ZTE