3GPP TSG RAN WG3 Meeting #128	R3-253416

St Julian, Malta, 19 - 23 May, 2025

Title: 	On per-UE UE performance metrics

Source: 	Ericsson

Agenda item:	8.1

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution we discuss the Reply LS received from SA5 on the topic of per-UE UE performance metrics in R3-253023.

# 2. Discussion

RAN3 in R3#126 agreed to send an LS to SA5 (R3-247775) asking

“whether the formulas in fields c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558, respectively, refer to measurements that are specified with a per-UE granularity or with a per-DRB granularity as the measurement name in field e) of the same mentioned clauses starts with “DRB.”.”

SA5 has replied to this LS in R3-253023, stating that:

“the formulas specified in fields (c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558 calculate average UE throughput based on per DRB level measurement for a UE. The formulas aggregate data across DRBs for each UE, ensuring that the reported throughput measurement represents the overall throughput per UE rather than being limited to individual DRBs.”

Based on the above response, RAN3 can conclude that the average UE throughput DL and average UE throughput UL measurements defined in TS 28.558 are indeed per-UE throughput measurements and therefore these measurements can be used for UE performance feedback purposes.

Proposal 1: RAN3 to agree to reference the definition of the Average UE throughput UL and DL measurements in TS 28.558 for UE Performance Feedback throughput measurements.

In the context of UE performance in both the “AI/ML Enabled Slicing” agenda item and in the “Other interfaces discussions in Rel.18 Leftovers” agenda item, RAN3 has been discussing how to obtain throughput, delays, and packet loss as per-UE UE performance metrics.

Based on the received Reply LS from SA5, and assuming that the same logic explained by SA5 is valid for all per-UE measurements defined in TS28.558, it is proposed for RAN3 to agree to the following:

Proposal 2: The metrics defined in TS 28.558 can be referenced for the definition of per UE performance metrics in the RAN3 specifications.

Furthermore, RAN3 has been discussing how to derive per-S-NSSAI UE performance feedback as part of Slice UE Performance IE and the following was agreed in R3#127:

Per S-NSSAI per UE performance metrics is reported as part of the Slice UE Performance IE in the Data Collection Update message. An FFS on how the metrics are calculated will be added in the TP. If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement.

The Reply LS from SA5 has already clarified that the measurements specified in TS28.558 are indeed at a per-UE level.

It is logical to assume that any additional filter applied to a per UE measurement defined in TS28.558 is applied on top of the per-UE measurement granularity, e.g., a metric defined as DRB.UEThpUl.SNSSAI would imply that the measurement is derived as an aggregate of all the per DRB measurements, for the DRBs associated to the UE and to the specific S-NSSAI. In order to progress work in RAN3, it is proposed to take the following working assumption, based on the inputs received in the reply LS from SA5.

Proposal 3: RAN3 to agree to a working assumption that according to the existing specifications in TS 28.558, per-UE per-S-NSSAI measurements are defined in TS28.558 and can be referenced in RAN3 specifications.

However, and to clarify our assumptions, it is also proposed to send an LS to SA5 asking for confirmation that the measurement specified in TS28.558 can represent per UE, per S-NSSAI aggregated measurements.

Proposal 4: RAN3 to agree to send a Reply LS to SA5 requesting to confirm that the measurement specified in TS28.558 can represent per UE, per S-NSSAI aggregated measurements.

# 3. Conclusions

This paper makes the following observations and proposals:

Proposal 1: RAN3 to agree to reference the definition of the Average UE throughput UL and DL measurements in TS 28.558 for UE Performance Feedback throughput measurements.

Proposal 2: The metrics defined in TS 28.558 can be referenced for the definition of per UE performance metrics in the RAN3 specifications.

Proposal 3: RAN3 to agree to a working assumption that according to the existing specifications in TS 28.558, per-UE per-S-NSSAI measurements are defined in TS28.558 and can be referenced in RAN3 specifications.

Proposal 4: RAN3 to agree to send a Reply LS to SA5 requesting to confirm that the measurement specified in TS28.558 can represent per UE, per S-NSSAI aggregated measurements.

A draft reply LS is provided in R3-253416