3GPP TSG RAN WG3 Meeting #128	R3-253417

St Julian, Malta, 19 - 23 April, 2025

Title:	Reply LS on per-UE UE performance metrics

Response to:	R3-253023 (S5-252056)

Release:	Rel-19

Work Item:	NR_AIML_NGRAN-Core

Source:	RAN3

To:	SA5

Cc:

Contact person:	Angelo Centonza

Angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 would like to thank SA5 for their Reply LS on per-UE UE performance metrics regarding the interpretation of Average UE Throughput DL and Average UE Throughput UL measurements defined in TS 28.558, particularly with respect to their granularity and the "DRB." prefix in their measurement names. RAN3 thanks SA5 for the clarification that the "DRB." prefix in field (e) of clause 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558 indicates that the contributing measurements originate from DRBs, but the measurements are actually aggregated to reflect per-UE throughput.

RAN3 would like to request for further clarification and confirmation of the interpretation of additional filters specified for several measurements defined in chapter 6.3.1 of TS 28.558, e.g., DRB.AirIfDelayDlUe.SNSSAI, DRB.RlcSduDelayDlUe.SNSSAI, DRB.UEThpDl.SNSSAI, DRB.UEThpUl.SNSSAI.

RAN3 would like to request SA5 to confirm that the same interpretation as described by SA5 in the Reply LS on per-UE UE performance metrics (R3-253023) also applies for all measurements defined with the above-described name pattern in TS 28.558.

Namely, RAN3 would like confirmation that, whenever a measurement follows the name structure DRB.measurementname.FILTER, the prefix ‘DRB.’ only indicates that the measurement is derived from per-DRB measurements, but the reported measurement value is aggregated to reflect per-UE per-FILTER measurements. As an example, the DRB.UEThpDl.SNSSAI measurement would provide the aggregated DL throughput per UE and per S-NSSAI.

# 2	Actions

To SA5 group

ACTION: 	RAN3 respectfully asks SA5 to provide feedback on the above question.

# 3	Dates of next TSG-RAN WG3 meetings

TSG-RAN3 Meeting #129			August 25-28, 2025			Bangalore, India

TSG-RAN3 Meeting #129-bis			October 13-17, 2025			Prague, Czech Republic

TSG-RAN WG3 Meeting RAN3#130		17 - 21 Nov, 2025			Dallas, USA