3GPP TSG-RAN WG3 Meeting #127-bis	R3-253418

Wuhan, China, 7-11 April 2025

Title: 	AI/ML enabled Network Slicing

Source: 	Ericsson, JIO Platforms, InterDigital, Deutsche Telekom

Agenda item:	11.2

Document Type:	Discussion and agreement

# Introduction

At the RAN3#127-bis meeting the following agreement and FFS were captured regarding the per slice UE performance reporting:

Slice UE performance request:

For Slice UE performance request, take Opt2: Extend Existing Report Characteristics Bitmap.

Failed Report Characteristics:

Extending the existing Node Measurement Failed Report Characteristics IE to include slice-level UE performance metrics for indicating the failed result for slice-level UE performance metrics.

Per Slice UE performance report:

Add Slice UE performance IE in the DATA COLLECTION UPDATE message.

Per S-NSSAI per UE performance metrics is reported as part of the Slice UE Performance IE in the Data Collection Update message. An FFS on how the metrics are calculated will be added in the TP. If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement.

Introduce the measured CCCV within the predicted slice available capacity.

Through this contribution we discuss these remaining topics and propose potential solutions for covering the required stage 3 details.

# 2. Discussion

## Requesting list of slices for UE Performance

For the source gNB to request from the target gNB the reporting of per slice UE performance after a successful UE handover the following agreement was made at RAN3#127-bis:

For Slice UE performance request, take Opt2: Extend Existing Report Characteristics Bitmap.

In addition, the source gNB must be able to indicate to the target gNB which are the network slices for which the per slice UE Performance reporting is requested. In absence of this indication, the target gNB will have no options but to report the per slice UE Performance for all the network slices for which the UE has an active PDU session in the target cell despite that:

- the source gNB may not need the per slice UE Performance to be reported for this entire set of slices. Indeed, for some slice such as the default slice or slices for eMBB, there might not be the need for UE performance reporting, as UE performance in these cases is best effort

- some of the slices the UE is using in the target gNB may not be available or supported in the source gNB. In this case the source gNB would receive per slice performance information that are not useful for AI/ML purposes because they cannot be compared with any slice performance at the source

Observation 1: Lack of an indication from source gNB to Target gNB of the slices for which UE performance is needed implies signalling from target to source of unnecessary and useless per slice performance information

To avoid unrequested transferring of per slice UE Performance reports from the target gNB to the source gNB we propose adding the list of slices for which the per slice UE performance reporting is requested by the source gNB into the UE Performance Collection Configuration IE as shown below:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond |
| Slice To Report List for UE Performance |  | 0..1 |  | S-NSSAI list for which UE performance measurements are requested. |
| >Slice To Report Item for UE Performance |  | 1 .. < maxnoofBPLMNs > |  |  |
| >>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >>S-NSSAI List |  |  | 1 |  |
| >>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  |

Proposal 1: RAN3 to agree adding a list of slices to the UE Performance Collection Configuration IE of the DATA COLLECTION REQUEST message, to enable the source gNB to indicate to the target gNB which are the slices for which per slice UE Performance reporting is requested.

## Calculating UE Performance feedback

The discussions on per slice UE Performance reporting have started at RAN3#125 when the following working assumption was turned into an agreement:

WA turned into agreement: Slice UE performance needs to be introduced, while the granularity to support slice UE performance can be further checked in WI phase

Starting with RAN3#125-bis the focus of the discussions was set on defining the granularity of per slice UE Performance and the following working assumption and FFS was made during the meeting:

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

FFS if other finer granularity of UE performance is feasible.

At RAN3#126 the working assumption above was challenged by both, proposals aiming at defining a finer granularity for per slice UE Performance such as, e.g. per slice per QoS flow, and also by proposals claiming that a finer granularity is not needed. Finaly the following agreement was captured during the meeting:

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

At RAN3#127 meeting the following agreement was captured:

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases ...

In light of this agreement and based on the previous meeting discussions and further considerations, we came to the conclusion that a per slice, i.e., per S-NSSAI, granularity for per slice UE Performance, is enough.

Finally, at RAN3#127-bis meeting the following agreement was captured:

Per S-NSSAI per UE performance metrics is reported as part of the Slice UE Performance IE in the Data Collection Update message. An FFS on how the metrics are calculated will be added in the TP. If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement.

It is understood that SA5 is discussing on the details of how to enhance their specifications to better describe how per S-NSSAI metrics are calculated.

We therefore propose to wait for SA5 to clarify in their specification such aspects and to simply reference the measurement definitions in SA5. Alternatively, RAN3 can send an LS top SA5 asking SA5 to define how per S-.NSSAI metrics are calculated and reference the SA5 specifications accordingly.

It should be avoided that RAN3 defines a new way to derive per S-MNSSAI measurements because this may contradict any enhancements SA5 applies to their specifications when it comes to the description of per S-NSSAI measurements.

Observation 2: RAN3 should avoid defining how per S-NSSAI metrics are calculated as this may contradict any current or future description on per S-NSSAI metrics in SA5 specifications

Proposal 2: RAN3 to wait for SA5 to progress on details on how to calculate per S-NSSAI metrics. Alternatively, RAN3 to draft an LS to SA5 asking SA5 to define how per S-NSSAI metrics are calculated.

# Conclusions

In this contribution we discuss the details of per slice UE Performance reporting. Throughout the discussion we derived the following observations  and proposals:

Observation 1: Lack of an indication from source gNB to Target gNB of the slices for which UE performance is needed implies signalling from target to source of unnecessary and useless per slice performance information

Proposal 1: RAN3 to agree adding a list of slices to the UE Performance Collection Configuration IE of the DATA COLLECTION REQUEST message, to enable the source gNB to indicate to the target gNB which are the slices for which per slice UE Performance reporting is requested.

Observation 2: RAN3 should avoid defining how per S-NSSAI metrics are calculated as this may contradict any current or future description on per S-NSSAI metrics in SA5 specifications

Proposal 2: RAN3 to wait for SA5 to progress on details on how to calculate per S-NSSAI metrics. Alternatively, RAN3 to draft an LS to SA5 asking SA5 to define how per S-NSSAI metrics are calculated.

The proposals above are reflected in the Text Proposal on AI/ML enabled Network Slicing for (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN provided in R3-253419.

# References

[1]	3GPP TS 38.413 V18.4.0 (2024-12)