3GPP TSG-RAN WG3 Meeting #128	R3-253421

St Julian’s, Malta, May 19th – 23rd, 2025

Title:	(TP to TS 38.401) AI/ML support

Source:	Ericsson, FiberCop, Verizon, Jio Platforms, Deutsche Telekom, BT

Agenda item:	11.2

Document Type:	Discussion and agreement

# 1. Introduction

In this paper, we present a TP to TS 38.401 for Support of AI/ML for NG-RAN based on the agreements reached in the previous meetings.

The TP aims at covering support for al use cases developed in Rel19.

# 2. Discussion

In the following we gather the agreements made in the previous meetings regarding the support for AI/ML for NG-RAN in the case of the split architecture.

From RAN3#125-bis we note the following agreements:

AI/ML enabled Coverage and Capacity Optimization:

WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

Introducing separate time information for future coverage state and predicted CCO issue.

R18 leftovers Split Architecture Support:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

From RAN3#126 we note the following agreements:

AI/ML enabled Coverage and Capacity Optimization:

Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1.

Introducing new IE for Future Coverage Modification Notification over F1.

Introducing new IE for predicted CCO assistance information over F1.

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn.

R18 leftovers Split Architecture Support:

In split architecture, the Energy Cost of gNB is the sum of the Energy Cost of its gNB-DUs.

Mapping rule is only configured at gNB-DU and shall be unified for all the gNB-DUs within a defined area.

From RAN3#127 we note the following agreements:

R18 leftovers Split Architecture Support:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

From RAN3#127-bis we note the following agreements:

R18 leftovers Split Architecture Support:

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level.

WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

Change the agreement: Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

To the following WA:

WA: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

Based on the above we propose RAN3 to agree to the TP for TS 38.401 that can be found in Annex I.

Proposal 1: RAN3 to agree to the TP for TS 38.401 that can be found in Annex I.

# 3. Conclusions

Based on the discussion in this paper the following proposal was made.

Proposal 1: RAN3 to agree to the TP for TS 38.401 that can be found in Annex I.

# 4. Annex I

# TP to TS 38.401 - Support of AI/ML for NG-RAN

-------------------------------------- START OF CHANGES --------------------------------------

## 7.11	Support of AI/ML for NG-RAN

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

### 7.11.x	Data Collection and Reporting

The following information can be configured to be reported by the gNB-DU:

-	Energy Cost (EC);

-	UE performance feedback.

The following information can be configured to be reported by the gNB-CU-UP:

-	UE performance feedback.

The collection and reporting of Energy Cost is configured through the Resource Status Initiation procedure, while the actual reporting is performed through the Resource Status Reporting procedure. The Packet delay measured for UE Performance feedback is sent from gNB-DU to gNB-CU-UP via the F1-U ASSISTANCE INFORMATION DATA frame.

For CCO, , a gNB-DU can receive in a GNB-CU CONFIGURATION UPDATE message the Predicted CCO Assistance Information including a predicted CCO issue, the Time for Predicted CCO Issue, the predicted affected cells and SSBs and Future Coverage Modification Notification for cells not served by the gNB-DU. The gNB-DU indicates to the gNB-CU in a GNB-DU CONFIGURATION UPDATE message a Future Coverage Modification Notification including Future Coverage State for cells and SSB under its control, and associated Time for Future Coverage Modification.

The collection of UE performance feedback is triggered upon request, at successful establishment of a UE context (for a gNB-DU) and/or at successful establishment of a bearer context (for a gNB-CU-UP). The transfer of UL/DL UE throughput and UL/DL packet delay from gNB-CU-UP to gNB- CU-CP is done via Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface.

-------------------------------------- END OF CHANGES --------------------------------------