3GPP TSG-RAN WG3 Meeting #128	R3-253461

Malta, MT, 19 – 23 May 2025

Agenda item:	20.2

Source:	Ericsson, Xiaomi, CATT, Nokia

Title:	(TP for TS 38.455) Support of Sample-based measurement (case 3b)

Document for:	other

# 1	Introduction

This contribution addresses RAN3 impacts for sample-based measurements in Rel-19 AI/ML-based positioning (Case 3b).

In last RAN3 meeting, it has been agreed that the LMF signals the parameters of the sample-based measurements to gNB. This contribution focuses on the details of the measurement request and report to support the sample-based measurements and provide TP for TS 38.455 in the Annex.

# 2	Open issues

### New measurement types

Based on the RAN1 agreements, the AI-ML measurements are these two Measurements Type-A and Type-B with their associated parameters.

Agreement (RAN1#120) For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b).

Type-A are legacy path-based measurements, with parameters being the path info. These parameters are already supported in the NRPPa spec from a request and reporting perspectives.

Type B are NEW sample based measurements, with parameters being the Channel Response Window Size (Nt), Channel Response Number (Nt’) and granularity factor (k), which were introduced to NRPPa BL CR last meeting in [1].

The sample-based measurements consist of time-domain channel response, optionally coupled with a power information

Agreement(RAN1#116) - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information.

This new type of information in general is dedicated to Rel-19 AI-ML positioning sample-based UL-RTOA. RAN3 does not need however to specify this use for AI-ML purpose. Instead, it can be defined as a new Measurement Type codepoint in NRPPa, for sample-based measurement including the timing and optionally the power information.

Proposal 1: RAN3 agree to introduce a generic sample-based UL-RTOA (FFS name) codepoint and associated IE in the TRP Measurement Type including the list of channel responses, where each channel response item includes timing information and optionally power information. The details are pending to RAN1.

Similarly to how LMF can request that the path-based measurement should optionally include power info (UL-SRS-RSRPP) during a measurement request, RAN3 can enable this same logic where the LMF requests gNB for the associated power info of sample based measurement by adding a bit in the Measurement Characteristics Request Indicator IE. Otherwise, the LMF may not have all required information needed to perform model training.

9.2.81  Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: UL SRS-RSRPP in Additional Path Fourth Bit: Multiple UL AoA in Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Tenth bit: Mobile TRP location information Eleventh bit: SRS bandwidth aggregation used for joint UL positioning measurement. Twelfth bit: Aggregated Positioning SRS resources IDs used for joint UL positioning measurement. Thirteen bit (NEW): UL SRS-RSRPP in sample-based UL-RTOA Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

Proposal 2: RAN3 to introduce a new bit in the Measurement Characteristics Request Indicator IE to allow LMF to request for power info in the sample-based UL-RTOA IE.

### Measurement parameters decided by gNB

According to the RAN1 agreements, the gNB may use different Nt’, Nt and/or k other than the parameters signalled from LMF. In this case, gNB needs to report the different parameters, if used, so that LMF can further process the measurements.

Agreement (RAN1#119) - The gNB/TRP may use different Nt’, Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

The new sample-based UL-RTOA IE includes the table mapping which will indicate the value of k used by gNB. The list of channel responses reported within the measurement can indicate the value of Nt’ used by gNB implicitly. The Channel Response Window Size IE however should be introduced explicitly in the sample-based UL-RTOA, to indicate the value of Nt used by gNB, but it is FFS pending RAN1 decision.

Observation 1: The reported sample-based UL-RTOA will indicate implicitly the values of k and Nt’. Whether the report should also indicate the value of Nt used by the gNB is FFS and pending RAN1

For case 3b, F1 must support same enhancement as NRPPa due to the RAN split of positioning functionality agreed since Rel-16.

Proposal 3: Add same enhancement as NRPPa impact in F1AP for case 3b.

# 3	Conclusion

In this contribution, we discussed the AI-based positioning from RAN3 perspective and have the following proposals.

Proposal 1: RAN3 agree to introduce a generic sample-based UL-RTOA (FFS name) codepoint and associated IE in the TRP Measurement Type including the list of channel responses, where each channel response item includes timing information and optionally power information. The details are pending to RAN1.

Proposal 2: RAN3 to introduce a new bit in the Measurement Characteristics Request Indicator IE to allow LMF to request for power info in the sample-based UL-RTOA IE.

Observation 1: The reported sample-based UL-RTOA will indicate implicitly the values of k and Nt’. Whether the report should also indicate the value of Nt used by the gNB is FFS and pending RAN1

Proposal 3: Add same enhancement as NRPPa impact in F1AP for case 3b.

# 4	References

- R3-252484 (TP for BL CR TS 38.455) Support of Sample-based measurement for LMF-side model (case 3b) agreed

# 5	Annex A TP to 38.455 (support of case 3b)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP, sample-based UL-RTOA (FFS) ) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Channel Response Information | O |  | 9.2.Y | Applicable to sample-based UL-RTOA only |  |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.81	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: UL SRS-RSRPP in Additional Path Fourth Bit: Multiple UL AoA in Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Tenth bit: Mobile TRP location information Eleventh bit: SRS bandwidth aggregation used for joint UL positioning measurement. Twelfth bit: Aggregated Positioning SRS resources IDs used for joint UL positioning measurement. Thirteen bit: UL SRS-RSRPP in sample-based UL-RTOA. Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >>Sample-based UL-RTOA |  |  | 9.2.X |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.X	Sample-based UL-RTOA (FFS)

This information element contains the Channel Response of the sample-based UL-RTOA.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Channel Response List |  | 1 |  |  |
| >Channel Response Item |  | 1..<maxnoofChannelRes> |  |  |
| >>CHOICE Timing Information | M |  |  |  |
| >>>k0 |  |  | INTEGER (0.. 1970049) | TS 38.133 [16] |
| >>>k1 |  |  | INTEGER (0.. 985025) | TS 38.133 [16] |
| >>>k2 |  |  | INTEGER (0.. 492513) | TS 38.133 [16] |
| >>>k3 |  |  | INTEGER (0.. 246257) | TS 38.133 [16] |
| >>>k4 |  |  | INTEGER (0.. 123129) | TS 38.133 [16] |
| >>>k5 |  |  | INTEGER (0.. 61565) | TS 38.133 [16] |
| >>Power Information | O |  | UL SRS-RSRPP 9.2.72 |  |
| Channel Response Window Size (FFS) | O |  | ENUMERATED (32, 64, 128, …) | It represents the window size over which to select channel response values. |

| Range bound | Explanation |
| --- | --- |
| maxnoofChannelRes | Maximum no of channel response. Value is 24 |

### 9.2.Y	Channel Response Information

This IE contains information about the requested sample-based UL-RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Channel Response Window Size | M |  | ENUMERATED (32, 64, 128, …) | Represents the window size over which channel response values are selected. |
| Channel Response Number | M |  | ENUMERATED (8, 16, 24, …) | Represents the number of channel responses selected over the channel response window size. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< change end>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>