3GPP TSG-RAN WG3 Meeting #128	R3-253471

Malta, MT, May 19th – 23rd, 2025

Agenda Item:	15.2

Source:	CATT

Title:	Discussion about IoT NTN

Document for:	Decision

- Introduction

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

In this paper, we provide our initial discussion about S&F scenario, and start with the attachment procedure.

- Discussion

2.1 S&F operation

In the S&F operation, the precondition is the regenerative payload is supported by the eNB on board, and considering whether exist any S1AP impact.

In split MME case, SA2 introduced a new metric, list of satellite ID, which is generate by MME for indicating the UE can re-attempt attach to which satellites. Similar, in Full CN case, S&F monitoring list is also introduced for the same purpose. One open issue is whether needs to send the satellite list information to the eNB explicitly. We give a further clarification on this issue.

Usually, the UE obtains the ephemeris information from the broadcasting SIB information, including SIB31, SIB32, and SIB33.

- SIB 31 contains satellite assistance information for the serving cell

- SIB 32 contains satellite assistance information for prediction of discontinuous coverage.

- SIB 33 contains satellite assistance information for neighbour cells.

stemInformationBlockType32 information element

-- ASN1START

SystemInformationBlockType32-r17 ::= SEQUENCE {

satelliteInfoList-r17				SatelliteInfoList-r17	OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[	satelliteInfoList-v1800			SatelliteInfoList-v1800	OPTIONAL	-- Need OR

]],

[[	satelliteInfoList-v1830			SatelliteInfoList-v1830	OPTIONAL	-- Need OR

]]

}

SatelliteInfoList-r17 ::=	SEQUENCE (SIZE (1..maxSat-r17)) OF SatelliteInfo-r17

maxSat-r17					INTEGER ::= 4	-- Maximum number of satellites

According to RAN2 definition above, limited by the size of SIB, the max number of ephemeris set as 4. If this satellite information is transfer to UE by NAS, the eNB have no idea of which emphases shall be broadcast to the UE. As eNB only can broadcast 4 ephemeris, the eNB only can do best effort broadcasting. By this way, from the network side it will increase the load for broadcasting unnecessary information, form the UE side, it may only get the satellite list from MME, but don’t have the ephemeris of the incoming satellite, and cannot attach successful.

Proposal 1: MME transfers the list of satellite and the S&F monitoring list of satellites IDs to the eNB explicitly for assisting ephemeris broadcast.

  - Regeneration payload

The enhance in NR NTN about regeneration payload are applicable for IoT NTN, including the TNL Functionality, TNL Change, and FLSO.

Proposal 2: introduce the same enhancement as NR NTN for TNL Functionality, TNL Change, and FLSO in IoT NTN.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: MME transfers the list of satellite and the S&F monitoring list of satellites IDs to the eNB explicitly for assisting ephemeris broadcast.

Proposal 2: introduce the same enhancement as NR NTN for TNL Functionality, TNL Change, and FLSO in IoT NTN.

4. Reference

[1]	TR 23.700 Study on integration of satellite components in the 5G architecture.

[2]	S2-2410918 Reply LS on Support of Regenerative-based Satellite Access