3GPP TSG RAN WG3 Meeting #128	R3-253476

Malta, MT, 19-23 May 2025

Agenda Item:	11.4

Title:	Discussion on the SA5 reply LS on the Continuous MDT

Source:	Ericsson, InterDigital, Deutsche Telekom, BT

Document for:	Discussion

# Introduction

This paper continues the discussion on Continuous MDT and proposes solutions that are based on existing fields within the Trace Records specified by SA5 for MDT, in line with recent agreements in RAN3 and the guidance that has received in the latest SA5 reply LS [2].

# Discussion

During the RAN3#127 meeting the discussions on Continuous MDT have continued and resulted in the following agreement:

RAN3 assumes a solution for continuous MDT should minimise impacts on the current Management Based MDT framework.

Furthermore, the group concluded that, while it would be beneficial for RAN3 to discuss solutions to enable continuous MDT, it is essential to involve in this discussion SA5, which is ultimately the group that owns Stage 2 description and OAM-related Stage 3 description for the MDT functionalities. It was agreed to send an LS to SA5 [1] to seek guidance on the continuous MDT solutions.

SA5 has discussed the RAN3 LS and the Reply LS has been received [2]. In particular, in its Reply LS, SA5 provided responses to the set of questions posed by RAN3 on guidance on possible solutions for Continuous Management Based MDT. In particular, the following comments/answers have been provided:

- For Measurement Continuity Problem, regarding RAN3 agreements:

- RAN3: “- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.”

Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

- RAN3: “- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.”

Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

- RAN3: “- A solution should minimise impacts on the current Management Based MDT framework.”

Answer from SA5: SA5 agrees that the solution should minimise impacts on the current Management Based MDT framework.

- For Trace Correlation Problem,

  - RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.”

Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

This contribution discusses the SA5 responses in the received Reply LS and suggests a way forward based on the received input.

## Measurement Continuity Problem [item a]

The guidance from SA5 as expressed above [items in a)] imply that solutions for continuous MDT should:

- Rely on the OAM to configure the RAN with a Management Based MDT configuration to be used for Continuous MDT [item a1)]

- Reuse the Management Based MDT framework [item a3)]

As discussed extensively in the past the measurement continuity problem is to ensure that the same UE is selected for collection of MDT measurements during sessions in the same RRC state, across different RRC states and across RRC_Connected mobility [item a2)]

The main task to address is to enable means to consistently select the UE across RRC state changes and across mobility between gNBs.

As per current agreements and the SA5 guidance in items a), the OAM should be in charge of configuring management based MDT for continuous MDT. Therefore, a management based MDT configuration shall be used to convey information that let the RAN understand that the process to configure is Continuous MDT. In line with this logic, such configuration can be achieved by reusing existing parameters and operations, such as the existing Trace References (TR) identifier. 
Indeed, the OAM could select a TR in a specific range or it could use a specific pre-defined Trace Reference prefix for the management based MDT Trace Jobs issued for Continuous MDT. If continuous MDT is needed, a TR with one of the pre-defined values or prefix is sent from the OAM to the RAN as part of the TraceJob. In such a case, the receiving end of the TraceJob, the gNB, will understand from the TR prefix or from the TR value that the management based MDT process corresponds to a continuous MDT configuration.

Such a choice fulfils two agreements established in SA5 and RAN3, namely:

- OAM triggered configuration mechanisms to enable Continuous MDT reuse management based MDT configurations.

- Impacts on the current Management Based MDT framework is minimised.

An example of applying such high level principle to the problem at hand is presented below, showing how such a simple way to reusing the existing identifiers in the Trace Record can achieve the purpose of UE selection continuity during RRC state changes:

  - The NG-RAN receives from the OAM a Management Based MDT configuration, including a specific Trace Reference ID. From such trace Reference ID the RAN deduces that the Management Based MDT configuration is for Continuous MDT.

  - The RAN selects a UE in RRC _Connected for the Continuous MDT measurement process. Continuous MDT measurements are collected while the Ue is in RRC_Connected.

  - While the UE is in RRC_CONNECTED, the NG-RAN would signal to the selected UE the continuous logged MDT configuration. The serving gNB would include in the logged MDT configuration the TR that indicates that the trace is for Continuous MDT, assuming that SA5 specified a range of TRs (e.g. via the allocated prefix) that is specific for Continuous MDT.

  - When the UE goes to RRC_IDLE/RRC_INACTIVE, it measures and logs measurements. These logs include the TR assigned by the OAM and the Trace Recording Session Reference (TRSR) assigned by the gNB.

  - When the UE goes to RRC_CONNECTED and it reports Logged MDT logs, such logs would include the Continuous MDT TR assigned by the OAM and the TRSR assigned by the gNB.

  - The receiving gNB understands that the UE has to be configured for Continuous MDT by checking the TR in the Logged MDT logs, which is specific for Continuous MDT.

  - For cases of UE mobility during RRC_CONNECTED and while a Continuous MDT process is running for the UE

    - Either the source gNB signals to the Target gNB a Continuous MDT TR, as part of the Handover Signalling, or the source gNB signals to the target gNB, as part of the Handover Signalling, an explicit “Continuous MDT indication”. In both cases, the target gNB understands that the UE has been configured with Continuous MDT at source gNB and it can select it again.

Proposal 1: RAN3 to agree to the following principle to solve the measurement continuity problem in accordance to the received SA5 guidance:

- The OAM provides to the RAN specific values of the existing Trace Record IE, which indicates that the Management Based MDT process is for Continuous MDT

Proposal 2: RAN3 to agree that Source gNB signals to Target gNB, as part of the Handover Signalling, an indication that the UE has been selected for Continuous MDT, such as the TR assigned to the UE. Target gNB uses this information to re-select the UE for Continuous MDT as proposed in [2].

In order to clarify the configuration requirements on OAM for continuous MDT and respond to requirement expressed in item a1) above, it is suggested to introduce the continuous MDT process description above in the relevant RAN stage 2 specification TS37.320 as proposed in [3].

Proposal 3: RAN3 to agree to introduce the continuous MDT process description in TS37.320 as proposed in [3].

## Trace Correlation Problem [item b)]

During the Rel-18 discussions it was acknowledged that Management Based MDT does not offer the possibility to merge Immediate and Logged MDT data collected by the same UE across different RRC states and across different serving gNBs. In order to do this an identifier would be needed that is consistently maintained across Management Based MDT data collection in different RRC states and across different gNBs, and that is associated to the collected MDT data.

In a previous LS exchange with SA5, the need of such identifier as a requirement for the realization of Continuous MDT has been raised [6] and in its response SA5 has acknowledged the benefits of such requirement [7]. In particular:

“SA5 agree that there is a benefit to have a (temporary) identifier in the management-based MDT measurement report. This identity shall be unique within the lifetime of the measurement session. So, some data analytics function (e.g., AIML related function) can have a better understanding that the collected data sets are measured from the same source. Therefore, it would be good to provide some kind of identifier to indicate the measurement was done in the same or different UE.”

It is important to understand that a Continuous MDT correlation identifier would enable to achieve continuity of MDT measurements and resolve trace correlation.

The question at hand is how to select a proper Correlation ID that would be tagged to the Trace Records associated to the same Continuous MDT process and to the same UE.

To address this, it could be helpful to check the existing information carried out by the Trace Record Header. According to section 5.2.2 of TS32.423

### 5.2.2 Trace Record Header The trace record header contains the common fields as specified in the Table 5.2.2-1, in addition it may also contain vendor specific extensions. Table 5.2.2.1 : Common fields in the trace record header Trace Record Header field name; Description. timeStamp (M); Time stamp (in milliseconds since Epoch) of when the streaming trace record is produced internally in the Producer encoded as (64 bit integer). nfInstanceId (M); Unique id of the Producer NF instance that produced this trace record represented by a (String). nfType (M); Type of the Producer NF that produced this trace record represented by a (String). traceReference (M); Trace Reference (see clause 5.6 of 3GPP TS 32.422 [23]) (represented by a 3 bytes octet string). traceRecordingSessionReference (M); Trace Recording Session Reference (see clause 5.7 of 3GPP TS 32.422 [23]) represented by a (2 byte octet string. See Note 1.). traceRecordTypeId (M); Identifier of the trace record type (see clause 5.2.4 for details) represented by an ENUM with the following values: NORMAL TRACE_SESSION_START, TRACE_SESSION_STOP, TRACE_RECORDING_SESSION_START, TRACE_RECORDING_SESSION_STOP, TRACE_STREAM_HEARTBEAT. TRACE_RECORDING_SESSION_NOT_STARTED, TRACE_RECORDING_SESSION_DROPPED_EVENTS, TRACE_FILE_OPEN, TRACE_FILE_CLOSE, TRACE_FILE_ABNORMAL_CLOSED TRACE_RECORDING_SESSION_THROTTLED_START TRACE_RECORDING_SESSION_THROTTLED_STOP (See Note 2).. ranUeId (O); RAN defined id to represent a UE (8 byte octet string. See Note 3.). payloadSchemaURI (O); URI identifying the schema to be used in order to decode the payload represented by a (String. See Note 4.). globalGnbId (CM); Global gNB ID, as defined in [23]. Applied for trace reported by gNB-CU-CP, gNB-CU-UP, gNB-DU.. vendorExtension (O); Vendor-specific extension(s) represented by a (Arraylist of String. See Note 5.). - NOTE 1: The traceRecordingSessionReference must be present for the Streaming Trace Records with non-zero size payload where the payload carries data captured for a Trace Recording Session and in administrative messages related to a Trace Recording Session (e.g. "Trace Recording Session Start" or "Trace Recording Session Stop"). - NOTE 2: The traceRecordTypeId with value "NORMAL" is used for Streaming Trace Records that do not carry an administrative message. - NOTE 3: The ranUeId field is present in the trace record header when the identifier is supported by RAN. If RAN UE Id (see 3GPP TS 38.463 [25] and 38.473 [26]) has been captured in the traced signaling messages that value is used. - NOTE 4: The payloadSchemaURI is not required for Streaming Trace Records with payload of zero-size, or payload using common payload format (e.g. used to convey Streaming Trace administrative messages). - NOTE 5: The vendorExtension is typically a generic list of key-value pairs..

Conclusion 1: Some information, like the TR/TRSR, are always present in the trace records and could be exploited as correlation identifiers without the need to modify the Trace Recording header. Other information, like the RAN UE ID, are optionally present in the Trace Record and could also be considered as potential candidate for a correlation identifier.

It is worth noting that deriving a correlation identifier from the parameters contained in the Trace Record has a number of advantages, among which the following:

- Trace Record parameters are already included in Trace Records by the gNB today. Reusing Trace Records parameters minimizes the impact on the current MDT design, which is in line with the agreements taken by RAN3 and confirmed by SA5 in their LS in [2]

- Reusing Trace Record parameters ensures avoiding impacts on the UE, which has been agreed by RAN3 as a key principle for continuous MDT

- Reusing Trace Record parameters maintains the MDT functionality in the RAN and in the OAM. This avoids impacts on other systems. Making MDT depend on support from new systems/nodes creates further dependency and it generates higher impact

In their reply LS, SA5 agreed the following:

“SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE” [item b]

However, SA5 asks RAN3 the following:

“SA5 would like RAN3 to elaborate the details of how to avoid overlapping”

To address SA5´s request, the following two main ssolutions for Trace correlation with reuse of TR/TRSR and avoiding overlapping are suggested:

- Solution A: Based on TR/TRSR

- A TraceJob is sent to all participating RAN nodes at the time of Continuous management-based MDT initiation. The TraceJob includes a specific Trace Reference identifier (i.e. TR) to indicate this is a Continuous management-based MDT procedure. The same TR is used in all TraceJob signalled to the participating RAN nodes. A dedicated TRSR range is assigned to each participating RAN node, which is used by RAN node during Trace Recording Session Reference (TRSR) assignment. It should be noted that the TRSR has a length of 2 bytes, giving space for over 65 thousand values. If for example the gNBs participating in the Continuous MDT process are 10, this ensures to be able to trace over 6 thousand UEs per node, which is a rather large number.

- When a UE is selected for the continuous management-based MDT, and a Trace Recording session is initiated, the gNB allocates a TRSR from the received range. The same TRSR will be used for the whole duration of the Continuous MDT trace of a UE for correlation purpose. Namely, the TRSR is kept for the UE even if the UE moves to a different participating node.

- If the selected UE is handed over to a participating RAN node, the target RAN node recognize the UE as a Continuous management-based MDT UE based on its TR. Therefore, the UE will be reselected in the target RAN node for the Continuous management-based MDT.

  - If the UE has been configured with Immediate Continuous management-based MDT and the UE moves to a different RAN node, the source RAN node may signal to the target RAN node, e.g. as part of the Handover Request message, the TR and TRSR assigned to the UE. If the target RAN node is a participating RAN node, it understands that the UE was selected for Continuous management-based MDT (from the received TR). Hence the target RAN would select the UE again for further Continuous management-based MDT measurements. The target RAN node assigns to the UE the TRSR received from the source RAN node.

  - If the UE has been configured with Logged Continuous management-based MDT, and the UE moves to RRC_Idle mode or RRC_Inactive state, the UE will collect measurements corresponding to the measurement configuration the UE has received for the logged continuous management-based MDT process. Such measurements will be stored by the UE together with the assigned TR, and TRSR.     
Once the UE moves to RRC_Connected mode, the UE will report to the serving RAN node the logs of measurements collected while in RRC_Idle or RRC_Inactive state. If the serving RAN node is a participating RAN node, it will therefore identify that the UE was selected for Continuous management-based MDT by looking at the reported TR. The serving RAN would therefore further select the UE for Continuous management-based MDT, i.e., the UE will be selected for further continuous management-based MDT measurements. The serving RAN node will assign to the UE the same TRSR contained in the reported MDT logs.

- Trace Correlation is achieved by the assigned TR/TRSR which can identify that the trace records belong to the same selected UE. The same TRSR will be used for the whole duration of the Continuous management-based MDT trace of a UE.

- Solution B: Based on List of TRs/TRSR

  - Similar as solution A) with the following differences

      - each TraceJob signalled to a participating RAN node will have a dedicated TR, together with a TR list. The TR list contains all dedicated TRs being used in the same continuous MDT Job in each participated gNB. This TR list is used by the RAN node to deduce that a UE establishing connection or handing over from a neighbour node needs to be configured with continuous MDT. 
By assigning one TR per MDT Job per RAN node, the number of UEs that can be configured with Continuous MDT increases to over 65 thousands per node. If the continuous MDT process includes 10 participating RAN nodes, the total maximum number of UEs that can be configured with Continuous MDT would be approximately 65000*10, namely 650000, which is a considerable number.

      - Each selected UE has a TRSR assigned. The TR/TRSR combination is unchanged for a selected UE during the lifetime of this continues MDT.

      - At handover, the target gNB recognizes the UE is a continues MDT UE from its TR. The target gNB receives from the source gNB the TR+TRSR for the UE and it reassigns to the UE the same TR+TRSR.

      - When the UE moves from RRC_Idle/Inactive to RRC_Connected and it reports Logged MDT logs, the new serving gNB detects the TR+TRSR for the UE and re-assigns the same TR+TRSR for the immediate continuous MDT process for the same UE.

Proposal 4: Send an LS reply to SA5 to suggest re-using TR, TRSR, and propose Solution A (minimal impact) or Solution B to address the Trace Correlation Problem.

## AMF based solutions

During past RAN3 meetings, solutions were proposed where the Temporary Identifier for Continuous MDT report correlation is assigned and stored by the AMF. We believe that such solutions shall be avoided due increased signalling complexity, dependency and impact on the AMF, and shifting of MDT functionalities from the OAM/RAN to the CN. In particular. Consequences of such solutions could be as follows:

- There would be an impact on the AMF, and a dependency of the whole feature on AMF support: no AMF support, no feature.

- There would be a dependency on the AMF load and whether the AMF can keep a continuous MDT context active for the UE for a long time. If the AMF has issues with e.g. load, continuous MDT will fail.

- MDT is a process owned by the OAM, where the identifiers assigned to the MDT reports should be controlled by the OAM. If the AMF controls such identifiers, the process moves under AMF control, which implies a shift of functionalities from OAM to CN and that is not in line with the MDT functionality. Note that the OAM system is tightly coupled with the RAN for perfect interoperability, while the AMF is often from a different vendor than the RAN, creating potential interoperability problems for the whole feature.

- At change of AMF the identifier provided by the source AMF can no longer be used. Continuity of measurements is interrupted.

Furthermore, it should be pointed out that a solution based on assignment of identifiers by the AMF for the correlation and continuity problem was taken in SA5, but there were strong objections by a number of companies in the involvement of the AMF in management based MDT. Hence, this approach should be avoided as it would anyhow find resistance in SA5.

Proposal 5: Avoid the complexity and the functional changes caused by involving AMF and the Core Network in the Continuous MDT operation.

# Conclusions and Proposal

In this contribution the SA5 response LS is being discussed and the following proposals and conclusions were derived:

Proposal 1: RAN3 to agree to the following principle to solve the measurement continuity problem in accordance to the received SA5 guidance:

- The OAM provides to the RAN specific values of the existing Trace Record IE, which indicates that the Management Based MDT process is for Continuous MDT

Proposal 2: RAN3 to agree that Source gNB signals to Target gNB, as part of the Handover Signalling, an indication that the UE has been selected for Continuous MDT, such as the TR assigned to the UE. Target gNB uses this information to re-select the UE for Continuous MDT as proposed in [2].

Proposal 3: RAN3 to agree to introduce the continuous MDT process description in TS37.320 as proposed in [3].Conclusion 1: Some information, like the TR/TRSR, are always present in the trace records and could be exploited as correlation identifiers without the need to modify the Trace Recording header. Other information, like the RAN UE ID, are optionally present in the Trace Record and could also be considered as potential candidate for a correlation identifier.

Proposal 4: Send an LS reply to SA5 to suggest re-using TR, TRSR, and propose Solution A (minimal impact) or Solution B to address the Trace Correlation Problem.

Proposal : Avoid the complexity and the functional changes caused by involving AMF and the Core Network in the Continuous MDT operation.

# References

- R3-250891, LS on Continuous MDT, RAN3

- R3-253024/S5-252114, Reply LS to LS on Continuous MDT, SA5

- R3-253477, (TP for AI/ML BLCR to TS38.423) Continuous MDT collection targeting the same UE across RRC states

- R3-253478, (TP to 37.320) - Continuous MDT collection targeting the same UE across RRC states

- R3-243941, Reply LS to SA5 on improved KPIs involving end-to-end data volume transfer time analytics, RAN3.

- RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN.

R3-250464, [DRAFT] LS_to_SA5_Reply_LS_On_Imporved_KPIs, Ericsson (to be RAN3)