3GPP TSG-RAN WG3 Meeting #128	R3-253489

St. Julian’s, Malta, 19th – 23rd May, 2025

Title: 	Discussion on additional finer granularities for UE performance in AI/ML-based Network Slicing

Source: 	Huawei, Jio Platforms, Deutsche Telekom

Agenda item:	11.2

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing.

This contribution analyzes further the technical enablers for the AI/ML-based Network Slicing use case to be introduced over the Xn interface, and provides proposals for discussion based on the outcome of past RAN3 meetings.

# 2. Discussion

In the last RAN3 meeting the following agreements were achieved [4]:

Stage3 signalling design: Introduce the measured CCCV within the predicted slice available capacity. Slice UE performance: For Slice UE performance request, take Opt2: Extend Existing Report Characteristics Bitmap. Extending the existing Node Measurement Failed Report Characteristics IE to include slice-level UE performance metrics for indicating the failed result for slice-level UE performance metrics. Add Slice UE performance IE in the DATA COLLECTION UPDATE message. Per S-NSSAI per UE performance metrics is reported as part of the Slice UE Performance IE in the Data Collection Update message. An FFS on how the metrics are calculated will be added in the TP. If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement.

We will discuss further the granularity of the UE performance feedback for the AI/ML-based Network Slicing use case.

## 2.1 Additional finer granularities of UE performance feedback

So far RAN3 agreed that per-slice per-UE UE performance can be requested to be reported [5]:

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases ...

Other possible options for the UE performance finer granularity are listed as follows:

- Option 1: per PDU session per slice (S-NSSAI)

- Option 2: per-QoS-flow-group

In RAN3#125bis meeting [6], the majority of companies preferred to have UE performance to be reported on a per PDU session per slice granularity, meaning that UE performance should be collected and reported for each PDU session belonging to a certain slice in use by the UE. This led to the following Working Assumption, with the possibility to further discuss about the feasibility of other granularities [6]:

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node. FFS if other finer granularity of UE performance is feasible.

However, concerns were raised on the actual feasibility of such granularity. Our understanding is that most of the UE performance metrics are typically measured on a per-DRB per-UE granularity, and multiple DRBs can be mapped into the same PDU session, hence it seems there is currently no standardized way to determine per PDU session measurements so to ensure that source and target gNBs apply the same methodology for deriving per PDU session per slice UE performance feedback before and after an AI/ML-based mobility action while the UE is using a certain slice, hence making the comparison between the two per PDU session measurements meaningful.

For Option 2 (per-QoS-flow-group UE performance), it is still unclear whether this could actually be implemented. In our understanding the “QoS-flow-group” represents the set of QoS flows that are mapped to a certain DRB. Therefore, in the source gNB side, the gNB can measure per-DRB per-UE UE performance metrics and then implicitly derive the per-QoS-flow-group measurement (which corresponds exactly to the per DRB measurement). However, after the AI/ML-based mobility action for the UE using a certain slice, the target gNB may decide to map the set QoS flows (mapped into a single DRB in the source gNB) into multiple different DRBs. In other words, we think that only in case the target node decides to map the same set of QoS flows into a single DRB (as done by the source gNB) the comparison of UE performance before and after the AI/ML-based mobility action could be possible. However, it is kind of a corner case that this could occur, and for sure the target node cannot be forced to apply the same QoS-flows-to-DRB mapping as used by the source gNB.

Based on the above, and considering the limited time to complete the Rel-19 WI, we think that the discussion on whether further finer granularities need to be introduced can be deferred to future 3GPP releases.

Proposal: RAN3 to agree that the discussion on whether further finer granularities need to be introduced can be deferred to future 3GPP releases.

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

Proposal: RAN3 to agree that the discussion on whether further finer granularities need to be introduced can be deferred to future 3GPP releases.

# 4. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- 3GPP RAN3#125bis Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)