3GPP TSG-RAN WG3 #128	R3-253510

# St. Julian’s, Malta, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1284 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of aerial UE’s altitude information reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3, TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on TS 23.256, for the aerial UEs, SA2 specified new procedures to enable the altitude reporting from NG-RAN to CN. Moreover, RAN3 agreed in RAN3#127 to reuse the Location Reporting Control and Location Report procedures to configure and perform the aerial UE’s altitude reporting, respectively, and that the altitude reporting can be configured to be periodic, threshold-based or event-based based on NG-RAN implementation. This CR follows the above mentioned agreements and introduces the necessary enhancements for the Location Reporting Control and Location Report procedures for the purpose of aerial UE’s altiture reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the following in the Location Reporting Control procedure: - new codepoints ’aerial UE altitude information reporting’ and ’stop aerial UE altitude information reporting’ for the existing Event Type IE within the Location Reporting Request Type IE of the LOCATION REPORTING CONTROL message, to indicate the start and stop of the aerial UE’s altitude reporting; - new Aerial UE Altitude Information Reporting Configuration IE within the Location Reporting Request Type IE of the LOCATION REPORTING CONTROL message, to configure altitude thresholds for reporting and, optionally, reporting periodicity; - updated procedural text and ASN.1 accordingly. Introduce the following in the Location Report procedure: - new Aerial UE Altitude Information IE within the User Location Information IE of the LOCATION REPORT message, which encodes the aerial UE’s altitude information; - updated semantics description of the existing Age of Location IE within the User Location Information IE of the LOCATION REPORT message, to indicate that the Age of Location IE is also used to indicate the point in time at which the NG-RAN node received the altitude report from the aerial UE; - updated procedural text and ASN.1 accordingly. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous release of the specification because the changes only affect the Location Reporting Control and Location Report procedures to enable aerial UE’s altitude reporting from NG-RAN to AMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The aerial UE’s altitude reporting from NG-RAN to AMF is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.1.1, 8.12.1.2, 8.12.3.1, 9.3.1.16, 9.3.1.65, 9.3.1.xx (new), 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.410 CR 0054 TS 38.423 CR 1496 TS 38.300 draftCR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of Changes ***

## 8.12	Location Reporting Procedures

### 8.12.1	Location Reporting Control

#### 8.12.1.1	General

The purpose of the Location Reporting Control procedure is to allow the AMF to request the NG-RAN node to report the UE's current location, or the UE's last known location with time stamp, or the UE's presence in the area of interest while in CM-CONNECTED state, or the aerial UE’s altitude information as specified in TS 23.501 [9], TS 23.502 [10] , and TS 23.256 [54]. The procedure uses UE-associated signalling.

#### 8.12.1.2	Successful Operation

Figure 8.12.1.2-1: Location reporting control

The AMF initiates the procedure by sending a LOCATION REPORTING CONTROL message to the NG-RAN node. On receipt of the LOCATION REPORTING CONTROL message the NG-RAN node shall perform the requested location reporting control action for the UE.

The Location Reporting Request Type IE indicates to the NG-RAN node whether:

-	to report directly;

-	to report upon change of serving cell;

-	to report UE presence in the area of interest;

-	to stop reporting at change of serving cell;

-	to stop reporting UE presence in the area of interest;

-	to cancel location reporting for the UE;

-	to report upon change of serving cell and to report UE presence in the area of interest;

-	to report the altitude information for the aerial UE;

-	to stop reporting the altitude information for the aerial UE.

If the Area Of Interest List IE is included in the Location Reporting Request Type IE in the LOCATION REPORTING CONTROL message, the NG-RAN node shall store this information and use it to track the UE's presence in the area of interest as defined in TS 23.502 [10].

NOTE: 	The NG-RAN reports the UE presence for all set of Location Reporting Reference IDs for inter-NG-RAN node handover.

If the Additional Location Information IE is included in the LOCATION REPORTING CONTROL message and set to "Include PSCell” then, if Dual Connectivity is activated, the NG-RAN node shall include the current PSCell in the report. If a report upon change of serving cell is requested, the NG-RAN node shall provide the report also whenever the UE changes the PSCell, and when Dual Connectivity is activated.

If reporting upon change of serving cell is requested, the NG-RAN node shall send a report immediately and shall send a report whenever the UE’s location changes.

If the Event Type IE is set to "stop UE presence in the area of interest", and if the Additional Cancelled Location Reporting Reference ID List IE is included in the Location Reporting Request Type IE within the LOCATION REPORTING CONTROL message, the NG-RAN node shall, if supported, stop reporting UE presence for all received Location Reporting Reference IDs.

*** Next Change ***

### 8.12.3	Location Report

#### 8.12.3.1	General

The purpose of the Location Report procedure is to provide the UE's current location, the UE's last known location with time stamp, or the UE's presence in the area of interest to the AMF, or the aerial UE’s altitude information. The procedure uses UE-associated signalling.

#### 8.12.3.2	Successful Operation

Figure 8.12.3.2-1: Location report

The NG-RAN node initiates the procedure by sending a LOCATION REPORT message to the AMF. The LOCATION REPORT message may be used as a response to the LOCATION REPORTING CONTROL message.

*** Next Change ***

#### 9.3.1.16	User Location Information

This IE is used to provide location information of the UE or the altitude information of the aerial UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 | This IE is ignored if the NR NTN TAI Information IE is present. | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >>NID | O |  | 9.3.3.42 |  | YES | reject |
| >>NR NTN TAI Information | O |  | 9.3.3.53 |  | YES | ignore |
| >>Mobile IAB-MT User Location Information | O |  | 9.3.1.260 | Indicates the user location information of a mobile IAB-MT, which is co-located with the mobile IAB-DU which serves the UE. | YES | Ignore |
| >>Aerial UE Altitude Information | O |  | INTEGER (-420..10000,…) | Corresponds to information provided in the Altitude IE, as defined in TS 38.331 [18]. Unit [meters] | YES | ignore |
| >N3IWF user location information with port number |  |  |  |  |  |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF | - |  |
| >>Port Number | M |  | OCTET STRING (SIZE(2)) | UDP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TNGF user location information |  |  |  |  | YES | ignore |
| >>TNAP ID | M |  | OCTET STRING | TNAP Identifier used to identify the TNAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the TNGF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TWIF user location information |  |  |  |  | YES | ignore |
| >>TWAP ID | M |  | OCTET STRING | TWAP Identifier used to identify the TWAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | Non-5G-Capable over WLAN device's local IP address used to reach the TWIF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >W-AGF user location information |  |  |  | Indicates the location information via wireline access as specified in TS 23.316 [34]. | YES | ignore |
| >>W-AGF user location information | M |  | 9.3.1.164 |  | - |  |
| >N3IWF user location information without port number |  |  |  |  | YES | ignore |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | - |  |

*** Next Change ***

#### 9.3.1.65	Location Reporting Request Type

This IE indicates the type of location request to be handled by the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Event Type | M |  | ENUMERATED (direct, change of serving cell, UE presence in the area of interest, stop change of serving cell, stop UE presence in the area of interest, cancel location reporting for the UE, … change of serving cell and UE presence in the area of interest, aerial UE altitude information reporting, stop aerial UE altitude information reporting) |  | - |  |
| Report Area | M |  | ENUMERATED (cell, …) |  | - |  |
| Area of Interest List |  | 0..1 |  |  | - |  |
| >Area of Interest Item |  | 1..<maxnoofAoI> |  |  | - |  |
| >>Area of Interest | M |  | 9.3.1.66 |  | - |  |
| >>Location Reporting Reference ID | M |  | 9.3.1.76 |  | - |  |
| Location Reporting Reference ID to be Cancelled | C- ifEventTypeisStopUEPresinAoI |  | Location Reporting Reference ID 9.3.1.76 |  | - |  |
| Additional Location Information | O |  | ENUMERATED (Include PSCell, ...) |  | YES | ignore |
| Additional Cancelled Location Reporting Reference ID List |  | 0..1 |  |  | YES | reject |
| >Additional Cancelled Location Reporting Reference ID Item |  | 1..<maxnoofAoIMinusOne> |  |  | - |  |
| >>Location Reporting Reference ID to be Cancelled | M |  | Location Reporting Reference ID 9.3.1.76 |  | - |  |
| Aerial UE Altitude Information Reporting Configuration | C-ifEventTypeisAerialUEAltitudeInformationReporting |  | 9.3.1.xx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofAoI | Maximum no. of areas of interest. Value is 64. |
| maxnoofAoIMinusOne | Maximum no. of areas of interest minus one. Value is 63. |

| Condition | Explanation |
| --- | --- |
| ifEventTypeisStopUEPresinAoI | This IE shall be present if the Event Type IE is set to the value "stop UE presence in the area of interest". |
| ifEventTypeisAerialUEAltitudeInformationReporting | This IE shall be present if the Event Type IE is set to the value "aerial UE altitude information reporting". |

*** Next Change ***

#### 9.3.1.xx	Aerial UE Altitude Information Reporting Configuration

This IE indicates the configuration to be handled by the NG-RAN node to configure the aerial UE to report its altitude information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Minimum Altitude Threshold | M |  | INTEGER (-420..10000,…) | Corresponds to information provided in the Altitude IE, as defined in TS 38.331 [18], and it represents the minimum threshold for which the altitude reporting is initiated when the actual altitude goes below this value. Unit [meters] |
| Maximum Altitude Threshold | M |  | INTEGER (-420..10000,…) | Corresponds to information provided in the Altitude IE, as defined in TS 38.331 [18], and it represents the maximum threshold for which the altitude reporting is initiated when the actual altitude goes above this value. Unit [meters] |
| Reporting Periodicity | O |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 20480ms, 40960ms, 1min, 6mis, 12min, 30min, …) | Corresponds to information provided in the ReportInterval IE, as defined in TS 38.331 [18], and it indicates the periodicity for the reporting of altitude information that is applied if either of the thresholds are met. |

*** Next Change ***

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

*** skip unmodified parts ***

id-UserLocationInformationN3IWF-without-PortNumber,

id-AerialUEAltitudeInformationReportingConfiguration,

id-AerialUEAltitudeInformation,

maxnoofAllowedAreas,

*** skip unmodified parts ***

maxnoofRSPPQoSFlows

FROM NGAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM NGAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Container{},

NGAP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

NGAP-PROTOCOL-IES

FROM NGAP-Containers;

*** Next Change ***

#### -- A

*** skip unmodified parts ***

AerialUEAltitudeInformation ::= INTEGER (-420..10000, ...)

AerialUEAltitudeInformationReportingConfiguration ::= SEQUENCE {

minimumAltitudeThreshold						AerialUEAltitudeInformation,

maximumAltitudeThreshold						AerialUEAltitudeInformation,

reportingPeriodicity							ReportingPeriodicity														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AerialUEAltitudeInformationReportingConfiguration-ExtIEs} }		OPTIONAL,

...

}

AerialUEAltitudeInformationReportingConfiguration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

*** Next Change ***

#### -- E

*** skip unmodified parts ***

EventType ::= ENUMERATED {

direct,

change-of-serve-cell,

ue-presence-in-area-of-interest,

stop-change-of-serve-cell,

stop-ue-presence-in-area-of-interest,

cancel-location-reporting-for-the-ue,

...,

change-of-serving-cell-and-UE-presence-in-the-Area-of-Interest,

aerial-ue-altitude-information-reporting,

stop-aerial-ue-altitude-information-reporting

}

*** Next Change ***

-- L

*** skip unmodified parts ***

LocationReportingRequestType ::= SEQUENCE {

eventType										EventType,

reportArea										ReportArea,

areaOfInterestList								AreaOfInterestList								OPTIONAL,

locationReportingReferenceIDToBeCancelled		LocationReportingReferenceID					OPTIONAL,

-- The above IE shall be present if the Event Type IE is set to the value “stop UE presence in the area of interest”

iE-Extensions		ProtocolExtensionContainer { {LocationReportingRequestType-ExtIEs} }		OPTIONAL,

...

}

LocationReportingRequestType-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LocationReportingAdditionalInfo	CRITICALITY ignore	EXTENSION LocationReportingAdditionalInfo		PRESENCE optional }|

{ ID id-AdditionalCancelledlocationReportingReferenceIDList	CRITICALITY reject	EXTENSION AdditionalCancelledlocationReportingReferenceIDList  PRESENCE optional },

{ ID id-AerialUEAltitudeInformationReportingConfiguration	CRITICALITY ignore	EXTENSION AerialUEAltitudeInformationReportingConfiguration  PRESENCE conditional

-- The above IE shall be present if the Event Type IE is set to the value “aerial UE altitude information reporting”},

...

}

*** Next Change ***

-- R

*** skip unmodified parts ***

ReportingPeriodicity ::= ENUMERATED {

120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 20480ms, 40960ms, 1min, 6mis, 12min, 30min,

...

}

*** Next Change ***

-- U

UserLocationInformation ::= CHOICE {

userLocationInformationEUTRA	UserLocationInformationEUTRA,

userLocationInformationNR		UserLocationInformationNR,

userLocationInformationN3IWF-with-PortNumber	UserLocationInformationN3IWF-with-PortNumber,

choice-Extensions		ProtocolIE-SingleContainer { {UserLocationInformation-ExtIEs} }

}

*** skip unmodified parts ***

UserLocationInformationNR ::= SEQUENCE {

nR-CGI				NR-CGI,

tAI					TAI,

timeStamp			TimeStamp															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationNR-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationNR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellInformation						CRITICALITY ignore	EXTENSION NGRAN-CGI											PRESENCE optional	}|

{ ID id-NID										CRITICALITY reject	EXTENSION NID												PRESENCE optional	}|

{ ID id-NRNTNTAIInformation						CRITICALITY ignore	EXTENSION NRNTNTAIInformation 							PRESENCE optional	}|

{ ID id-MobileIAB-MTUserLocationInformation		CRITICALITY ignore	EXTENSION MobileIAB-MTUserLocationInformation 		PRESENCE optional	}|

{ ID id-AerialUEAltitudeInformation				CRITICALITY ignore	EXTENSION AerialUEAltitudeInformation			 		PRESENCE optional	},

...

}

*** Next Change ***

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

*** skip unmodified parts ***

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-AllowedNSSAI											ProtocolIE-ID ::= 0

*** skip unmodified parts ***

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-AerialUEAltitudeInformationReportingConfiguration	ProtocolIE-ID ::= aaa

id-AerialUEAltitudeInformation							ProtocolIE-ID ::= bbb

END

-- ASN1STOP

*** End of Changes ***