3GPP TSG-RAN WG3 Meeting #128	R3-253515

St Julian’s, Malta, 19-23 May 2025

Agenda Item:		11.3

Source:		Ofinno

Title:	AI/ML enabled Coverage and Capacity Optimization

Document for:		Discussion and decision

# Introduction

After RAN3 #127-bis meeting, the following open topics are remaining in the agenda item 11.3 AI/ML enabled Coverage and Capacity Optimization:

For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state?

Option1: no need

Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

Whether/what additional UE performance measurement metrics is needed?

Timing information for predicted CCO issue is NOT needed to exchange over Xn? Whether the predicted CCO issue and /or future CCO state can be updated over Xn?

This contribution proposes how to address some of these open topics.

# Discussion

## 2.1	Future CCO state as assistance information

A gNB-CU has an AI/ML model for predicting a coverage or capacity issue in cells and beams managed by a gNB-DU. We think that the AI/ML model may be enhanced to also generate proposed cells and/or beams coverage states for cells and beams of the gNB-DU. This may not require a large amount of additional inference data (if any) at the input of the AI/ML model, because the AI/ML model is already capable of predicting coverage and capacity issues.

Such approach may considerably simplify why simultaneously improve gNB-DU CCO operation. This may also allow some level of coordination of coverage state changes of neighbour gNB-DUs managed by the gNB-CU (and potentially some level of inter-CU coordination).

We believe that it may be beneficial if the gNB-CU provides to the gNB-DU proposed cells and/or beams coverage states for cells and beams of the gNB-DU.

Also, in order to align future CCO states of cells and beams of neighbour gNB-DUs, it is beneficial for gNB-DU to receive form gNB-CU a future coverage states of neighbour gNB-DUs.

Option 2 and Option 3 form previous meeting discussion can work together and complement each other.

Proposal 1: Support Option 2 and Option 3:

Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

## 2.2	Additional assistance information (e.g., additional UE performance measurement metrics)

As was discussed during the last meeting, additional assistance information from a gNB-CU to a gNB-DU may be beneficial for selecting future CCO states by the gNB-CU.

Examples of additional assistance information are: UE location or geographical areas, UE radio measurements (cell level RSRP, RSRP, SINR), node level UE performance etc.

We think that the gNB-CU may provide to gNB-DU some additional assistance information to help with reconfiguration of cell and/or beam coverage areas. For example, this may help the gNB-DU to understand why coverage and/or capacity issue is predicted by an AI/ML model in the gNB-CU.

In addition to the above assistance information, other examples may include prediction of number of UEs/traffic/load per cell/beam; prediction of L3 performance metrics, e.g., number of RLFs, HOFs, SHOs per cell/beam (when applicable).

Proposal 2: A gNB-CU sends to a gNB-DU, additional assistance information for CCO, for example, predicted number of UEs/traffic/load, current or predicted UE location information (e.g., geographic areas), node/cell/beam level UE performance, predicted L3 performance metrics (e.g., RLF, HOF, SHO).

# Proposals

Proposal 1: Support Option 2 and Option 3:

Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

Proposal 2: A gNB-CU sends to a gNB-DU, additional assistance information for CCO, for example, predicted number of UEs/traffic/load, current or predicted UE location information (e.g., geographic areas), node/cell/beam level UE performance, predicted L3 performance metrics (e.g., RLF, HOF, SHO).

# Reference

[1] R3-253001, RAN3#128 Meeting Agenda

[2] R3-253002, RAN3#127-bis Meeting Report

[3] R3-252288, Summary of Discussion on CB: # AIRAN2_CCO