3GPP TSG-RAN WG3 Meeting #128	R3-253522

Malta, MT, 19th – 23th May 2025

Agenda Item:	20.1

Source:	ZTE Corporation

Title:	[Draft LS] Discussion on Relpy LSs from RAN2

Document for:	Discussions & Approval

- Introduction

In this meeting, RAN3 received two reply LSs from RAN2. The first [1] addresses dataset and parameter sharing, and the second [2] addresses concerns network-side data collection.

Regarding the signaling feasibility of dataset and parameter sharing, RAN2 asks RAN3 to confirm RAN2’s assumption on non-OTA solutions. For the OAM-centric solution related to network-side data collection, RAN2 requests RAN3 to provide any concerns regarding their agreements.

In this contribution, we present our understanding and analysis of these topics and provide the corresponding reply LSs to RAN2.

2	Discussion

2.1 Dataset and parameter sharing

In the LS [1], RAN2 concluded the following approaches for NW-side sharing dataset/model parameter transfer:

Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) Alternative 2 (OTA approach): gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)

For non-OTA solutions, RAN2 identified the following candidate solutions (see below Table 1). From RAN2 point of view, it is assumed they can be supported within Rel-19 existing architecture framework.

Table 1. non-OTA candidate solutions

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5, SA3 | Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2, SA3 | Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2) |
| gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5, SA3 | Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

Therefore, RAN3 is requested to provide its understanding on the feasibility of Option 3: "gNB → OAM/CN → UE-side training entity". In this option, RAN3 is involved in two aspects:

- The transfer of data from the gNB to the OAM;

- The transfer of data from the gNB to the CN.

For the first aspect, the transfer of data from the gNB to the OAM, from the RAN3 perspective, Southbound Interface between gNB and OAM is up to implementation.

Currently, RAN3 supports UE-side data collection through MDT for data gathering, as well as QoE mechanisms. For MDT mechanism, OAM can configure the MDT configuration based on the requirement to NG-RAN through management-based MDT or signaling-based MDT. For QoE data collection, OAM can also deliver QoE configurations to the RAN side and specify which QoE measurements are RAN-visible. For these RAN-visible parameters, NG-RAN can be involved by initiating the corresponding RAN-visible configurations to the UE for use.

Furthermore, the specific data required for each case needs to be defined by RAN1 and RAN2 first. Based on that input, RAN3 can then do further analysis on data transfer case by case.

- RAN3 already supports MDT data collection as well as QoE data collection including RAN visible QoE measurements for UE data collection, which is reported from gNB to TCE/QMC.

- From RAN3 perspective, the southbound interface between gNB and OAM is up to implementation. The specific data required for each case needs to be defined by RAN1 and RAN2 first. Based on that input, RAN3 can then do further analysis on data transfer case by case.

For the second aspect, the transfer of data from the gNB to the CN, this topic has already been discussed under the AI/ML for NG-RAN work. Some supporting companies proposed to study, during the SI phase, whether AI/ML-related information could be exchanged between the gNB and the CN to support AI/ML-assisted network slicing. However, this proposal was not approved for Rel-19 study item, as it was considered to increase architectural complexity. Therefore, similarly, the exchange of AI/ML-related data between the CN and NG-RAN is not supported by RAN3 in Rel-19. Moreover, RAN3 does not have visibility into what specific data would need to be exchanged between the CN and the RAN side, and therefore is not in a position to further discuss this matter.

- From RAN3 perspective, the transfer of AI/ML related data (e.g. dataset and model parameters) from gNB to CN can not be supported within Rel-19 existing framework in Rel-19.

Based on the analysis above, we suggest providing the answer to RAN2 states that RAN3 cannot directly respond to whether the transfer of data from the gNB to OAM/CN is feasible or not without knowing the specific type of the data. RAN3 can provide the introduction of current mechanism supporting the data report from gNB to OAM, and provide the statement that the transfer of AI/ML related data (e.g. dataset and model parameters) from gNB to CN can not be supported within Rel-19 existing framework in Rel-19.

- Agree the reply LS in the Annex-A reflecting the Proposal 1 & Proposal 2.

2.2 Network-side data collection

Following is the content of LS on OAM-centric solution for NW-side data collection:

For OAM-centric solution for NW-side data collection, RAN2 has made the following agreements: RAN2 assumes to only adopt management based immediate MDT for NW-side data collection. For NR-DC, the configuration and reporting of NW sided data collection can only be via MN.

RAN3 should respect the first RAN2 assumption that only management-based immediate MDT is adopted for network-side data collection. However, regarding the second agreement — “For NR-DC, the configuration and reporting of NW-side data collection can only be via MN” — this appears to conflict with the existing MDT principles for NR-DC.

According to TS 37.320, for immediate MDT configuration, both the MN and SN can independently configure MDT and receive measurements from the UE.

5.4.1.3 Immediate MDT for MR-DC Immediate MDT is supported for (NG)EN-DC, NE-DC and NR-DC scenario. Signalling based immediate MDT is triggered for both MN and SN as default, if the MN only indication is provided by OAM, the MDT configuration applies to MN only. In signalling based immediate MDT, MME provides MDT configuration for both MN and SN towards MN including multi RAT SN configuration, specifically E-UTRA and NR MDT configuration. MN then forwards the NR MDT configuration towards SN (EN-DC scenario, SN is always NR). In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently. For both MN and SN, Management based MDT should not overwrite signalling based MDT. For immediate MDT configuration, MN and SN can independently configure and receive measurement from the UE. For MN terminated SCG bearer and SN terminated MCG bearer, the terminated node, e.g., MN in case of MN terminated SCG bearer, configures the configuration to UE. For configuring UL PDCP packet average delay (as specified in clause 4.3.1.1 in TS 38.314 [18]) in case of split bearer: only the terminated node of the split bearer can configure the measurement to UE, and the UE reports the measurement result to corresponding node where the configuration was received from.

- In the current specification, for immediate MDT configuration, both the MN and SN can independently configure MDT and receive measurements from the UE. RAN2’s agreement appears to conflict with the existing MDT principles for NR-DC.

Therefore, RAN3 believes that this agreement appears to be questionable. RAN3 would appreciate further information and clarification to better understand the rationale behind this agreement before introducing any enhancements on current MDT mechanism for Rel-19.

- Agree to send the reply LS in Annex B to seek further information and clarification regarding RAN2’s agreement that “For NR-DC, the configuration and reporting of network-side data collection can only be via the MN.”

3	Conclusion

Following is the proposals and observations:

Dataset and parameter sharing:

- RAN3 already supports MDT data collection as well as QoE data collection including RAN visible QoE measurements for UE data collection, which is reported from gNB to OAM.

- From RAN3 perspective, the southbound interface between gNB and OAM is up to implementation. The specific data required for each case needs to be defined by RAN1 and RAN2 first. Based on that input, RAN3 can then do further analysis on data transfer case by case.

- From RAN3 perspective, the transfer of AI/ML related data (e.g. dataset and model parameters) from gNB to CN can not be supported within Rel-19 existing framework in Rel-19.

- Agree the reply LS in the Annex-A reflecting the Proposal 1 & Proposal 2.

Network-side data collection:

- In the current specification, for immediate MDT configuration, both the MN and SN can independently configure MDT and receive measurements from the UE. RAN2’s agreement appears to conflict with the existing MDT principles for NR-DC.

- Agree to send the reply LS in Annex B to seek further information and clarification regarding RAN2’s agreement that “For NR-DC, the configuration and reporting of network-side data collection can only be via the MN.”

4	Annex A - Reply LS on dataset and paremeter sharing

3GPP TSG-RAN WG3 Meeting #128	R3-25xxxx

Malta, MT, 19th – 23th May 2025

itle:	[Draft] Reply LS on signalling feasibility of dataset and parameter sharing

Response to:	R2-2503169

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	ZTE Corporation[to be RAN3]

To:	RAN2

Cc:	RAN1, SA2, SA5, SA3

Contact person:	Chen Jiajun

Chen.jiajun1@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN3 thanks RAN2 for the LS related to dataset and parameter sharing, and for seeking the confirmation from RAN3 regarding non-OTA solutions.

Table 1. non-OTA candidate solutions

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5, SA3 | Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2, SA3 | Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2) |
| gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5, SA3 | Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

In the option 3 (gNB -> OAM/CN -> UE-side training entity), RAN3 is involved in two aspects:

- The transfer of data from the gNB to the OAM;

- The transfer of data from the gNB to the CN.

For the first aspect, the transfer of data from the gNB to the OAM, from the RAN3 perspective, the southbound interface between gNB and OAM is up to implementation. The specific data required for each case needs to be defined by RAN1 and RAN2 first. Based on that input, RAN3 can then do further analysis on data transfer case by case, e.g., whether current MDT or QoE mechanism can be reused or not.

For the second aspect, the transfer of data from the gNB to the CN, the exchange of AI/ML-related data between the CN and NG-RAN is not supported by RAN3 in Rel-19 based on the conclusion in “AI/ML for NG-RAN” topic.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take information above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#129				2025-08-25 – 2025-08-29				India, IN

RAN3#129bis			2025-10-13 – 2025-10-17				Prague, CZ

5	Annex B - Reply LS on network-side data collection

3GPP TSG-RAN WG3 Meeting #128	R3-25xxxx

Malta, MT, 19th – 23th May 2025

itle:	[Draft] Reply LS on OAM-centric solution for NW-side data collection

Response to:	R2-2503170

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	ZTE Corporation[to be RAN3]

To:	RAN2

Cc:	SA5

Contact person:	Chen Jiajun

Chen.jiajun1@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN3 thanks RAN2 for the LS related to OAM-centric solution for NW-side data collection.

From RAN3 perspective, for immediate MDT configuration, both the MN and SN can independently configure MDT and receive measurements from the UE, which appears to conflict with the existing MDT principles for NR-DC in TS37.320.

RAN3 would appreciate further information and clarification to better understand the rationale behind this agreement before introducing any enhancements on current MDT mechanism in Rel-19..

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to provide further information and clarification.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#129				2025-08-25 – 2025-08-29				India, IN

RAN3#129bis			2025-10-13 – 2025-10-17				Prague, CZ