3GPP TSG-RAN WG3 Meeting # 128	 R3-253526

# St Julian's, Malta,  19th - 23th May, 2025

Agenda Item:	13.2

Source:	Google

Title:	(TP to BL CRs 38.300 and 37.340) Clarification on RS configuration and CSI Resource Configuration

Document for:	Discussions & Approval

# 1	Discussions

In the previous meetings, RAN2 and RAN3 had the following agreements related to the CSI resource and report configuration:

RAN2#127 CSI resource and report configuration: 2. The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration. 3. The source CU is responsible to generate the common CSI resource configuration. RAN2#127bis Agreements on inter-CU LTM 5. For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN. 6. The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. 7. The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. 8. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN. Agreements on L1 event triggered MR 7. For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3. 8. For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed. 13. Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) RAN2#128 Agreements on L1 event-driven MR: 4. Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation. 5. Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1. RAN2#129 Agreements on L1 event-triggered MR: 1. Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused". 2. Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide). 3. Do not create a new resource configuration just for the event triggered measurements. 4. Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

RAN3 agreements in Agenda 13.2 Support for Inter-CU LTM WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message. RAN3#127bis Include the following information in SN Change Required (Source SN to MN) message: - LTM request indication (explicit way) - Suggested PScell list (implicit way in INM, FFS on explicit way) - Measurement results (implicit way in INM) - Upper limit number of PScell(s) for each candidate SN (FFS) - CSI Resource configuration request (explicit way) - Early sync information request (explicit way) Include the following information in SN Addition Request (MN to Candidate SN) message: - LTM request indication (explicit way) - Suggested PScell list (implicit way in INM, FFS on explicit way) - Measurement results (implicit way in INM) - Upper limit number of PScell(s) for the candidate SN (explicit way) - CSI Resource configuration request (explicit way) - Early sync information request (explicit way)

Current RAN2 running CRs and RAN3 BLCRs (TS38.300, 37.340, 38.423, and 38.331) contain some ambiguity regarding the configuration of Layer 1 (L1) reference signals (RS) and CSI Resource Configuration, particularly the relationship between CSI-RS configuration and CSI Resource Configuration. For clarity, this discussion assumes "CSI Resource Configuration" to be LTM-CSI-ResourceConfig-r18, as defined in Release 18 and updated in Release 19 with LTM-CSI-RSResource-Set-r19. This CSI Resource Configuration can carry both SSB and CSI-RS as shown below. In the XnAP BLCR for the DC scenario, there is a FFS IE “Layer 1 Configuration”carried in the SN Addition Request Acknowledge message and SN Modification Request message. This discussion assumes this Layer 1 Configuration IE as the L1 RS Configuration.

#### R3-252520 (BL CR to 38.423) Xn support for inter-CU LTM If the Request for CSI Resource Configuration IE is contained in the LTM Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI Resource Configuration IE in the LTM Information Response IE in the HANDOVER REQUEST ACKNOWLEDGE message. #### 9.2.1.xx8 CSI Resource Configuration This IE contains the CSI resource configuration used for LTM. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CSI Resource Configuration To AddModList; O; OCTET STRING; Contains the ltm-CSI-ResourceConfigToAddModList as defined in TS 38.331 [10].. CSI Resource Configuration To Release List; O; OCTET STRING; Includes the ltm-CSI-ResourceConfigToReleaseList contained in the LTM-Config IE as defined in TS 38.331 [10].. SSB Information; O; 9.2.1.x10.

R2-2503785 RRC Running CR for “Introduction of event-triggered measurement reporting for RRC spec” #### – LTM-Candidate The IE LTM-Candidate concerns a LTM candidate configuration to add or modify. LTM-Candidate information element -- ASN1START -- TAG-LTM-CANDIDATE-START LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidatePCI-r18 PhysCellId OPTIONAL, -- Need M ltm-SSB-Config-r18 LTM-SSB-Config-r18 OPTIONAL, -- Need M ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, -- Need M ltm-ConfigComplete-r18 ENUMERATED {true} OPTIONAL, -- Need R ltm-EarlyUL-SyncConfig-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-EarlyUL-SyncConfigSUL-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-TCI-Info-r18 LTM-TCI-Info-r18 OPTIONAL, -- Need M ltm-NoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ltm-UE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ..., [[ ltm-NZP-CSI-RS-ResourceToAddModList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource OPTIONAL, -- Need N ltm-NZP-CSI-RS-ResourceToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet OPTIONAL, -- Need N ltm-NZP-CSI-RS-ResourceSetToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId OPTIONAL -- Need N ]] } LTM-SSB-Config-r18 ::= SEQUENCE { ssb-Frequency-r18 ARFCN-ValueNR, subcarrierSpacing-r18 SubcarrierSpacing, ssb-Periodicity-r18 ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL, -- Need R ssb-PositionsInBurst-r18 CHOICE { shortBitmap BIT STRING (SIZE (4)), mediumBitmap BIT STRING (SIZE (8)), longBitmap BIT STRING (SIZE (64)) } OPTIONAL, -- Need R ss-PBCH-BlockPower-r18 INTEGER (-60..50) OPTIONAL, -- Need R ... } -- TAG-LTM-CANDIDATE-STOP -- ASN1STOP LTM-Candidate field descriptions. ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration.. ltm-CandidatePCI This field identifies the PCI of the SpCell of the LTM candidate configuration contained in ltm-CandidateConfig.. ltm-NZP-CSI-RS-ResourceToAddModList Pool of CSI-RS resources sets which can be referred to in NZP-CSI-RS-ResourceSet.. ltm-NZP-CSI-RS-ResourceSetToAddModList Pool of CSI-RS resource set which can be referred to in LTM-CSI-RS-ResourceConfig.. ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.. ltm-NoResetID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID.. ltm-UE-MeasuredTA-ID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID. This field is absent if tag2 is present for this LTM candidate configuration.. #### – LTM-CSI-ResourceConfig The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations LTM-CSI-ResourceConfig information element -- ASN1START -- TAG-LTM-CSI-RESOURCECONFIG-START LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { ltm-CSI-ResourceConfigId-r18 LTM-CSI-ResourceConfigId-r18, ltm-SSB-ResourceSet-r18 LTM-SSB-ResourceSet-r18, ... , [[ ltm-NZP-CSI-RS-ResourceSet-r19 LTM-NZP-CSI-RS-ResourceSet-r19 OPTIONAL, -- Need R resourceType-r19 ENUMERATED {periodic, semi-persistent} OPTIONAL -- Cond CSI-RS ]] } LTM-SSB-ResourceSet-r18 ::= SEQUENCE { ltm-SSB-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index, ltm-CandidateIdList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18, ... } LTM-NZP-CSI-RS-ResourceSet-r19 ::= SEQUENCE { ltm-CSI-RS-ResourceList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF NZP-CSI-RS-ResourceId, ltm-CandidateIdList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF LTM-CandidateId-r18, ... } -- TAG-LTM-CSI-RESOURCECONFIG-STOP -- ASN1STOP LTM-CSI -ResourceConfig field descriptions. ltm-SSB-ResourceSet This field indicates the resource set for LTM measurement based on SSB.. ltm-CSI-RS-ResourceSet This field indicates the resource set for LTM measuremenet based on CSI-RS. When the field is present, the UE shall ignore the field ltm-SSB-ResourceSet.. resourceType Time domain behavior of LTM CSI resource (see TS 38.214 [19], clause 5.2.1.2). .. LTM-SSB-ResourceSet field descriptions. ltm-CandidateIdList his field indicates the LTM candidate configuration IDs related to the SSBs in the ltm-SSB-ResourceList. The list has the same number of entries as ltm-SSB-ResourceList. The first entry in this list shall be associated to the first entry in ltm-SSB-ResourceList, the second entry of this list shall be associated to the second entry in ltm-SSB-ResourceList, and so on.. ltm-SSB-ResourceList This field is used to indicate on SS/PBCH block resources from one or more LTM candidate cells.. LTM-NZP-CSI-RS-ResourceSet field descriptions. ltm-CandidateIdList This field indicates the LTM candidate configuration IDs related to the NZP-CSI-RSs in the ltm-CSI-RS-ResourceList. The list has the same number of entries as ltm-CSI-RS-ResourceList. The first entry in this list shall be associated to the first entry in ltm-CSI-RS-ResourceList, the second entry of this list shall be associated to the second entry in ltm-CSI-RS-ResourceList, and so on.. ltm-NZP-CSI-RS-ResourceList This field is used to indicate on NZP CSI-RS resources from one or more LTM candidate cells. If the resource type of the NZP-CSI-RS resource is semi-persistent, the ltm-ReportConfigType within LTM-CSI-ReportConfig that this resource set is associated with cannot be configured as eventTriggered.. Conditional Presence; Explanation. CSI-RS; This field is mandatory present, Need R, if the field ltm-NZP-CSI-RS-ResourceSet-r19 is configured. Otherwise, it is absent..

The central issue is determining whether the source base station (BS) in Inter-CU MCG LTM or the source secondary node (SN) in Inter-CU SCG LTM can request the LTM-CSI-ResourceConfig from the candidate BS or SN, as indicated by the CSI Resource Configuration IE (9.2.1.xx8) in the XnAP BLCR. Release 18 specifies that the gNB-CU generates the LTM CSI resource configuration and sends it to the gNB-DU to acquire the LTM CSI report configuration. Therefore, it seems technically feasible for the candidate BS or SN to directly provide the LTM CSI resource configuration.

However, this raises a concern: the LTM candidate configuration ID, used by the BS and the UE to identify an LTM candidate configuration and associate it with the candidate cell ID. It is questionable whether the candidate BS or SN should assign these IDs for the cells they serve, given the need for coordination with the source BS or SN to avoid conflicts, which could be exacerbated by parallel preparation at multiple candidate BSs or SNs.

Observation: For the candidate BS or SN to provide the LTM-CSI-ResourceConfig, they must be informed or be able to assign LTM candidate configuration IDs.

One potential solution is for the source BS or SN to provide the Configuration ID Mapping List from the beginning of LTM preparation. However, this may not be optimal for all scenarios, such as when the candidate BS or SN needs to perform admission control before proceeding with LTM preparation.

An alternative, and potentially cleaner, solution is for the candidate BS or SN to provide the L1 RS configuration (e.g., a list of SSB or CSI-RS), as agreed upon in RAN2#127bis and currently implemented in the TS 37.340 BLCR. This approach is independent of the availability of the Configuration ID Mapping List at the candidate BS or SN.

Proposal 1: RAN3 to clarify that the candidate BS or SN provides the L1 RS configuration (e.g. a list of SSB or a list of CSI-RS) instead of directly provide the LTM-CSI-ResourceConfig.

If this proposal is accepted, aligning the terminology between TS 38.300 (i.e., CSI-RS configuration) and TS 37.340 (i.e., L1 RS configuration) would enhance clarity, given that CSI-RS resources can be considered a type of L1 RS configuration, in addition to the SSB Positions In Burst.

Regarding the detailed information required for the L1 RS configuration in the XnAP specification, a choice structure between SSB and CSI-RS is proposed. For example, the CSI-RS part could include:

- ltm-NZP-CSI-RS-ResourceToAddModList (Pool of CSI-RS resource sets which can be referred to in NZP-CSI-RS-ResourceSet), and/or

- ltm-NZP-CSI-RS-ResourceSetToAddModList (Pool of CSI-RS resource set which can be referred to in LTM-CSI-ResourceConfig)

- ...

This structure, to be confirmed by the group, serves as a baseline.

      - 9.2.1.xxY	L1 RS Configuration (FFS)

This IE contains the L1 RS configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE L1 RS Configuration | M |  |  |  |
| >SSB |  |  |  |  |
| >>SSB Information | M |  | 9.2.1.x10 |  |
| >CSI-RS |  |  |  |  |
| >>CSI-RS Resource To AddMod List | M |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceToAddModList IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| >>CSI-RS Resource To Release List | M |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceToReleaseList IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| >>CSI-RS Resource Set To AddMod List | O |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceSetToAddModList IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| >>CSI-RS Resource Set To Release List | O |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceSetToReleaseList IE as defined in subclause 6.3.2 of TS 38.331 [10]. |

It is also worth noting that in the Xn Setup procedure, the NG-RAN nodes can exchange the Measurement Timing Configuration IE, which includes the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331. The CSI-RS-Config-r16 within the MeasurementTimingConfiguration contains some CSI-RS related parameters and time/frequency configurations. It may be enough for the source BS or the source SN to obtain the necessary information to obtain or generate the common CSI Resource Configuration; however, similar to the Rel-18, the source BS or the source SN still needs to configure the ltm-NZP-CSI-RS-ResourceToAddModList and the ltm-NZP-CSI-RS-ResourceSetToAddModList, etc., in the ltm-Candidate to the UE. Therefore, similar to the SSB Information, the exchange of the L1 RS Configuration is still required for the LTM preparation.

Proposal 2: RAN3 to confirm the required content of the CSI-RS configuration within the L1 RS configuration from the candidate BS or candidate SN.

# 2. Conclusion

In this contribution, we discussed issues of the L1 reference signal (RS) configuration, LTM CSI Resource Configuration, and configuration ID mapping list for inter-CU LTM. The related observations and proposals are summarized below:

Observation: For the candidate BS or SN to provide the LTM-CSI-ResourceConfig, they must be informed or be able to assign LTM candidate configuration IDs.

Proposal 1: RAN3 to clarify that the candidate BS or SN provides the L1 RS configuration (e.g. a list of SSB or a list of CSI-RS) instead of directly provide the LTM-CSI-ResourceConfig.

Proposal 2: RAN3 to confirm the required content of the CSI-RS configuration within the L1 RS configuration from the candidate BS or candidate SN.

# 3. Text Proposal to Inter-CU LTM BL CR to TS 38.300

<< First Change >>

##### 9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and/or the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement.

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends a HANDOVER REQUEST ACKNOWLEDGE message to the source gNB including the generated RRC configuration for the accepted candidate cell. The candidate gNB(s) may also include the L1 RS configuration upon request.

6.	The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI resource configuration and the LTM configuration ID mapping list.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI report configuration.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

NOTE :	Step 6 may also be triggered after step 14, or after step 17 by implementation for subsequent LTM.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early synchronization to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB if early TA acquisition is performed to the candidate gNB(s).

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

NOTE :	Up to implementation, data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 14.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNB(s). This message may include the new security key(s) to be used with the UE.

21. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

<< End of Changes >>

# 3. Text Proposal to Inter-CU LTM BL CR to TS 37.340

## 10.5	Secondary Node Change (MN/SN initiated)

<< Skip Unchanged Parts >>

### 10.5.2	MR-DC with 5GC

<< Skip Unchanged Parts >>

SN initiated inter-SN SCG LTM

The SN initiated inter-SN SCG LTM procedure is used to configure inter-SN SCG LTM.

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

Figure 10.5.2-5 shows an example signalling flow for the inter-SN SCG LTM initiated by the SN:

Editor’s Note: RAN2 assumes that the figure should be anchored together with RAN3. The final figure is up to the progresses made in RAN3.

1.	The source SN initiates the inter-SN SCG LTM procedure by sending the SN Change Required with a LTM Request Indication to the MN providing a list of candidate PSCell(s) for inter-SN SCG LTM preparation. The message may include an SCG reference configuration. Source SN may additionally send measurement results of candidate PSCells to the MN. Source SN may send L1 RS configuration request and/or early sync information request for suggested candidate PSCell(s).

Editor’s Note: RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3.

2.	The MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN also provides a list of KSN and associated sk-Counter values (FFS) for each candidate SN, and forwards the received measurement results to each candidate SN(s). The MN may transfer L1 RS configuration request and/or early sync information for suggested candidate PSCell(s) to SN. The MN also provides upper limit for the number of PSCells to each candidate SN. The MN may select one of the candidate SN(s) and request providing the reference SCG configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s).

3.	The candidate SN(s) determines the LTM candidate cells based on the measurement results (if provided) and the upper limit for the number of PSCells that can be prepared by each candidate SN. The candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL/DL sync configuration, by sending SN Addition Request Acknowledge message to the MN.

4.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

5.	The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message.

6.	The source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message. The source SN sends the configuration ID mapping list for the accepted candidate PSCell(s) to the MN for subsequent inter-SN SCG LTM.

7.	The MN transfers, during the LTM preparation phase, configuration ID mapping list, the common CSI resource configuration and the collected information of candidate cells to each candidate SN via the SN Modification Request message.

8.	The candidate SN(s) responds with the updated candidate SCG configuration to the MN via the SN Modification Request Acknowledge message. The candidate gNB(s) may also provide the CSI report configuration.

9.	The MN generates the reference configuration for SN initiated inter-CU SCG LTM, which may include the MCG part of the reference configuration generated by the MN (if any), together with the SCG part of the reference configuration generated by the source or candidate SN. The MN sends an RRCReconfiguration message to the UE including the inter-SN SCG LTM configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required measurements.

10.	The UE applies the RRCReconfiguration message received in step 9, stores the inter-SN SCG LTM configuration and replies to the MN with an RRCReconfigurationComplete message, including an SN RRCReconfigurationComplete*** message to the source SN.

11.	The MN informs the source SN with the SN RRCReconfigurationComplete*** message via SN Change Confirm message, to indicate that SCG LTM is prepared.

Editor’s Note: FFS up to RAN3.

13.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14a.The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14b. The candidate SN(s) transfers the calculated TA value of candidate cell(s) to the source SN via the MN via the TA Information Transfer Message.

Editor’s Note: Details for TA information transfer procedure is up to RAN3 discussion.

15.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source SN, if the L1 measurement configuration in RRCReconfiguration is received in step 9.

16.	The source SN decides to execute cell switch to a target PSCell.

17.	The source SN transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target PSCell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE applies the candidate configuration indicated by the target configuration ID including PSCell change.

18/19.	The source SN sends Cell Switch Notification Message by Cell Switch Notification procedure, to indicate the triggering of LTM to the UE , including the target PScell ID and the TCI state ID(s).

20.	The UE performs the random access procedure towards the target PSCell, if the UE does not have valid TA of the target PSCell.

21. The UE sends an MN RRCReconfigurationComplete* message to the MN, which includes an SN RRCReconfigurationComplete** message to the target SN.

22.	The MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message.

23~29. Editor’s Note: Details for related procedures and descriptions are up to RAN3 discussion.

NOTE X:	The steps 12-29 can be performed multiple times for subsequent SCG LTM cell switch execution using the SCG LTM candidate configuration(s) provided in step 9.

Editor’s Note: In the above descriptions and signalling flow for the overall procedure of SN-initiated inter-SN SCG LTM, the detail procedures and messages between MN, source or candidate SN(s) are up to RAN3 discussion.