3GPP TSG-RAN WG3 Meeting #128	R3-253532

Malta, 19 – 23 May, 2025

Agenda item:	17.2 and 17.4

Source:	Nokia

Title:	Open points on OD-SSB. Adaptation of common signals/channels

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses:

- open points on OD-SSB (AI 17.2)

- Adaptation of common signals/channels (AI 17.4)

# 2	Open points on OD-SSB

It was discussed further at RAN3#127bis whether and how the gNB-CU should provide some assistance information to the gNB-DU for activation/deactivation of the OD-SSB functionality (summary of discussion in [1]). E.g. it was discussed reuse of the existing Cells Allowed to be Deactivated List IE to indicate the allowed cells for on-demand SSB operation to the DU, without agreement.

We here remind background for our view that such assistance information is not needed.

RAN1 has agreed two cases for OD-SSB operation:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

Here are some examples of OD-SSB operation:

Figure 1: Examples of OD-SSB operation case #2 (always-on SSB is periodically transmitted on the cell) and case #1 (only on-demand SSB is transmitted on the cell)

For case #2, the cell will keep always-on SSB operation in addition to the OD-SSB operation. Case #2 operation therefore doesn’t alter the capability of the cell to operate as a suitable or acceptable cell for idle-mode UEs, or to serve UEs as PCell or PSCell.

Observation 1:  OD-SSB case #2 operation doesn’t prevent the cell to operate as a suitable or acceptable cell for idle-mode UEs, or to serve UEs as PCell or PSCell.

It therefore seems clear that there is no need for the gNB-CU to be involved in activation and deactivation of OD-SSB operation in case #2.

Proposal 1: For OD-SSB operation case #2 (always-on SSB is periodically transmitted on the cell), it is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”. gNB-CU assistance is not needed.

Also for OD-SSB operation case #1 (no always-on SSB on the cell) we believe that no such assistance information is required over F1 for the following reasons:

- The concept of SSB-less SCell operation for intra-band CA was introduced in Rel-15. It was enhanced to inter-band CA also in Rel-18. In these legacy functionalities, no such assistance information over F1 was considered require – we don’t see why it should be required for the Rel-19 functionality under discussion.

- RAN1 and RAN2 have been discussing the various aspects of the functionality – the only agreement which could involve gNB-CU is the one where RRC signalling could be used to activate the functionality (if it is done along with the configuration for the first time). There are no other aspects that may have any gNB-CU relevance. Even this agreement doesn’t require any authorization or assistance information from the gNB-CU but rather reuse existing RRC/F1 signalling.

We further observe that OD-SSB operation case #1 can’t happen while there are UEs camping on the cell. This means that the cell has to be barred. Similary, the transition can’t happen while the cell is serving UEs as PCell or PSCell. However, such transition can happen while the cell is serving UEs as SCell, because the OD-SSB pattern can be provided both at the time of SCell addition and at any time during Scell operation.

We therefore conclude that the gNB-DU has sufficient information to determine whether it can switch from always-on SSB operation to OD-SSB operation case #1 because:

- The DU owns the MIB, so it will know whether the cell is barred (or e.g. DU-OAM could take the decision to bar the cell, but there is no CU involvement)

- The DU knows whether the cell serves UEs as PCell / PSCell

Proposal 2: For OD-SSB operation case #1 (no always-on SSB on the cell), the gNB-DU has sufficient information to determine whether it can switch from regular SSB transmission to OD-SSB SCell operation. Additional gNB-CU assistance is therefore not needed.

If, nevertheless, RAN3 concludes there is benefit to provide assistance information from the gNB-CU to the gNB-DU, we believe it is important that this is done in a way that doesn’t impact legacy (e.g. Rel-18) functionality. One of the options discussed at previous meeting was to avoid ASN.1 impact by reusing the F1AP Cells Allowed to be Deactivated List IE within the GNB-CU CONFIGURATION UPDATE message:

| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

The corresponding procedural text is:

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

It is understood that deactivation of all SSB beams within a cell corresponds to deactivating the cell itself. In deployments where this feature (IE) is implemented, the gNB-CU sending this IE can expect that the gNB-DU receiving such information will take the information into account in decisions to deactivate the cell. And although the gNB-DU has no mandate to keep other cells operational, a gNB-CU implementation can also expect that cells that are not included in the list would remain operational as per regular operation conditions.

However, if any assistance information from gNB-CU to gNB-DU is introduced for OD-SSB operation, we believe this information should be used to enable that the gNB-CU gets visibility on cells that the gNB-DU considers for OD-SSB case #1 operation, i.e. that the gNB-CU can rely on that these cells remain available to serve UEs as SCells. However, the legacy signalling can’t serve this purpose because the gNB-DU may also consider that the signalled cells may be switched off – in which case they would become unavailable for SCell operation.

Observation 2: If the legacy F1AP Cells Allowed to be Deactivated List IE is reused to as assistance information for activation of OD-SSB operation, the gNB-DU may consider either that the signalled cells may enter OD-SSB case #1 operation or may be switched off – and in the latter case these cells will become unavailable for SCell operation.

We therefore think that assistance information from gNB-CU to gNB-DU relative to OD-SSB operation should, if such information is needed, be introduced using a new, dedicated IE.

Proposal 3: If assistance information from gNB-CU to gNB-DU relative to OD-SSB operation is needed, introduce a new, dedicated IE for this purpose.

# 3	Adaptation of common signals/channels

## 3.1	Adaptation of SSB in time domain

The Rel-19 outcome from RAN1 relative to adaptation mechanism(s) of SSB in time-domain is as follows:

•	For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

•	There is no consensus in RAN1 to support adaptation for SSB that is not CD-SSB (A2)

•	There is no consensus in RAN1 to support Adaptation for SSB not on sync raster (A3)

•	For Rel-19 NES-capable UE’s SCell, adaptation of SSB configured for the SCell is supported for the following cases

•	Adaptation for SSB that is not CD-SSB on sync raster (B2’)

•	Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Also, for SCell, adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec has been discussed in RAN1 but is not supported (no consensus).

RAN2 has agreed that neighbour cell measurements for SCell are based on legacy SMTC.

Based on this, we believe there is no RAN3 impact on adaptation of SSB in time domain in Rel-19.

Proposal 4: RAN3 identifies no impact on RAN3 specification coming from adaptation of SSB in time domain in Rel-19.

## 3.2	Adaptation of PRACH in time domain

In previous RAN1 meetings, a list of candidate mechanisms for PRACH adaptation in time domain was discussed. The agreed mechanism is based on additional PRACH resources configured and activated for NES-capable UEs only when needed, hence allowing the network to enter longer sleeping periods in low load periods.

Agreement from RAN1#116 (Feb. 2024):

For adaptation of PRACH in time-domain, support at least the following:

-	Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

Observation 3:

For PRACH adaptation, two types of PRACH resources will be defined in a cell:

•	Legacy (Rel-15) PRACH resources (minimum set of PRACH resources) – always activated

•	Additional PRACH resources (new in Rel-19 for NES-capable UEs) – activated dynamically

In subsequent RAN1 meetings, the mechanism (with the associated details) to support the additional PRACH resources for NES-capable UEs has been discussed. We assume that from configuration perspective the OAM will configure the new additional PRACH resources in the gNB-DU.

Proposal 5: OAM configures the new additional PRACH resources to the gNB-DU

We also observe that RAN1 has designed the additional PRACH resources to be activated or deactivated in a dynamic manner. RAN3 needs to determine which entity that decides this activation and deactivation. From our side we believe this can be done autonomously by the gNB-DU.

Proposal 6: Activation and deactivation of additional PRACH resources to be decided autonomously by the gNB-DU.

RACH optimization was introduced in Rel-16, based on Rel-15 PRACH resource definition. RAN3 is tasked by the Rel-19 NES WID [1] to enable NG-RAN support of the new Rel-19 PRACH resources. Support of RACH optimization is part of the NG-RAN functionality, and it therefore seems that RAN3 within the present work item should evaluate whether there is impact on the RACH optimization feature and determine enhancements needed, if any.

At the present stage, we observe that in the legacy case the gNB-CU has the full view of the PRACH resources configured in the gNB-DU, and these resources are considered to be statically allocated and activated. Information about these resources may today also be transferred over F1 to gNB-DUs serving neighbour cells, and transferred over Xn to neighbour gNBs. But, due to dynamic activation and deactivation of the new additional PRACH resources defined in Rel-19, the network entities in charge of RACH optimization (gNB-CU, gNB-DU) may, based on the existing information exchange, have incomplete information available about the PRACH resources actually used in different cells.

We therefore propose that RAN3 further works on the analysis of impact of Rel-19 additional PRACH resources on the legacy RACH optimization feature.

Proposal 7: RAN3 to further work on the analysis of impact on Rel-19 additional PRACH resources on the legacy RACH optimization feature

## 3.3	Adaptation of paging occasions

RAN1 and RAN2 have made the following key agreements on adaptation of paging occasions:

- The solution option b) – where the paging frame interval is extended – has been agreed as the baseline solution to be worked upon

- RAN2 to aim for signaling overhead minimization

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration

- Legacy UE is not barred

The agreed option b) can be illustrated as in Figure 1:

Figure 1: Illustration of adaptation of paging occasions based on extension of the paging frame interval as per RAN1’s agreement.

As can be observed, this solution targets to increase energy savings by extending the paging time interval, thereby allowing for paging resource configuration over a longer period of time. While this is desirable from the energy savings perspective, it also has the effect to reduce the resources allocated for paging. In a legacy network, paging resources will be dimensioned in order to reasonably support periods of high load. If for some reason the paging load becomes particularly high in a given cell or beam for a given Paging Occasion (PO), some of the paging messages will not be transmitted in that PO and in case the UE is located in that cell or beam the network will have to escalate the paging. However, although paging escalation is based on implementation dependent strategies, at one point the mechanism requires extension of the paging to wider areas (additional cells/nodes) so that the full registration area of the UE can eventually be paged. Paging escalation is therefore resource consuming and should be avoided as far as possible, and this aspect is therefore given much attention in deployed networks.

However, the need for network energy saving is equally important, and the reduction of POs based on  extension of the paging frame interval as per RAN1’s agreement allows to increase sleep opportunities for the transmitter (due to reduced number of POs), as well as for the receiver due to reduced number of PRACH resources as illustrated in Figure 1. It can be noted that as per RAN2’s agreement “Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration”, and an energy saving action based on paging adaptation will therefore unavoidably create similar risk for paging overload (and hence skipped paging messages) in periods of low paging load as the risk currently existing in high-load periods in legacy networks.

RAN3 will therefore first need to determine the entity to be in charge of the decision of activation and deactivation of adaptation of the POs for the purpose of NES (paging energy saving). In non-split architecture this could be the OAM or the gNB. We here assume that the gNB will need to be able to at least take decision of deactivation of paging energy saving in order to cope with situation of paging overload as fast as possible. And in order to optimize opportunities for micro-sleep modes, the gNB then also needs to take the decision of activation of paging energy saving. Cell level granularity seems to us to be the good granularity for this decision, hence enabling same paging capacity in all beams within the cell which is needed e.g. for escalated paging. We here derive the following principles for paging energy saving:

Proposal 8: Paging energy saving is done on a per cell granularity

Proposal 9: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB takes the decision to enter paging energy saving mode

- The gNB takes the decision to resume regular paging operation (exit paging energy saving mode)

In split architecture, the gNB-DU will be the entity that can determine potential energy saving gains from paging energy saving. Both the gNB-CU and the gNB-DU know the current paging load. The gNB-CU has a better knowledge to determine upcoming paging load based on information about UEs in RRC_Inactive for which it is anchor gNB, and RAN paging represents a significant part of the paging load in deployed networks. Of course, still the gNB-CU will just transparently forward Paging requests for CN paging and RAN paging requests received over Xn. Also, entering/leaving paging energy saving mode will not be instantaneous but rely on the delay for update of SIB1. Based on this it seems that the gNB-DU could take the decision to enter/exit paging energy saving mode, but the gNB-CU should be aware of this mode in order to take the energy saving mode into account in its paging strategy and paging escalation strategy for RAN paging. It therefore seems the following principles can be taken as base-line for paging energy saving in split architecture:

Proposal 10: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB-DU takes the decision to enter paging energy saving mode and informs the gNB-CU.

- The gNB-DU takes the decision to resume regular paging operation (exit paging energy saving mode) and informs the gNB-CU.

On top of this, in order to enable more precise handling of gNB-CU decided paging escalation for RAN paging, the gNB-DU could inform the gNB-CU about Paging messages that could not be transmitted over the air due to shortage of paging resources.

Proposal 11: RAN3 to discuss whether the gNB-DU should inform the gNB-CU about Paging messages that could not be transmitted over the air due to shortage of paging resources.

# 4	Conclusion

We have made the following observations and proposals:

For OD-SSB:

Observation 1:  OD-SSB case #2 operation doesn’t prevent the cell to operate as a suitable or acceptable cell for idle-mode UEs, or to serve UEs as PCell or PSCell.

Proposal 1: For OD-SSB operation case #2 (always-on SSB is periodically transmitted on the cell), it is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”. gNB-CU assistance is not needed.

Proposal 2: For OD-SSB operation case #1 (no always-on SSB on the cell), the gNB-DU has sufficient information to determine whether it can switch from regular SSB transmission to OD-SSB SCell operation. Additional gNB-CU assistance is therefore not needed.

Observation 2: If the legacy F1AP Cells Allowed to be Deactivated List IE is reused to as assistance information for activation of OD-SSB operation, the gNB-DU may consider either that the signalled cells may enter OD-SSB case #1 operation or may be switched off – and in the latter case these cells will become unavailable for SCell operation.

Proposal 3: If assistance information from gNB-CU to gNB-DU relative to OD-SSB operation is needed, introduce a new, dedicated IE for this purpose.

For adaptation of common signals/channels:

Proposal 4: RAN3 identifies no impact on RAN3 specification coming from adaptation of SSB in time domain in Rel-19.

Observation 3: For PRACH adaptation, two types of PRACH resources will be defined in a cell:

•	Legacy (Rel-15) PRACH resources (minimum set of PRACH resources) – always activated

•	Additional PRACH resources (new in Rel-19 for NES-capable UEs) – activated dynamically

Proposal 5: OAM configures the new additional PRACH resources to the gNB-DU

Proposal 6: Activation and deactivation of additional PRACH resources to be decided autonomously by the gNB-DU.

Proposal 7: RAN3 to further work on the analysis of impact on Rel-19 additional PRACH resources on the legacy RACH optimization feature

Proposal 8: Paging energy saving is done on a per cell granularity

Proposal 9: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB takes the decision to enter paging energy saving mode

- The gNB takes the decision to resume regular paging operation (exit paging energy saving mode)

Proposal 10: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB-DU takes the decision to enter paging energy saving mode and informs the gNB-CU.

- The gNB-DU takes the decision to resume regular paging operation (exit paging energy saving mode) and informs the gNB-CU.

Proposal 11: RAN3 to discuss whether the gNB-DU should inform the gNB-CU about Paging messages that could not be transmitted over the air due to shortage of paging resources.

# References

[1]	R3-252357, SoD – OD-SSB, RAN3# 127-bis