3GPP TSG-RAN WG3 Meeting #128	R3-253533

Malta, 19 – 23 May 2025

Agenda item:	17.3

Source:	Nokia

Title:	(TP for BLCR to TS 38.423) Message definitions for inter-node signalling for On-Demand SIB1

Document for:	Discussion and Decision

# 1	Introduction

RAN3 has agreed on overall signalling procedures for network support of OD-SIB1 feature, however several aspects related to the message definitions remain open. We here provide related discussions and proposals.

# 2	Discussion

## 2.1 XnAP

We here list RAN3’s agreements relevant for the XnAP BLCR (numbering added by us for the purpose of this discussion):

- [1] UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- [2] Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

- [3] The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

- [4] Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- [5] The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

- [6] The procedure uses non UE-associated signaling.

- [7] Cell A stores the UL WUS configuration information after it has received it.

- [8] A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- [9] The procedure uses non UE-associated signalling.

- [10] After Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity.

- [11] From signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx.

- [12] The NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

- [13] UL WUS CONFIGURATION PROVISION REQUEST message : One Choice “Start with UL WUS Condiguration”

- [14] UL WUS CONFIGURATION PROVISION REQUEST message : One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

- [15] It is agreed that UL WUS Configuration broadcast can start as a consequence of UL WUS CONFIGURATION PROVISION REQUEST without need to trigger sending of UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message.

- [16] The Xn/F1 TP capturing the above agreements in the tabular are agreed in: R3-252385 (Xn) and R3-252386 (F1).

One open point with impact on message contents relates to which entity (NES gNB, cell A gNB) determines the required cell A’s for OD-SIB1 operation in a given NES cell. There was so far no corresponding explicit agreement in RAN3, but we believe that at least the agreement 2 indicates that the cell A selection is done at the NES gNB:

Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

We believe that whether one of the options (NES gNB or cell A gNB) is better than the other can be analyzed e.g. in light of whether a centralized (OAM) or distributed (gNB) solution is chosen for the cell A selection. In case of centralized solution, it would be the OAM which determines the required cell A’s per given NES cell. In this case, including cell ID(s) of cell A in the UL WUS CONFIGURATION PROVISION REQUEST message would have the advantage of avoiding requirement that OAM systems of NES gNB and cell A gNB are coordinated.

Observation 1: In case of centralized solution to determine required cell A’s per NES cell, including cell ID(s) of cell A in the UL WUS CONFIGURATION PROVISION REQUEST message comes with the advantage of reduced need for coordination between OAMs of NES gNB and cell A gNB.

However, the NG-RAN has well developed standardized framework for automatic (distributed) detection and handling of neighbour cell relations (ANR, automated Xn setup, …). If implementations would like to leverage this framework and information already available in the gNB in order to determine the NES cell / cell A relationships, we believe either gNB (NES gNB, cell A gNB) could in principle do this. However, in order to ensure a robust solution, we believe that collection of the required information (e.g. measured radio coverage overlap and UE mobility patterns between potential NES cell and potential cell A’s) requires such solution to be activated well before activation of the OD-SIB1 operation in the NES cell. We believe that a cell A gNB will not have such prior information about potential OD-SIB1 activation in other nodes, and would therefore not be able to trigger required data collection to be prepared when the request arrives. Similarly, the NES gNB, even though not in charge of cell A selection, still would need to know towards which node(s) it should trigger the UL WUS Configuration Provision procedure, and also this aspect seems to be a hurdle for any cell A selection mechanism located in the cell A gNB. Such distributed solution should therefore be located in the NES gNB.

Observation 2: A distributed solution for cell A selection should be located in the NES gNB.

Observations 1 and 2 indicate it would be beneficial to include the cell ID of the cell A in the UL WUS CONFIGURATION PROVISION REQUEST message. Also, because the NES gNB may serve multiple cells, inclusion of the NES cell ID in this message is needed to help the cell A gNB to differentiate UL WUS provisioning requests for different cells in the same gNB. The same reasoning applies to the other messages, and we suggest to mandatorily include the NCGI of the cell to which the UL WUS configuration applies (NES cell) and the NCGI of the cell broadcasting the UL WUS configuration in all related messages.

Proposal 1: Include the NCGI of the cell to which the UL WUS configuration applies (NES cell) and the NCGI of the cell broadcasting the UL WUS configuration in all related messages.

Agreements 4, 8 and 9 apply to this procedure, which means that at current stage the codepoint “no broadcast” should be included in the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message, meaning that the NG-RAN node 2 doesn’t broadcast the UL WUS Configuration. Also, as per the agreement 15, this can be used by the cell A gNB to indicate to the NES Cell gNB that it has stopped broadcast of the WUS Configuration.

Proposal 2: Include the following codepoint in the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message: no broadcast.

Furthermore, RAN3 has not yet taken any agreement on whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

Proposal 3: RAN3 to discuss whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

One possible solution could be to enhance Served Cell Information with an indication whether a cell may serve as cell A.

A XnAP TP is provided in annex of this paper reflecting proposals above and further enhancing procedural text and tabular definitions.

## 2.2 F1AP

The following agreements are relevant for the F1AP BLCR:

(10) Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

This agreement is covered by the F1AP BLCR that was agreed at last meeting. By the agreed signalling, the CU-CP may trigger the UL WUS Configuration Provisioning procedure over Xn. When the CU-CP can confirm successful response from the required  cell As (both intra-gNB scenario and inter-gNB scenario should be supported), it should inform the gNB-DU so that it can activate the OD-SIB1 mode of operation. A simple solution for this could be to include an indicator in the Cells to be Activated List Item IE. However, in order to also support ‘discontinue’ and ‘stop’ scenarios, as discussed in section 2.1, we suggest that this indicator contains two codepoints e.g. as follows:

| >>SIB1 Operation Mode | O |  | ENUMERATED (on demand, periodic, …) | SIB1 operation mode as described in TS 38.300 [4]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

This indicator should be included within the Cells to be Activated List Item IE in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 4: Include a SIB1 Operation Mode indicator in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 5: The SIB1 Operation Mode indicator is an extendible ENUMERATED with the following codepoints: on demand, periodic

# 3	Conclusion

We have made the following observations and proposals:

Observation 1: In case of centralized solution to determine required cell A’s per NES cell, including cell ID(s) of cell A in the UL WUS CONFIGURATION PROVISION REQUEST message comes with the advantage of reduced need for coordination between OAMs of NES gNB and cell A gNB.

Observation 2: A distributed solution for cell A selection should be located in the NES gNB

Proposal 1: Include the NCGI of the cell to which the UL WUS configuration applies (NES cell) and the NCGI of the cell broadcasting the UL WUS configuration in all related messages

Proposal 2: Include the following codepoint in the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message: no broadcast.

Proposal 3: RAN3 to discuss whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

Proposal 4: Include a SIB1 Operation Mode indicator in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 5: The SIB1 Operation Mode indicator is an extendible ENUMERATED with the following codepoints: on demand, periodic

# References

[1]		R3-252467, NES BL CR to TS 38.423 after RAN3#127bis

[2]		R3-252466, NES BL CR to TS 38.473 after RAN3#127bis

[3]		R3-252468, NES BL CR to TS 38.300 after RAN3#127bis

# Annex	- TP for NES BL CR TS 38.423

The TP is based on [1].

<<< start of changes >>>

Editor’s Note: The procedure, message and the IE names are FFS

Editors Note: How to address the role of the NG-RAN node1 and NG-RAN node2 to be further discussed.

### 8.x.x	UL WUS Configuration Provisioning

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provisioning procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2 “and request NG-RAN node2 to transmit UL WUS configuration information (FFS)”.

“The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to stop transmission of UL WUS configuration.  (FFS)”

The procedure uses non UE-associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION PROVISION REQUEST message to NG-RAN node2 to provide UL WUS configuration information to NG-RAN node2 and control broadcast of this information in its served cells.  Upon receipt, NG-RAN node2 shall store, per cell indicated in the Broadcast Cell ID IE, the received UL WUS configuration information for the cell indicated in the UL WUS Cell ID IE, or overwrite UL WUS Configuration information earlier received for this cell. The NG-RAN node2 further:

- if Request Type IE is set to  “start”, the NG-RAN node2 shall, if supported, initiate the requested broadcast and respond with the UL WUS CONFIGURATION PROVISION RESPONSE message.

- if Request Type IE is set to “stop” the NG-RAN node2 shall, if supported, stop broadcasting, respond with the UL WUS CONFIGURATION PROVISION RESPONSE message and release thereceived UL WUS Configuration.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the NG-RAN node2 cannot broadcast according to the parameters given in the UL WUS CONFIGURATION PROVISION REQUEST message, it shall send the UL WUS CONFIGURATION PROVISION FAILURE messagewith with an appropriate cause value.

#### 8.x.x.4	Abnormal Conditions

Void.

### 8.x.y	UL WUS Configuration Transmission Status Update (FFS)

#### 8.x.y.1	General

The purpose of the UL WUS Configuration Provision procedure is to report a status change for UL WUS configuration broadcast.

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update, successful operation

The NG-RAN node2 initiates the procedures by sending the UL WUS TRANSMISSION STATUS UPDATE message to report status change for earlier admitted UL WUS configuration broadcast.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

*************** Next change ***************

### 9.x.x.x		UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request or stop provisioning of an UL WUS configuration in one or more cells.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >>UL WUS Provision Start List |  | 1 |  |  | - |  |
| >>> UL WUS Provision Start Item |  | 1 .. < maxnoofCellsProvidingULWUSConfiguration> |  |  | - |  |
| >>>>UL WUS Cell ID | M |  | 9.2.2.7 | Cell at the NG-RAN node1 for which the UL WUS Configuration is requested to start | - |  |
| >>>>UL WUS Configuration | M |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331 [10]. | - |  |
| >>>>Broadcast Cell List |  | 1 |  | List of cells at the NG-RAN node2 recommended to start transmission of the UL WUS Configuration | - |  |
| >>>>>Broadcast Cell Item |  | 1 .. < maxnoofULWUSConfigurationBroadcastCells> |  |  | - |  |
| >>>>>>Broadcast Cell ID | M |  | 9.2.2.7 | Cell at the NG-RAN node2 recommended to broadcast the UL WUS configuration | - |  |
| >Stop |  |  |  |  | - |  |
| >>UL WUS Provision Stop List |  | 1 |  |  | - |  |
| >>> UL WUS Provision Stop Item |  | 1 .. < maxnoofCellsProvidingULWUSConfiguration> |  |  | - |  |
| >>>>UL WUS Cell ID | M |  | 9.2.2.7 | Cell at the NG-RAN node1 for which transmission of the UL WUS Configuration is requested to stop | - |  |

| Range bound | Explanation |  |  |
| --- | --- | --- | --- |
| maxnoofCellsProvidingULWUSConfiguration | Maximum no. of cells for which UL WUS Configuration can be requested. Value is 256 (FFS). |  |  |
| maxnoofULWUSConfigurationBroadcastCells |  | Maximum no. of cells that can be recommended to broadcast an UL WUS configuration. Value is 32 (FFS). |  |

#### 9.1.3.x2	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate successful outcome of the request for at least one of the requested cells in NG-RAN node1.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| UL WUS Transmission Confirmation List |  | 1 |  |  | YES | reject |
| >UL WUS Transmission Confirmation Item |  | 1 .. < maxnoofCellsProvidingULWUSConfiguration> |  |  | - |  |
| >>UL WUS Cell ID | M |  | 9.2.2.7 | Cell at the NG-RAN node1 for which the UL WUS Configuration is confirmed to be transmitted by the NG-RAN node2 | - |  |
| >>Broadcast Cell List |  | 1 |  |  | - |  |
| >>>Broadcast Cell Item |  | 1 .. < maxnoofULWUSConfigurationBroadcastCells> |  |  | - |  |
| >>>>Broadcast Cell ID | M |  | 9.2.2.7 | Cell at the NG-RAN node2 confirmed to broadcast the requested UL WUS configuration | - |  |

#### 9.1.3.x3	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the NG-RAN node2 cannot transmit UL WUS Configuration for any requested cell in NG-RAN node1.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |

#### 9.1.3.x4	UL WUS TRANSMISSION STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report a status change for UL WUS configuration transmission.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.2.3.1 |  | YES | ignore |
| Interface Instance Indication | O |  |  | 9.2.2.39 |  | YES | reject |
| UL WUS Transmission Status Update List |  |  | 1 |  |  |  |  |
| >UL WUS Transmission Status Update Item |  |  | 1 .. < maxnoofCellsProvidingULWUSConfiguration> |  |  |  |  |
| >>UL WUS Cell ID | M |  |  | 9.2.2.7 | Cell at the NG-RAN node1 for which the UL WUS Configuration broadcast status has changed at the NG-RAN node2 | YES | ignore |
| >>Broadcast Cell List |  |  | 1 |  |  |  |  |
| >>>Broadcast Cell Item |  |  | 1 .. < maxnoofULWUSConfigurationBroadcastCells> |  |  |  |  |
| >>>>Broadcast Cell ID | M |  |  | 9.2.2.7 | Cell at the NG-RAN node2 that has changed UL WUS configuration broadcast status | YES | ignore |
| >>>>Transmission Status | M |  |  | ENUMERATED (not transmitting, ...) | Indicates the transmission status of the UL WUS configuration | YES | ignore |

<<< end of changes >>>