3GPP TSG-RAN WG3 Meeting #128	R3-253534

Malta, 19 – 23 May, 2025

Agenda Item:	19.2

Source:	Nokia

Title:	(TP for BLCRs to TS 38.300, 38.401, 38.420, 38.470) Stage 2 updates for Sub-band Full Duplex operation

Document for:	Text Proposal

# 1	Introduction

RAN3 endorsed initial versions of baseline CRs for Sub-band Full Duplex operation at last meeting. However some aspects were missing or imprecise, which we propose to handle further in this paper.

# 2	Discussion

The BLCR to TS 38.300 enhances the description of clause 17.2 relative to CLI management, initially introduced in Rel-16 for dynamic TDD, to also described CLI for the case of SBFD. The legacy text defines CLI as follows: “UL transmission in one cell may interfere with DL reception in another cell”. This definition corresponds to UE-to-UE CLI, which may occur both in case of dynamic TDD and in case of SBFD. However, RAN3 is tasked to provide network support for gNB-to-gNB CLI, and it therefore seems needed to define also such term in clause 17.2. In the present TP (annex A) we propose the following definition: “DL transmission in one cell may interfere with UL reception in another cell”.

Proposal 1: Introduce definition of gNB-to-gNB CLI in BLCR to TS 38.300 as follows: “DL transmission in one cell may interfere with UL reception in another cell”

We also believe that the stage 2 text should clarify that the introduced network signalling conveys information for gNB-to-gNB CLI. This applies to all the stage 2 CRs.

Proposal 2: Clarify in stage 2 text (all BLCRs) that the introduced network signalling conveys information for gNB-to-gNB CLI.

The current stage 2 BLCR for non-split architecture (TS 38.300) describes action taken by the the gNB upon reception of CLI information over the Xn interface. Because this action is taken by the gNB-DU based on received information over the F1 interface in case of split architecture, we propose to introduce similar description in TS 38.401.

Proposal 3: In TS 38.401, describe action taken by the gNB-DU upon reception of CLI information over the F1 interface.

We also include various more editorial updates in the included TPs.

# 3	Conclusion

We have proposed:

Proposal 1: Introduce definition of gNB-to-gNB CLI in BLCR to TS 38.300 as follows: “DL transmission in one cell may interfere with UL reception in another cell”

Proposal 2: Clarify in stage 2 text (all BLCRs) that the introduced network signalling conveys information for gNB-to-gNB CLI.

Proposal 3: In TS 38.401, describe action taken by the gNB-DU upon reception of CLI information over the F1 interface.

TPs for BLCRs to TS 38.300, TS 38.401, TS 38.420 and TS 38.470 are included in annex A-D in this paper.

# References

[1]		R3-253110, BL CR to TS 38.300

[2]		R3-253111, BL CR to TS 38.401

[3]		R3-253112, BL CR to TS 38.420

[4]		R3-253114, BL CR to TS 38.470

# Annex A	- TP for BLCR to TS 38.300

The TP is based on [1].

<<< start of changes >>>

## 17.2	Cross-Link Interference Management

When different TDD DL/UL patterns are used between neighbouring cells, UL transmission in one cell may interfere with DL reception in another cell: this is referred to as Cross Link Interference (CLI).

To mitigate CLI, gNBs can exchange and coordinate their intended TDD DL-UL configurations over Xn and F1 interfaces; and the victim UEs can be configured to perform CLI measurements. There are two types of CLI measurements:

-	SRS-RSRP measurement in which the UE measures SRS-RSRP over SRS resources of aggressor UE(s);

-	CLI-RSSI measurement in which the UE measures the total received power observed over RSSI resources.

Layer 3 filtering applies to CLI measurement results and both event triggered and periodic reporting are supported.

In case of Sub-band full duplex (SBFD) operation CLI may also be present. Additionaly, DL transmission in one cell may interfere with UL reception in another cell: this is referred to as gNB-to-gNB CLI. A gNB reports gNB-to-gNB CLI related information measured by its serving cells to neighbour gNBs. The neighbour gNB should evaluate the received information and may take interference mitigation actions when necessary.

<<< end of changes >>>

# Annex B	- TP for BLCR to TS 38.401

The TP is based on [2].

<<< start of changes >>>

### 7.3	Cross-Link Interference Management

The Cross-Link Interference Management function in non-split gNB case is specified in TS 38.300 [2].

In case of split gNB architecture, the gNB-CU forwards the TDD DL/UL patterns received from neighboring nodes to each concerned gNB-DU. The gNB-DU reports the TDD DL/UL patterns of its serving cells to the gNB-CU if Cross-Link Interference is detected.

In Sub-band full duplex (SBFD) operation, Cross Link Interference (CLI) may also be present. In case of split gNB architecture, the gNB-DU reports gNB-to-gNB CLI related information measured by its serving cells to the gNB-CU if CLI is detected. The gNB-CU forwards gNB-to-gNB CLI related information received from served gNB-DUs and from neighboring gNBs to each concerned gNB-DU. The concerned gNB-DU should evaluate the received information and may take interference mitigation actions when necessary.

<<< end of changes >>>

# Annex C	- TP for BLCR to TS 38.420

The TP is based on [3].

<<< start of changes >>>

# 6	Xn interface procedures

## 6.1	General

The Xn interface supports procedures over the control plane (Xn-C) and user plane (Xn-U).

## 6.2	Control plane protocol procedures

<<<Unchanged part omitted>>>

### 6.2.x	CLI mitigation procedure

Editor’s Note: The name for procedure and message might be updated pending on further discussion.

The CLI mitigation procedure is used to report the result of gNB-to-gNB CLI measurements, and to request the CLI mitigation.

-	CLI MEASUREMENT UPDATE

<<< end of changes >>>

# Annex D	- TP for BLCR to TS 38.470

The TP is based on [4].

<<< start of changes >>>

## 5.2	F1-C functions

<<<Unchanged part omitted>>>

### 5.2. x	CLI mitigation function

The CLI mitigation function enables the transfer of gNB-to-gNB CLI measurements and CLI mitigation request between gNB-DU and gNB-CU.

<<<Next change>>>

## 6.1	Control plane procedures

<<<Unchanged part omitted>>>

### 6.1. x	CLI mitigation procedure

Editor’s Note: The name of procedure and message might be updated pending on further discussion.

The following procedure is used to report the result of gNB-to-gNB CLI measurements and to request CLI mitigation:

-	CLI MEASUREMENT UPDATE

<<< end of changes >>>