3GPP TSG-RAN WG3 Meeting #128	R3-253545

Malta, MT, 19 – 23 May, 2025

Title:	(TP to BLCR for TS 38.413) Further discussion of MBS broadcast service and mapped cell and TAI pre-reconfiguration

Agenda Item:	14.3

Source:	Huawei

Document for:	other

# 1	Introduction

Previous RAN3 meetings already agreed that the intended MBS service area provided from the AMF to the gNB can be mapped cell ID(s), TA(s), or geographical service area information as captured in the NGAP BLCR in [1]. And RAN2 running TS 38.331 CR is updated in [2]. Taking the above into account, this paper provides our further considerations and proposals.

In this paper, we also continue to discuss the mapped cell and the TAI pre-configuration in the NR NTN regenerative payload.

# 2	Discussion

## 2.1	MBS Update

In the NGAP BLCR [1], the MBS service area information refers to the IntendedServiceAreaInfo IE as defined in TS 38.331 as follows.

#### 9.3.1.x Intended Service Area Information This IE provides information on the Intended Service Area for MBS Broadcast Service. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Intended Service Area Information; M; OCTET STRING; Encoded in the same format as the IntendedServiceAreaInfo IE as defined in TS 38.331 [18].

According to the latest RRC running CR [2], the IntendedServiceAreaInfo IE, consisting of an intended service area ID and an area representation of either circle or polygon, is structured as follows:

#### – SIBXX SIBXX contains the description of the intended service area of a broadcast service in an NTN cell. SIBXX information element -- ASN1START -- TAG-SIBXX-START SIBXX-r19 ::= SEQUENCE { intendedServiceAreaList-r19 IntendedServiceAreaList-r19 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } IntendedServiceAreaList-r19 ::= SEQUENCE (SIZE (1.. maxNrofMBS-Area-r19)) OF IntendedServiceAreaInfo-r19 IntendedServiceAreaInfo-r19 ::= SEQUENCE { intendedServiceAreaId-r18 MBS-IntendedAreaID-r19, areaCoordinates-r19 CHOICE { polygonArea OCTET STRING, circleArea SEQUENCE { referenceLocation-r19 ReferenceLocation-r17, distanceRadius-r19 INTEGER(0..65535) } } } -- TAG-SIBXX-STOP -- ASN1STOP

The reason for RAN2 to introduce this intended service area ID, is to link the intended service areas and the different MBS services included in the MCCH by the intended service area ID. Hence this should be decided by the RAN node itself.

RAN2-129 meeting agreements

- In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list

- It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH)

Hence there is no reason that the area ID is provided by the CN over NGAP. And the current semantics descriptions for the Intended Service Area Information is not aligned with rules for referring to the TS 38.331. It is proposed that the MBS service area information refers to the areaCoordinates contained in the IntendedServiceAreaInfo as defined in TS 38.331, and update the corresponding semantics descriptions.

- RAN3 to update the MBS service area information by referring to the areaCoordinates as defined in TS 38.331, and update the corresponding semantics descriptions (see Annex1).

## 2.2	Mapped cell Id

In the previous meetings, there were proposals that the gNB ID contained in the mapped cell ID should remain constant in a geographical area. But we see some restrictions for such proposal.

First, it would be natural to pre-configure the satellite ephemeris information on the CN. When the satellite gNB ID remains constant as the regenerative payload moves, similar to the TN gNB, it is easy for the CN to discern the relationship between the gNB ID, cell ID and its location, and finally to know the geographical location corresponding to the mapped cell ID. Hence there is no need to restrict the gNB ID to an earth-static location.

Second, it would be very challenging to define the exact “geographic area” for the gNB ID contained in the mapped cell, considering the moving cell scenario and the different coverage range of different satellites (e.g., with different altitudes). For example, when two regenerative payloads with different ephemeris cover the different range of areas, it is very little hard to define a unified “geographic area” concept. Also, it would be also difficult to define such “geographic area” when a satellite covers a space across multiple “geographic areas”.

Observation 1: The AMF is able to know the relationship between the gNB ID and its location by pre-configuration if the satellite gNB ID remains constant as it moves.

Observation 2: For the case of moving cell and different satellites with different coverage range, defining the geographic area would be very challenging.

- Not to specify that the gNB ID in the mapped cell ID remains constant in a geographical area.

## 2.3 OAM/pre-configuration

The RAN3#125bis made the following agreement:

- The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration

With the further reply LS-es from RAN3 and SA5 [2-5], it can be observed that:

- SA5 agreed RAN3 solutions and would support it in the normative work.

- When further looking at the TR 28.874, SA5 agreed to include NTNgNBCapability (including gNB ID and TAI list) towards the AMF, which is associated with a time window.

- SA5 support AMF is pre-configured with multiple subsequent time windows instances within which the NTN gNB ID and supported TAI list are specified, which avoids the OAM configuration for each time as the satellite moves.

Hence a TP for TS 38.300 is provided to capture the OAM based configuration in the Annex.

- RAN3 to agree the stage 2 TP for capturing the OAM based configuration.

# 3	Conclusion

In this paper, we provide the following proposal:

- RAN3 to update the MBS service area information by referring to the areaCoordinates as defined in TS 38.331 and update the corresponding semantics descriptions (see Annex1).

Observation 1: The AMF is able to know the relationship between the gNB ID and its location by pre-configuration if the satellite gNB ID remains constant as it moves.

Observation 2: For the case of moving cell and different satellites with different coverage range, defining the geographic area would be very challenging.

Proposal 2:	Not to specify that the gNB ID in the mapped cell ID remains constant in a geographical area (see Annex2).

Proposal 3:	RAN3 to agree the stage 2 TP for capturing the OAM based configuration (see Annex2).

# 4	Reference

- R3-252525	(BL CR to 38.413) Support for Regenerative Payload and MBS broadcast in NR NTN 	CATT, et al.

- R2-2502676 Running RRC CR for NR NTN phase 3	Ericsson

- R3-250030/S5-247311, LS reply on OAM requirements to support regenerative payload, SA5#15

- R3-250895, Reply LS on OAM requirements to support regenerative payload transport links, RAN3#127

- R3-251528, Reply LS to LS on reply to LS on OAM requirements to support regenerative payload, RAN3#127b

# Annex1	Text Proposal for TS 38.413 against R3-252525

#### 9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| MBS Intended Service Area List |  | 0..<maxnoofIntendedAreasforMBS> |  |  |
| >Intended Service Area Coordinates | M |  | 9.3.1.x |  |

#### 9.3.1.x	Intended Service Area Coordinates

This IE provides information on the Intended Service Area Coordinates for MBS Broadcast Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Intended Service Area Coordinates | M |  | OCTET STRING | Includes the areaCoordinates contained in the IntendedServiceAreaInfo IE as defined in TS 38.331 [18] |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBS-ServiceArea ::= CHOICE {

locationindependent		MBS-ServiceAreaInformation,

locationdependent		MBS-ServiceAreaInformationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBS-ServiceArea-ExtIEs} }

}

MBS-ServiceArea-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformationItem

MBS-ServiceAreaInformationItem ::= SEQUENCE {

mBS-AreaSessionID				MBS-AreaSessionID,

mBS-ServiceAreaInformation		MBS-ServiceAreaInformation,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformationItem-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-IntendedServiceAreaList		CRITICALITY ignore	EXTENSION MBS-IntendedServiceAreaList	PRESENCE optional },

...

}

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NR-CGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF TAI

MBS-IntendedServiceAreaList ::= SEQUENCE (SIZE(1..maxnoofIntendedAreasforMBS)) OF IntendedServiceAreaCoordinates

IntendedServiceAreaCoordinates ::= OCTET STRING

MBS-SessionID ::= SEQUENCE {

tMGI								TMGI,

nID								NID																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-SessionID-ExtIEs} } 		OPTIONAL,

...

}

MBS-SessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex2: TP for BLCR for TS 38.300

<<<<<<<<<<<<<<<<<<<< Changes begin>>>>>>>>>>>>>>>>>>>>

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS;

-	The Cell Identity used for MBS broadcast service.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

The AMF may provide the gNB with the service area information for MBS broadcast service. This information may be signaled as cell ID list, TAI list, and/or MBS intended service area list, as defined in TS 38.413 [26]. Based on this received information, the gNB provides the intended service area information to the UEs in the relevant cell(s) as defined in TS 38.331 [12].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 16.14.Y	Supported TAI in regenerative payload

The supported TAIs of the regenerative payload may be configured to the AMF by the O&M. Under the configuration from the O&M, the on-board gNB may not update the supported TAI list towards the AMF when the TAI(s) changes.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>