3GPP TSG-RAN WG3 Meeting #128	R3-253547X

Malta, MT, 19th – 23rd May 2025

Agenda Item:	15.2

Source:	Huawei

Title:	(TP for TS 36.300) Support of Full eNB as Regenerative Payload - General issues and LS response to SA2

Document for:	Discussion

# 1	Introduction

In the RAN3#127 meeting, the following agreements were achieved for the support of full eNB as regenerative payload of Rel-19 IoT NTN [1]:

Introduce stage 2 description of Store and Forward operation.

Introduce stage 2 description of full eNB as regenerative payload.

Introduce the S1 Removal procedure.

In this paper, we further discuss the support of full eNB as regenerative payload based on the above agreements.

We also further discuss the S&F mode transition issue.

# 2	Discussion

## 2.1 eNB only as regenerative payload without CN functions

In the RAN3#126 meeting, RAN3 received the LS from SA2 on the Regenerative-based Satellite Access for E-UTRAN/EPC, where only the eNB is on-board, without any CN components [2]. Based on it, the WID was updated accordingly, and the stage 2/3 details were agreed to support the eNB-only regenerative payload architecture at RAN3#127.

To assist the SA2 progress, RAN3 should send a reply LS back to SA2, indicating the support for the eNB-only regenerative payload architecture, and including the potential agreements made at this meeting for SA2’s consideration.

- RAN3 to send a reply LS to SA2, as response to R3-247016/ S2-2410918, containing all potential agreements having SA2 impacts made at this meeting (the draft is in [5]).

## 2.2 S1 suspend/resume indication/procedure

For the regenerative payload, the NG and S1 removal procedures for NR NTN and IoT NTN were agreed and captured in the corresponding baseline CRs, considering the onboard gNB/eNB movement resulting the discontinuity of the NG/S1 interface when the gNB/eNB moves out of the service area of the AMF/MME.

Regarding the suspend/resume procedure, RAN3 agreed that no new NGAP suspend/resume procedures will be introduced for NR NTN at the RAN3#127 bis meeting, as follows [3]:

- No new NGAP suspend/resume procedures in R19.

The same agreement can be taken for IoT NTN.

- RAN3 to agree that no new S1 suspend/resume procedures are needed.

In addition, the NG suspend/resume indication towards the AMF is under further discussion for NR NTN. If there is any conclusion made for NR NTN, the IoT NTN can also simply follow the same conclusion.

- Whether S1 suspend/resume indication is needed for IoT NTN is pending on NR NTN conclusion.

## 2.3 Feeder link switch

For NR NTN, the RAN3#125 meeting had the following agreement:

- RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked

This means the during the feeder link switch, the AMF is not changed. This agreement should also apply for the eNB only as regenerative payload. We consider this is an important agreement applicable for all RRC state UEs, which should be captured in the stage 2.

- For eNB only as regenerative payload, RAN3 assumes that during the feeder link switch, the MME is not changed for the UEs. RAN3 to agree the Stage 2 TP to capture this agreement.

In addition, currently the S1 interface supports the TNL recovery using the UE Information Retention IE, supported in the S1 SETUP REQUEST message. Specifically, the eNB can establish the new TNL association and the new S1 interface towards the same MME immediately after the feeder link switch, and include the UE Information Retention IE in the S1 SETUP REQUEST. In this way, the MME will not move the UE to IDLE after the old TNL is shut down, but retains the UE context. When receiving the UE Information Retention IE in the S1 SETUP REQUEST, the MME regards the TNL is recovered, and the UE context is transferred to the new S1 interface. Note that this solution is also documented in the second bullet of solution #35 in SA2 TR 23.700-29 [4]:

---Excerpt from TR 23.700-29--- Scenario 1 (no AMF/MME change): In case of deployment scenario 1, the TNL between gNB/eNB and AMF/MME would change while the gNB/eNB is serving a specific set of cells and Tracking Areas. To update the TNL association between gNB/eNB and AMF/MME different options are possible based on the existing standard: - Support for multiple TNL associations is available in 5GS (i.e. N2), as described in TS 23.501 [2] and TS 38.413 [12]. In this case the gNB can initiate a new TNL association via the new feederlink and remove the old TNL association that was active over the old feederlink. - Support for replacing an old N2/S1 interface with a new N2/S1 interface while preserving the active UE contexts is supported for both N2 and S1, as described in TS 38.413 [12] and TS 36.413 [14]. In this case the gNB/eNB establishes a new SCTP association via the new feederlink and sends a N2/S1 SETUP Request including the UE Retention Information IE. The AMF/MME accepts to retain the existing UE related contexts and signalling connections. The AMF/MME sends a N2/S1 SETUP Response and includes the UE Retention Information IE. - To update the N3/S1-U GTP-U tunnels with the new gNB/eNB TNL address, the gNB/eNB can provide the new tunnel endpoint information (F-TEIDs) to the core network by sending PDU SESSION RESOURCE MODIFY INDICATION / E-RAB MODIFICATION INDICATION. ---End of excerpt---

Hence, for eNB only as regenerative payload, the existing mechanism can support the feeder link switch without introduction of multiple TNL associations over the S1 interface.

- For eNB only as regenerative payload architecture, the existing mechanism can support the feeder link switch without introduction of multiple TNL associations over the S1 interface.

## 2.3 S1 S&F mode transition

Previous RAN2 meetings discussed the S&F operation mode transition with the following agreements.

RAN2-129 agreements: 1. RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change 2. The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information) 3. Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time). 4. A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply RAN2-129bis agreements: 1. For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1. 2. We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS 3. We don’t introduce any new release cause related to S&F operation

According to the above, RAM2 assumes that a S&F mode transition does not necessarily imply an MME change.

RAN2 has assumed that a S&F mode change does not necessarily imply an MME change.

Building on this assumption, further analysis is provided from RAN3 perspective as follows.

First, for the transition from the S&F mode to normal mode, for those served UEs served by the on-board eNB before the mode transition, they were already served by the on-board CN. Clearly there is no strong motivation to update the MMEs. That is, there is no need for the eNB to trigger the S1 based handover to update the MME for connected UEs. And there is no need to update the broadcast TAC list to trigger the UE to perform TAU update. Otherwise, this would lead to a lot of signalling overload, which is not favourable for the satellite communication.  Hence given the interfaces between the on-board eNB and on-board CN are still maintained, there is no need to update the MME for all RRC state UEs.

Similarly, for the transition from the normal mode to the S&F mode, for those UEs already served by the on-board eNB before the mode transition, it is better to select the on-board CN. Otherwise, the same issues as analysed above also apply here.

Therefore, when the S&F mode changes, there is no need to switch down/on the on-board CN and change the UE’s serving MME.  The following proposals are made.

- RAN3 assumes that during the S&F mode transition (i.e., from the S&F mode to the normal mode or vice versa), the MME is not changed for its served UEs (for all connected/inactive/idle UEs).

- RAN3 to agree the stage 2 TP to capture the above.

Given that this agreement may have SA2 impact considering the MME change also involves SA2 work, and SA2 is also discussing the S&F mode transition, it is better to inform SA2 our agreements.

- RAN3 to send an LS to SA2 to inform the agreements as described in Proposal 1.

# 3	Conclusion

This paper provides analysis for the eNB regenerative payload, the following proposals are made:

- RAN3 to send a reply LS to SA2, as response to R3-247016/ S2-2410918, containing all potential agreements having SA2 impacts made at this meeting (the draft is in [5]).

- RAN3 to agree that no new S1 suspend/resume procedures are needed.

- Whether S1 suspend/resume indication is needed for IoT NTN is pending on NR NTN conclusion.

- For eNB only as regenerative payload, RAN3 assumes that during the feeder link switch, the MME is not changed for the UEs. RAN3 to agree the Stage 2 TP to capture this agreement.

- For eNB only as regenerative payload architecture, the existing mechanism can support the feeder link switch without introduction of multiple TNL associations over the S1 interface.

- RAN2 has assumed that a S&F mode change does not necessarily imply an MME change.

- RAN3 assumes that during the S&F mode transition (i.e., from the S&F mode to the normal mode or vice versa), the MME is not changed for its served UEs (for all connected/inactive/idle UEs).

- RAN3 to agree the stage 2 TP to capture the above (see Annex).

- RAN3 to send an LS to SA2 to inform the agreements as described in Proposal 6.

# 4. Reference

- RAN3 #127 chairman notes.

- R3-247016 Reply LS on Support of Regenerative-based Satellite Access, SA2

- RAN3 #127bis chairman notes.

- TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19)

- Annex1, [DRAFT] Reply LS on Support of Regenerative-based Satellite Access

# 5. Annex1: TP for TS 36.300 against R3-250861

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 23.21.5	Switchover

#### 23.21.5.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure.

Both hard and soft feeder link switchover are supported in transparent or regenerative NTN payloads.

#### 23.21.5.2	Assumptions

A feeder link switchover may result in transferring the established connection for the affected UEs between two eNBs.

For soft feeder link switchover, an NTN payload is able to connect to more than one NTN Gateway during a given period i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switchover, an NTN payload only connects to one NTN Gateway at any given time i.e. a radio link interruption and/or NG link interruption may occur during the transition between the feeder links.

#### 23.21.5.3	Procedures

The NTN control function determines the point in time when the feeder link switchover between two eNBs is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UEs' context between the two eNBs at feeder link switchover is performed by means of either S1 based or X2 based handover, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

NOTE:	For regenerative payload, the MME is not changed for NTN payload served all RRC state UEs during the feeder link switch over.

# 6. Annex2: LS to SA2

3GPP TSG-RAN3 Meeting #128	R3-xxxxxx

Malta, MT, 19th – 23rd May 2025

Title:	[DRAFT] [DRAFT] Reply LS on Support of Regenerative-based Satellite Access

Response to:	LS R3-247016/ S2-2410918 on Support of Regenerative-based Satellite Access

Release:	Rel-19

Work Item:		NR_NTN_Ph3-Core, 5GSAT_Ph3_ARCH

Source:	Huawei [will be RAN3]

To:	SA2

Cc:	RAN2, RAN

Contact person:	Philippe Reininger

philippe.reininger@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN3 thanks SA2 for the LS on the Support of Regenerative-based Satellite Access.

For the Regenerative-based Satellite Access for E-UTRAN/EPC, RAN plenary already approved the updated WID in RP-243278 to support the eNB only as the regenerative payload. RAN3 would like the provide the following agreements for SA2 consideration.

For the eNB only as the regenerative payload

- S1 removal procedure is introduced.

- No new S1 suspend/resume procedures.

- RAN3 assumes that during the feeder link switch, the MME is not changed for the UEs.

- The existing S1 mechanisms can support the feeder link switch over.

For the S&F operation mode

- During the S&F mode transition (i.e., from S&F mode to normal mode or vice versa), the MME(s) is not changed for the eNB regenerative payload served UEs.

# 2	Actions

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above feedback into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#129	2025-08-25 - 2025-08-29		India (TBC), IN

RAN3#129-bis	2025-10-13 -  2025-10-17		Prague, CZ

RAN3#130	2025-11-17 -  2025-11-21		Dallas, US

# 7. Annex3: TP for TS 36.300 against R3-250861

## 4.12	Support of Non-Terrestrial Networks

E-UTRAN supports radio access over non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs. Support for non-terrestrial networks encompasses platforms that provide radio access through Geosynchronous orbits (GSO), Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO) or High Altitude Platform Systems (HAPS).

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

With NGSO satellites, the eNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while eNB operating with GSO satellites can provide Earth fixed cell coverage or quasi-Earth-fixed cell coverage.

Store and Forward satellite operation is supported as described in TS 23.401 [17].

NOTE:	In case of the Store and Forward mode transition, the MME(s) is not changed for the NTN regenerative payload served UEs.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>>