3GPP TSG-RAN WG3 Meeting #128	R3-253549

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:	17.2

Source:	Ericsson

Title:	On-demand SSB SCell operation

Document for:	Discussion, Decision

# 1	Introduction

The revised WID for “Enhancements of network energy savings for NR” [1] in Rel-19 states the following objectives:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

This paper discusses the “on-demand SSB SCell operation” topic (Objective 1).

A complete list of RAN1 and RAN2 agreements on this objective is provided at the end of the paper.

# 2	Discussion

## 2.1	Observations and current state of discussion

In our previous paper [2], we observed the following:

- On-demand SSB SCell operation is a new SCell operation mode where SSB transmissions on an SCell are triggered when needed and indicated to the UE via RRC or MAC CE based signaling.

- On-demand SSB parameters are preconfigured to the UE via RRC based signaling.

- On-demand SSB transmission is tightly connected to SCell operation at the gNB-DU, and it is triggered by the gNB-DU.

- On-demand SSB transmission enables energy-efficient SCell operation (with sparse or no always-on SSB) in cases where SSB-less SCell operation is not possible.

Besides, in the summary of the offline discussion on on-demand SSB SCell operation at the RAN3#127 meeting [3], the following was noted:

For the allowing On-demand SSB SCell, we discussed the two way-forward as follows:

- WF1: No new List from the gNB-CU to the gNB-DU for “allowing OD SSB SCell” is needed, instead we can reuse, if needed, the Rel-18 existing “Cells Allowed to be Deactivated List” (CU to DU). Potential rewording of the procedure may be needed to make the “list” more general.

- WF2: It is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”.

But the above is not agreed, there are two concerns raised during the discussion:

- One concern is that gNB-DU does not always need the “CU to DU” assistance in order to go to the OD-SSB SCell operation.

- Another concern is how and whether to handle the transition from PCell normal operation to OD-SSB SCell only operation.

To be continued at the next meeting on the above basis.

During the RAN3#127-bis meeting, there was an initial understanding to move forward with WF1, as it was noted in the summary of the offline discussion on on-demand SSB SCell operation [4]:

It is proposed to take the following as baseline and to be discussed at the next meeting (no need to capture into the Chairman notes).

Take the following texts as baseline:

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may take it into account to enable the on-demand SSB SCell operation for the indicated cells when they are operating as SCells.

In what follows, we first discuss the potential impact (of on-demand SSB SCell operation) on the Xn interface and then discuss potential impact on the F1 interface.

## 2.2	Discussion on potential Xn impact

On-demand SSB transmission on a cell is only applicable to scenarios where the cell is configured as an SCell (at the UE). Consequently, on-demand SSB transmission on the cell is not visible to any other UEs and cannot impact them, thus cannot benefit them.

Moreover, in CA operation, all serving cells of a UE belong to one gNB, more specifically, one gNB-DU.

On-demand SSB transmission in a cell of a gNB(-DU) is visible to and impacts only (Rel-19 NES-capable) UEs served by that gNB(-DU) with that cell as an SCell.

Consequently, there is no need to inform a neighbor node about on-demand SSB transmission or request a neighbor node to perform on-demand SSB transmission. Because of this and since SCell operation is a gNB-internal process, we conclude that on-demand SSB SCell operation has no Xn impact.

RAN3 to conclude that on-demand SSB SCell operation has no Xn impact.

## 2.3	Discussion on potential F1 impact

In past RAN3 meetings, it was discussed whether the gNB-DU can autonomously decide on configuring on-demand SSB SCell operation or whether it needs some form of assistance from the gNB-CU, e.g., a list of cells in which on-demand SSB transmission is allowed.

However, we got the impression that the discussion was not actually about whether the gNB-DU is allowed to configure and transmit on-demand SSB for UEs’ SCells but whether it is allowed to operate a cell without any always-on SSB.

Note that there is the common understanding in RAN1 that always-on SSB corresponds to legacy SSB (before Rel-19). Moreover, as part of the Rel-18 NES work, the Cells Allowed to be Deactivated List IE was introduced on the F1AP GNB-CU CONFIGURATION UPDATE message, from the gNB-CU to the gNB-DU. According to the F1AP specification, the following applies:

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

SSB beams are defined by SSB transmissions (in certain directions).

In our view, deactivating the SSB beams means stopping the transmissions of legacy/always-on cell-defining SSB on the sync raster (at least until Rel-19). This generally impacts the cell’s coverage for UEs in RRC Idle/Inactive mode as well as the cell’s ability to serve as a PCell for UEs in RRC Connected mode, at least in the affected SSB beams. This is why the gNB-DU cannot autonomously decide on this. The gNB-CU, on the other hand, has a better understanding of the NW deployment, the current load across neighboring cells, and it can coordinate with other RAN nodes.

In legacy (prior to Rel-19), deactivating the SSB beams (in a cell) means stopping the transmissions of always-on SSB, which requires signaling of a per-cell permission from gNB-CU to gNB-DU.

We understand that, for a cell whose NR CGI (NR Cell Global Identifier) is included in the Cells Allowed to be Deactivated List IE, the gNB-DU is allowed to cease transmission of legacy/always-on SSB. Reversely, for a cell whose NR CGI is not included, the gNB-DU must transmit always-on SSB (as before introduction of this Rel-18 NES feature). Consequently, we believe the question of whether a cell is operated with or may be operated without always-on SSB has already been addressed by legacy (Rel-18) signaling over F1.

The question of whether a cell is operated with or may be operated without always-on SSB has already been addressed by legacy (Rel-18) signaling over F1.

We share the view that gNB-DU does not always need assistance from gNB-CU to apply on-demand SSB SCell operation, we see no reason to limit the use of on-demand SSB at the gNB-DU. It is understood that the gNB-DU configures (via RRC based signaling) and triggers (via RRC or MAC CE based signaling) on-demand SSB. On-demand SSB transmissions are additional SSB transmissions, in addition to legacy SSB transmissions, if any. Hence, the gNB-DU should be free to apply on-demand SSB as it sees fit, since it is the entity that is in control of SCell operation and that observes the effect on CA performance and NW energy consumption. The gNB-DU should not require permission from the gNB-CU for on-demand SSB SCell operation in a cell, similar to legacy, where the gNB-DU does not need assistance from the gNB-CU for SSB-less SCell operation (cf. [2]).

In legacy SSB-less SCell operation, the gNB-DU does not need assistance or permission from the gNB-CU for SSB transmission omission.

RAN3 to agree that the gNB-DU does not require permission from the gNB-CU for on-demand SSB SCell operation in a cell.

In this sense, we believe that the following text (proposed at the previous RAN3 meeting) is appropriate with “may” statement:

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may take it into account to enable the on-demand SSB SCell operation for the indicated cells when they are operating as SCells.

According to RAN1 agreements, L3 measurement on on-demand SSB is supported. However, further work on this topic is left up to RAN2 and RAN4. The usability of L3 measurement on on-demand SSB at the gNB-CU is limited. Since on-demand SSB is only for configured SCell, not for neighbor cell, L3 measurement on on-demand SSB can only be used to decide on SCell removal, not to decide on SCell addition (or swap). Thus, L3 measurement on on-demand SSB is not applicable to SCell candidates. For UE mobility decisions, in particular CA related mobility decisions, the gNB-CU either relies on L3 measurement on always-on SSB, Case #2, or NW deployment knowledge (i.e., blind SCell configuration), Case #1.

The usability of L3 measurement on on-demand SSB at the gNB-CU, e.g., for UE mobility decisions, in particular CA related mobility decisions, is limited.

Considering the limitations of on-demand SSB transmissions in Rel-19, we do not see sufficient motivation to introduce signaling over F1 to enable the gNB-CU to request the gNB-DU to transmit on-demand SSB, at least not in this release. On-demand SSB is transmitted by the gNB-DU in connection to SCell activation (solely controlled by the gNB-DU), e.g., to speed up the SCell activation process. The gNB-DU does not need an indication from the gNB-CU to trigger on-demand SSB transmission for this purpose.

Moreover, we see no reason for the gNB-DU to inform the gNB-CU about transmission of on-demand SSB. It is not obvious what the gNB-CU would do with such information (also see the discussion related to Observation 1). It was suggested by one company in [5] that the gNB-CU would know to expect L3 measurements. However, whether the UE reports L3 measurements depends on the reporting configuration (e.g., measurement events). Besides, it is not clear why the gNB-CU would need such a pre-warning for receiving L3 measurements. Instead, L3 measurements can be sent to the gNB-CU as in legacy.

Not to introduce further signaling over F1 for requesting on-demand SSB transmission (gNB-CU to gNB-DU) or informing about on-demand SSB transmission (gNB-DU to gNB-CU) in the current release.

We think that the on-demand SSB related configuration is best included in the gNB-DU-owned part of the UE configuration (e.g., in the cell config). As such, the legacy F1 signaling, e.g., the existing UE context management procedures, would suffice to enable the gNB-DU to configure the on-demand SSB related parameters and indicate the on-demand SSB transmission to the UE, like the de-/activation state of the on-demand SSB configuration (which is already agreed in RAN3). In that case, there would be no obvious RAN3 impact from on-demand SSB SCell operation.

RAN3 to assume introduction of the on-demand SSB related configuration in the gNB-DU-owned part of the UE configuration (but it is up to RAN2 to discuss and decide).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	On-demand SSB transmission in a cell of a gNB(-DU) is visible to and impacts only (Rel-19 NES-capable) UEs served by that gNB(-DU) with that cell as an SCell.

Observation 2	In legacy (prior to Rel-19), deactivating the SSB beams (in a cell) means stopping the transmissions of always-on SSB, which requires signaling of a per-cell permission from gNB-CU to gNB-DU.

Observation 3	The question of whether a cell is operated with or may be operated without always-on SSB has already been addressed by legacy (Rel-18) signaling over F1.

Observation 4	In legacy SSB-less SCell operation, the gNB-DU does not need assistance or permission from the gNB-CU for SSB transmission omission.

Observation 5	The usability of L3 measurement on on-demand SSB at the gNB-CU, e.g., for UE mobility decisions, in particular CA related mobility decisions, is limited.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN3 to conclude that on-demand SSB SCell operation has no Xn impact.

Proposal 2	RAN3 to agree that the gNB-DU does not require permission from the gNB-CU for on-demand SSB SCell operation in a cell.

Proposal 3	Not to introduce further signaling over F1 for requesting on-demand SSB transmission (gNB-CU to gNB-DU) or informing about on-demand SSB transmission (gNB-DU to gNB-CU) in the current release.

Proposal 4	RAN3 to assume introduction of the on-demand SSB related configuration in the gNB-DU-owned part of the UE configuration (but it is up to RAN2 to discuss and decide).

# 4	References

RP-250343, Revised WID on enhancements of network energy savings for NR

R3-250551, On-demand SSB SCell operation and other Energy Saving topics, Ericsson

R3-250800, Summary of Offline Discussion on on-demand SSB SCell operation, Huawei (Moderator)

R3-252357, Summary of offline discussions: OndemandSSB, Huawei (Moderator)

R3-250129, (TP to BLCR for TS 38.473) Discussion on on-demand SSB SCell operation, Huawei

# 5	RAN1 and RAN2 agreements for Objective 1

  - RAN1: On-demand SSB for SCell operation

    - RAN1#116

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

    - RAN1#116bis

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

  - For on-demand SSB on the cell, downselect between the following alternatives

    - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

    - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

      - FFS: Further limitations to on-demand SSB

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

    - FFS further details on L1 and/or L3 measurement

Agreement

The following agreement from RAN1#116 is modified (in red)

- For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

Agreement

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

    - RAN1#117

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

Agreement

- At least support L1 measurement based on on-demand SSB

  - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

    - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Agreement

For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

  - The SSB time domain positions of on-demand SSB burst are configured by gNB.

- FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

- Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

- FFS: Whether the value of T is predefined or indicated/configured by gNB

- FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE.

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Location of on-demand SSB burst

  - Downlink transmit power of on-demand SSB

  - FFS: Other parameters

  - FFS: Whether each of above parameters is configured/indicated explicitly or not

    - RAN1#118

Agreement

- Update the previous RAN1 agreement as follows.

  - At least support L1 measurement based on on-demand SSB

    - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

      - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

      - The support of LTM is a separate discussion point

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement

For a cell supporting on-demand SSB SCell operation, at least the following is supported

- On-demand SSB on the cell is not located on synchronization raster.

- On-demand SSB on the cell is non-cell-defining SSB

FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Agreement

Support L3 measurement based on on-demand SSB

- Further work on L3 measurement is up to RAN2/RAN4

Agreement

LS to RAN2 for on-demand SSB SCell operation is agreed. Final LS in R1-2407438.

Agreement

The previous RAN1 agreement made in RAN1#117 is revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

  - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

  - (Working assumption) T=T_min

  - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

LS on timeline for On-demand SSB operation on SCell is agreed in R1-2407565.

    - RAN1#118bis

Agreement

- For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

  - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

  - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - Option 3: Configuration/indication of the duration of on-demand SSB transmission window

  - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

  - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

  - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

  - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

  - FFS: Each option is applicable to which Cases or Scenarios

  - FFS: Details related to each of the above options

Agreement

- For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1.

  - Sub-carrier spacing of the on-demand SSB

    - FFS if this can be absent

  - Physical Cell ID of the on-demand SSB

  - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index

  - Downlink transmit power of on-demand SSB

  - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

Agreement

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB.

  - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB

  - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB

  - FFS: Whether OD-SSB and always on SSB have same beam or not

Conclusion

No consensus on the support of on-demand SSB SCell operation triggered by UE.

Agreement

The previous RAN1 agreement is partly confirmed and further revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

      - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - (Working assumption) T=T_min

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases.

- Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB

- Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

    - RAN1#119

Agreement

Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1:

- The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms.

- The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB.

- RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

Further update to be made based on RAN1#119 progress.

Agreement

Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1:

- The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Agreement

Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1:

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Agreement

- For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2.

  - For Case #1 (i.e., No always-on SSB on the cell),

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives.

    - Alt A: Same as for Case #1

    - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

Agreement

- New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.

Agreement

Down-select at least one of the following alternatives.

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

- Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster.

Down-select at least one of the following alternatives.

- Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration.

- Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

Agreement

Response to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?) of Obj.1:

- RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst

- RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

    - RAN1#120

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, at least for the case when the center frequency locations of always-on SSB and on-demand SSB are same, down-select one of the following.

- Alt Time-A: It is not allowed that time location of an SSB within on-demand SSB burst overlaps with time location of any SSB within always-on SSB burst.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-B: Time-domain locations of always-on SSB are a subset of time-domain locations of on-demand SSB.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-C: The specification has no restriction with regards to overlapping

Agreement

- For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC,

  - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e.,

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

    - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

      - By defining reference point as SFN where AO-SSB burst exists

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

Agreement

For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

Conclusion

The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19

- Scenario #3A and Case #1

- Scenario #3A and Case #2

Above does not impact discussion on SSB periodicity adaptation in time domain

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

    - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain.

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

- NOTE: AO-SSB periodicity is not adapted

Send an LS to RAN4 to inform them of the above agreement. Final LS in R1-2501633.

[Active] Proposal #6-3a (SCS of T):

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, when UE determines time instance A, the SCS to determine the value of T is down selected among the following options

- Option 1: the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication

- Option 2: the minimum of “the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication” and “the SCS of the active DL BWP where UE receives on-demand SSB”

- Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

  - RAN2: On-demand SSB for SCell operation

    - RAN2#125bis

- No discussion in RAN2 this meeting

    - RAN2#126

- No discussion in RAN2 this meeting

    - RAN2#127

- Scenarios on OD-SSB:

  - RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

- OD-SSB transmission indication:

  - RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

  - New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

- Measurement on OD-SSB:

  - Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

  - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

  - RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

R2-2407414: Proposal 3: RAN2 WG to discuss the issue of false measurement report triggering due to no SSB transmission when on-demand SSB is deactivated.

- RAN2 does not handle this issue in OD-SSB

  - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

    - RAN2#127bis

- OD-SSB transmission indication:

  - No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

  - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

  - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

    - RAN2#128

- Not discussed

    - RAN2#129

- Agreements on OD-SSB

  - RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.