3GPP TSG-RAN3 Meeting #128                                                                    R3-253563

# St.Julian, Malta, May 19th – 23rd, 2025                                     Revision of R3-251758

Agenda Item:	19.2

Source: 	Qualcomm Inc.

Title: 	Signaling design to enable NR SBFD operation.

Document for:	Discussion/Decision

# Background

Following are agreements for SBFD WI from previous RAN3 WG meetings.

RAN3#125bis Agreements:

RAN3 agrees to Exchange the following information over Xn interface and F1 interface:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

- Measurement resource configuration

- Strongest DL beam information (FFS on the details)

- CLI-mitigation request (FFS on the details)

RAN3#126 Agreements:

- The SBFD time and frequency location configuration with cell granularity in Served Cell Information NR IE shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

- Exchange reference signalling configuration(SSB and NZP CSI-RS) in interface management procedures: Xn Setup and NG-RAN node Configuration Update procedure?

RAN3#127 Agreements:

- RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs in Served Cell Information NR IE.

Exchange of SBFD configuration:

- gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message.

- The SBFD time and frequency configuration is encoded as container defined by RAN2.

Exchange of measurement configuration:

- For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec.

- For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition?

Exchange of measurement result:

- For exchange of strongest DL beam information over Xn, SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64).

Exchange of CLI-mitigation request:

- To define CLI-mitigation request as an optional IE.

- FFS on the details.

- How to exchange the strongest DL beam information and the CLI-mitigation request over Xn, e.g. to reuse existing procedures such as NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures?

- Check progress in RAN1/2 and solution discussion…

RAN3#127bis Agreements:

- Introduce NZP-CSI-RS resource configuration in Served Cell Information NR IE of XnAP and F1AP specifications, which refers to the NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE defined in TS 38.331.

- Introduce new procedure to provide the strongest DL beam information and the CLI-mitigation request over XnAP and F1AP.

- gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Neighbour Cell Information List IE in gNB-CU Configuration Update message.

- The SBFD time and frequency location configuration with cell granularity in Served Cell Information IE is introduced in F1AP F1 SETUP REQUEST message and F1AP gNB-DU CONFIGURATION UPDATE message

- The SBFD time and frequency location configuration with cell granularity in the Neighbour Cell Information List IE is introduced in F1AP gNB-CU CONFIGURATION UPDATE message

- For exchange of NZP CSI-RS over F1, to put a new IE in Served Cell Information IE from gNB-DU to gNB-CU, and in Neighbour Cell Information List IE from gNB-CU to gNB-DU with reference to RAN2 definition

- In order to transfer the SSB info from gNB-CU to gNB-DU, to introduce an explicit IE in the Neighbour Cell Information List IE.

- To introduce a class 2 procedure, with tentative message name “CLI MEASUREMENT UPDATE”, in which both the strongest DL beam information and the CLI-mitigation request could be included as two optional IEs.

Check progress in RAN1/2 and solution discussion…

In this contribution, we will discuss remaining RAN3 related signalling design proposals to enable successful R19 NR SBFD operation.

# Sub Band (Non-Overlapping) Full Duplex (SBFD) Signalling design framework

## Signalling to exchange SFBD RACH Configuration exchange

During NR SON/MDT discussions, in both F1 and Xn interface, Served Cell Information-> NR PRACH Configuration-> NR PRACH Configuration List IE was introduced to avoid PRACH collision between neighbouring gNB cells. For SBFD, PRACH configuration enhancements are actively being discussed in both RAN1 and RAN2.

There are 2 RACH configuration options under discussion for SBFD configuration:

RAN1#118 Agreement :

For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following:

- For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon.

- For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

SBFD

SBFD

RO

DL

FL

SBFD

RO

DL

UL

RO

Non-SBFD

FL

SBFD

RO

Figure 6: RACH configuration Option 1 (Additional RO in red, Legacy RO in orange)

RAN2#127bis Agreements:

The following two RACH configuration options are considered for SBFD based random access:

    - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed.

    - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters.

The RACH configuration for SBFD is transmitted via SIB1.

RAN1 and RAN2 agreed to support new RACH configuration for SBFD aware UEs, which is option 2 as mentioned above. RAN3 need to introduce option 2 NR RACH configuration in both F1 and Xn signalling messages under Served Cell Information and Neighbour NR Cells for SON List IEs.

Thus, we propose

RAN3 agrees to introduce option 2 SBFD RACH configuration (which will have separate configuration for  legacy ROs and SBFD specific ROs) related IEs as part of Served Cell Information IE and Neighbour NR Cells for SON List IEs in both F1 and Xn signalling messages.

## Inter gNB - Cross Link Interference (CLI)

When SBFD mode is configured by gNBs, victim gNB’s UL reception in UL subband may be interfered by another neighbor aggressor gNB’s DL transmission in DL subband(s) in SBFD symbols. This type of inter-gNB interference is analogous to R16 RIM (Remote Interference Management) due to dynamic/flexible TDD configuration.

Figure 7

In order to enable interference measurement by each gNB, either SSB and/or NZP CSI-RS can be used by each gNB for interference measurement. This requires each gNB has to provide its serving cell SSB and/or NZP CSI-RS configuration to neighbouring gNBs. For interference detection, each gNB based on their implementation has to perform SSB and/or NZP CSI-RS measurements by taking SFBD configuration and SSB/NZP CSI-RS configuration into account.

For both FR1 and FR2-1, it is possible for victim gNB to perform interference measurements based on SSB and/or NZP CSI-RS configuration.

In case of FR1, typically beams are wide, victim gNB can measure received power of SSB/NZP-CSI-RS and report to the aggressor gNB. In order for aggressor gNB to take implementation based scheduling action to supress its interference to other victim gNBs, it is desirable for victim gNB to provide measured channel measurement (RSRP, RSRQ and/or RSSI value) results to aggressor gNB per cell level and/or beam level.

Upon receiving interference information from victim gNB, Aggressor gNB based on implementation is expected to do channel estimation of beams received from victim gNB and aggressor gNB can decide to adjust its Tx beams pre-coding to generate beam pattern aiming to avoid interference towards gNBs (i.e., beam nulling).

FR 2 reciprocity based beam measurements are depicted in below figure 8.

In case of FR2-1, SSB and NZP CSI-RS beams are much more granular compared to FR1 beams. Victim gNBs are expected to measure aggressor gNBs SSB beams, and/or NZP CSI-RS beams and select strongest DL beams causing interference for reporting to the aggressor gNBs. For SSB based measurements, SSB beam index will be reported and for NZP CSI-RS beam measurements (each NZP CSI-RS Resource corresponds to a CSI-RS beam and CSI-RS Resource Indicator (CRI)), CRI value representing a NZP CSI-RS beam will be reported along with victim gNB measured interference value. When and how often victim gNB provides interference measurements to aggressor gNBs (i.e., periodically or certain event based when interference level is about certain implementation based threasholds) is left upto to victim gNB implementation.

Based on interference measurement reports received by aggressor gNB from victim gNBs, it is left upto aggressor gNB implementation what action to take to mitigate inter-gNB interference. For example, aggressor gNB may use various scheduling techniques including TDM, FDM or Spacial domain schemes like beam pairing or beam nulling for interference mitigation.

During RAN3#127, RAN3 discussed about whether to provide single strongest beam or multiple interfering beams to be reported from victim gNB to aggressor gNB. Unfortunately, without any technical reasoning companies are blindly following RAN1 LS wording of strongest beam reporting even through there is no explicit RAN1 agreement to limit to only strongest beam reporting. In practice, there is no evidence that only one strongest beam causing interference. This is setting bad precedence for signalling frame work design and this will result in victim gNB sending interfering beams in multiple signalling messages and it adds unnecessary signalling overhead. It was strange that some companies are even trying to avoid sending an LS to RAN1 asking for their feedback to specify multiple interfering beam(s) reporting.

Additionally if victim gNB does not indicate what are radio channel conditions experienced by victim gNBs, just reporting beam IDs will not help aggressor gNBs to properly determine how much interference is caused by interfering beams. So it is desirable for victim gNB to have ability to report multiple beams and their measured radio channel conditions as well.

Based on above discussion we propose

For each reported SSB beam ID and CRI (CSI RS Resource Set ID), victim gNB reports radio channel measurements per beam level and cell level granularity (i.e., RSRP, RSRQ and RSSI radio channel quality metrics) to assist aggressor gNB to determine level CLI seen by victim gNB.

## L1 and L3 measurement based Intra-gNB Inter-UE Cross Link Interference (CLI) configuration

Figure 9

In R16 Inter-UE CLI for dynamic TDD scenario, L3 measurement configuration and reporting was used.  For R19 SBFD, in order to support fast and bursty interference measurement and reporting, RAN1/RAN2 are currently working on L1 measurement configuration and reporting.

For L1 based SBFD Inter-UE CLI measurements, RAN1 agreed to support aperiodic L1 measurement reports and L1 measurement resources can be periodic, aperiodic or semi-persis tent configuration.

L1 measurement configurations are generated by DU. But the question is whether to configure L1 measurements configuration to UE is decided by CU or DU. If it is CU making decision then CU has to request DU to generate L1 measurements resource configuration for Inter-UE CLI measurements purpose. Note that CU is aware of Inter gNB SBFD configuration as well.

It is possible that both L3 and L1 based measurement configuration are possible for Inter-UE CLI measurements but RAN1 is still discussing whether it is possible to configure both or limit to either L1 or L3 based configuration for UEs to simplify complexity. Based on RAN1 progress, RAN3 need to discuss CU-DU interactions for co-existence of L3 and L1 based measurement configurations.

Thus, we propose

RAN3 to discuss CU-DU interaction for L1 based inter-UE CLI measurement configuration and trigger mechanism, which node decides to trigger inter-UE CLI L1 measurement configuration.

RAN3 to discuss co-existence between L3 and L1 based inter-UE CLI measurement configuration and trigger mechanism.

# Summary

Based on above discussion, we have the following observations and proposals for SBFD signalling enhancements.

Signalling to exchange SBFD RACH configuration exchange:

Proposal 1.	RAN3 agrees to introduce option 2 SBFD RACH configuration (which will have separate configuration for  legacy ROs and SBFD specific ROs) related IEs as part of Served Cell Information IE and Neighbour NR Cells for SON List IEs in both F1 and Xn signalling messages.

Inter-gNB CLI Information exchange:

Proposal 2.	For each reported SSB beam ID and CRI (CSI RS Resource Set ID), victim gNB reports radio channel measurements per beam level and cell level granularity (i.e., RSRP, RSRQ and RSSI radio channel quality metrics) to assist aggressor gNB to determine level CLI seen by victim gNB.

L1 and L3 measurement configuration related for SBFD:

Proposal 3.	RAN3 to discuss CU-DU interaction for L1 based inter-UE CLI measurement configuration and trigger mechanism, which node decides to trigger inter-UE CLI L1 measurement configuration.

Proposal 4.	RAN3 to discuss co-existence between L3 and L1 based inter-UE CLI measurement configuration and trigger mechanism.

# References

# [1] RP-241614: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] R1-2407533 : LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation