3GPP TSG RAN WG3 Meeting #128	R3-253573

St Julian’s, Malta, 19 - 23 May, 2025

Agenda Item:	14.3

Source:	ETRI, Huawei, Ajou University

Title:	Discussion on Hard Feeder Link Switchover

Document for:	Discussion

- Introduction

The Rel-19 NR-NTN WID[1] includes the following objectives to support regenerative payload with full gNB on board:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In the previous meeting, RAN3 got the following progresses on NR-NTN regenerative payload:

RAN3#127bis: - Support RRC_INACTIVE UE in NTN by implementation in R19. - No enhancements on location-based CHO for NTN in R19. - In case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used. - No new NGAP suspend/resume procedures in R19. Work on the stage2/3 details…

In this contribution, we propose to discuss an alternative solution for hard feeder link switchover.

- Discussion

  - Agreed solution for the hard FLSO

In the previous RAN3 meetings, the soft and hard FLSO mechanisms for regenerative payload have been discussed.  Unlike the case of soft FLSO, the on-board gNB can maintain only one feeder link at a time in case of hard FLSO.  Therefore, the hard switch of feeder link from the old NTN-GW to the new NTN-GW leads to the interruption of NG interface.

As for UL data transmission, the on-board gNB can

- detect the upcoming hard FLSO based on the information received from the OAM (including the time window/schedule of the successive feeder link switchovers)

- and then suspend the transmission of UL NG-C/U packets before disconnecting the feeder link with the old NTN-GW.

Thus, there is no issue on the UL packet loss problem.

However, as for DL data transmission, the AMF/UPF

- cannot know the upcoming hard FLSO before the on-board gNB informs the AMF about the FLSO execution

- and hence continue the transmission of DL NG-C/U packets to the old NTN-GW even after the on-board gNB is disconnected with the old NTN-GW.

We thus need a solution to the DL packet loss problem, and it was agreed in the last RAN3 #127-bis meeting that

- the on-board gNB sends an indication to the AMF in order to suspend and resume data and signaling procedures over NG interface when the hard feeder link switch over occurs

[TP for TS 38.300] [TP for TS 38.413]

Observation 1: RAN3 agreed in the last meeting that the on-board gNB sends an indication to the AMF in order to suspend and resume data and signaling procedures over NG interface when the hard feeder link switch over occurs.

  - Alternative solution for the hard FLSO

In principle, we have no objection to the agreed solution, but we’d like to discuss an alternative solution which has minimum impact on RAN3 stage 2 and 3 specifications.

    - Analysis of the agreed solution

As shown in the example of signaling flow (Figure 1 (A) Hard FLSO case), the agreed solution requires the on-board gNB to

- detect the upcoming hard FLSO based on the information received from the OAM

- and suspend the transmission of UL NG-C/U packets

- and also send RAN CONFIGURATION UPDATE message to request the 5GC to suspend the transmission of DL NG-C/U packets before disconnecting the feeder link with the old NTN-GW.

It is note that the on-board gNB needs to send an additional RAN CONFIGURATION UPDATE message to the AMF in order to inform the 5GC about switching the feeder link to the new NTN-GW.

Consequently, the agreed solution introduces signaling overhead due to multiple RAN CONFIGURATION UPDATE procedures between the gNB and the AMF, both before and after the switch of feeder link.

Observation 2: The agreed solution introduces signaling overhead due to multiple RAN CONFIGURATION UPDATE procedures between the gNB and the AMF, both before and after the switch of feeder link.

(A) Hard FLSO Case                                                               (B) Soft FLSO Case

Figure 1: Example of signaling flow based on the agreed solution

    - Alternative solution for hard FLSO

As mentioned above, RAN3 has assumed an OAM-based solution to the UL packet loss problem in which

- the OAM can provide NTN related information including the time window/schedule of the successive feeder link switchovers and

- based on this information, the on-board gNB can detect the upcoming hard FLSO and suspend UL NG-C/U packet transmission before disconnecting the feeder link with the old NTN-GW.

RAN3 has also agreed an OAM-based solution for the supported TAI list of on-board gNB in which

- the OAM can provide the information of supported TAIs for NTN gNB to the AMF

It offers the benefit of reducing NG interface signaling between the gNB and the AMF.

Observation 3: In order to reduce NG interface signaling, RAN3 has agreed an OAM-based solution for the supported TAI list of on-board gNB in which the OAM can provide the information of supported TAIs for NTN gNB to the AMF.

Motivated by that, we can consider an alternative OAM-based solution to the DL packet loss problem.

[New OAM Requirement for hard FLSO]

- The OAM can provide NTN related information including the time window/schedule of the successive feeder link switchovers to the AMF as well as the on-board gNB.

With this requirement, as for DL data transmission, the AMF/UPF can

- detect the upcoming hard FLSO based on the information received from the OAM (including the time window/schedule of the successive feeder link switchovers)

- and then suspend the transmission of DL NG-C/U packets before the on-board gNB is disconnected with the old NTN-GW.

We thus have no issue on the DL packet loss problem.

As shown in the example of signaling flow (Figure 2 (A) Hard FLSO case), the alternative OAM-based solution does not introduce additional RAN CONFIGURATION UPDATE procedure before the hard FLSO occurs.  Therefore, it is advantageous in terms of reduced signaling overhead between the gNB and the AMF , as well as minimum impact on the RAN3 specifications.  The on-board gNB additionally has the advantage of performing the same signaling procedures over NG interface regardless of the type of feeder link switch as shown in Figure 2.

(A) Hard FLSO Case                                                               (B) Soft FLSO Case

Figure 2: Example of signaling flow based on the alternative solution

Observation 4: We can consider an alternative OAM-based solution to the DL packet loss problem in which the OAM can provide NTN related information including the time window/schedule of the successive feeder link switchovers to the AMF as well as the on-board gNB. Based on the OAM information, the AMF/UPF can detect the upcoming hard FLSO and suspend the transmission of DL NG-C/U packets before the on-board gNB is disconnected with the old NTN-GW.

Observation 5: The alternative OAM-based solution is advantageous in terms of reduced signaling overhead between the gNB and the AMF, as well as minimum impact on the RAN3 specifications.  The on-board gNB additionally has the advantage of performing the same signaling procedures over NG interface regardless of the type of feeder link switch.

Proposal 1: RAN3 to discuss the alternative OAM-based solution for the hard FLSO of regenerative payload in NTN.

- Conclusion

Based on the discussion in section 2, we have the following observations and proposals:

Observation 1: RAN3 agreed in the last meeting that the on-board gNB sends an indication to the AMF in order to suspend and resume data and signaling procedures over NG interface when the hard feeder link switch over occurs.

Observation 2: The agreed solution introduces signaling overhead due to multiple RAN CONFIGURATION UPDATE procedures between the gNB and the AMF, both before and after the switch of feeder link.

Observation 3: In order to reduce NG interface signaling, RAN3 has agreed an OAM-based solution for the supported TAI list of on-board gNB in which the OAM can provide the information of supported TAIs for NTN gNB to the AMF.

Observation 4: We can consider an alternative OAM-based solution to the DL packet loss problem in which the OAM can provide NTN related information including the time window/schedule of the successive feeder link switchovers to the AMF as well as the on-board gNB. Based on the OAM information, the AMF/UPF can detect the upcoming hard FLSO and suspend the transmission of DL NG-C/U packets before the on-board gNB is disconnected with the old NTN-GW.

Observation 5: The alternative OAM-based solution is advantageous in terms of reduced signaling overhead between the gNB and the AMF, as well as minimum impact on the RAN3 specifications.  The on-board gNB additionally has the advantage of performing the same signaling procedures over NG interface regardless of the type of feeder link switch.

Proposal 1: RAN3 to discuss the alternative OAM-based solution for the hard FLSO of regenerative payload in NTN.

- Reference

RP-240775	Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3