3GPP TSG-RAN WG3 Meeting #128	R3-253583

St Julian’s, Malta, 19 – 23 May, 2025

Agenda item:	11.2

Source:	Nokia, FiberCop, Deutsche Telekom, Jio Platforms (JPL)

Title:	AI/ML Slicing Discussion

Document for:	Decision

# 1	Introduction

In RAN3 #127bis we made the following agreements related to AI/ML slicing discussions:

Slice UE performance request:

For Slice UE performance request, take Opt2: Extend Existing Report Characteristics Bitmap.

Failed Report Characteristics:

Extending the existing Node Measurement Failed Report Characteristics IE to include slice-level UE performance metrics for indicating the failed result for slice-level UE performance metrics.

Per Slice UE performance report:

Add Slice UE performance IE in the DATA COLLECTION UPDATE message.

One issue that remained open in the previous meeting was how the slice capacity is scaled with respect to the total capacity of the cell in case multiple slices are available for a cell if we reuse the existing Slice Available Capacity IE for encoding of the reported predictions. Towards this end, there were proposals on whether the predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity. In the last meeting we agreed to introduce the measured Cell Capacity Class Value (CCCV) within the predicted slice available capacity instead, which subsequently implied that we introduce a new IE for the Predicted Slice Available Capacity:

Introduce the measured CCCV within the predicted slice available capacity.

Furthermore, we had discussions related to the granularity of UE Performance metrics and we captured the following agreement:

Per S-NSSAI per UE performance metrics is reported as part of the Slice UE Performance IE in the Data Collection Update message. An FFS on how the metrics are calculated will be added in the TP. If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement.

In this contribution we provide our views regarding some remaining open issues of AI/ML slicing.

# 2	Discussion

### 2.2.1	Slice-level UE Performance granularity

The topic of UE Performance metric granularity has been discussed extensively over the previous meetings. RAN3 had agreed to use existing UE performance metrics per S-NSSAI defined in SA5 as part of UE Performance reporting. In the last meeting, some companies raised the issue that UE Performance metrics may not be measured per slice but per DRB per slice according to SA5 specifications. Therefore, we captured an agreement that if no SA5 definition exists RAN3 shall decide whether those metrics are derived based on an average or weighted average.

Checking SA5 specification TS 28.558 we notice that indeed measurements for UE Performance can be reported on a slice level granularity namely

- Delay Measurements

  - Average delay DL air-interface: This is the Average delay in the DL, providing the average time it takes for packet transmission over the air interface in the downlink direction. It is measured at the gNB-DU. The measurement name has the form DRB.AirIfDelayDlUe.Filter,  where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. It is described in TS 28.558, clause 6.3.1.1.1.

  - Average delay DL in gNB-DU: This is the average RLC SDU delay in the DL within the gNB-DU, for initial transmission of all RLC packets. It is measured at the gNB-DU. The measurement name has the form DRB.RlcSduDelayDlUe.Filter, where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. It is described in TS 28.558, clause 6.3.1.1.2.

  - Average delay DL on F1-U: This is the average GTP packet delay DL on the F1-U interface. It is measured at the gNB-CU-UP. The measurement name has the form DRB.PdcpF1DelayDlUe.Filter,  where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. It is described in TS 28.558, clause 6.3.1.1.3.

  - Average delay DL in CU-UP: This is the average PDCP SDU delay on the downlink within the gNB-CU-UP, for all PDCP packets. It is measured at the gNB-CU-UP. The measurement name has the form DRB.PdcpSduDelayDlUe.Filter, where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. It is described in TS 28.558, clause 6.3.1.1.4.

Similarly, in the uplink:

  - Average delay UL on over-the-air interface: This is the air interface UL delay. It is defined in TS 38.314, clause 4.2.1.2.2, named "Average over-the-air interface packet delay in the UL per DRB per UE" (and in TS 28.558, clause 6.3.1.1.6). It is measured at the gNB-DU. The measurement name has the form DRB.AirIfDelayUlUe.Filter,  where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

  - Average RLC packet delay in the UL: This measures RLC packet delay in the UL, i.e., the delay within the gNB-DU. It is defined in TS 38.314, clause 4.2.1.2.3, named "Average RLC packet delay in the UL per DRB per UE" (and in TS 28.558, clause 6.3.1.1.7). It is measured at the gNB-DU. The measurement name has the form DRB.RlcDelayUlUe.Filter, where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

  - Average delay UL on F1-U): This is the average GTP packet delay UL on the F1-U interface. It is measured using the same metric as the average delay DL on F1-U defined in TS 28.558, clause 6.3.1.1.3. It is measured by the gNB-CU-UP. The measurement name has the form DRB.PdcpF1DelayUlUe.Filter,  where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

  - Average PDCP re-ordering delay in the UL: It measures PDCP re-ordering delay in the UL, i.e., delay within the gNB-CU-UP. It is defined in TS 38.314, clause 4.2.1.2.4, named "Average PDCP re-ordering delay in the UL per DRB per UE” (and in TS 28.558, clause 6.3.1.1.8). It is measured at the gNB-CU-UP. The measurement name has the form DRB.PdcpReordDelayUlUe.Filter, where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

- Average Throughput UL/DL

- Average UL UE throughput in gNB: This measurement provides the average UE throughput in DL. It is intended for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU. This measurement is measured by the gNB-DU. It is described in TS 28.558, clause 6.3.1.4.2.

The measurement name has the form

DRB.UEThpUl,or optionally 
DRB.UEThpUl.QoS, where QoS identifies the target quality of service class, and 
DRB.UEThpUl.SNSSAI, where SNSSAI identifies the S-NSSAI.

- Average DL UE throughput in gNB: This measurement provides the average UE throughput in DL. It is intended for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU. It is described in TS 28.558, clause 6.3.1.4.1.

The measurement name has the form 
DRB.UEThpDl, or optionally 
DRB.UEThpDl.QoS, where QoS identifies the target quality of service class, and 
DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

- Average Packet Loss DL

- DL F1-U Packet Loss Rate: This measurement provides the fraction of PDCP SDU packets which are not successfully received at the gNB-DU. It is a measure of the DL packet loss on the F1-U interface. This measurement is measured by the gNB-DU. It is defined in TS 28.558, Clause 6.3.1.3.3.

The measurement name has the form DRB.F1UpacketLossRateDlUe, and optionally DRB.F1UPacketLossRateDlUe.QoS where QoS identifies the target quality of service class, and DRB.F1UPacketLossRateDlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

From the above, we notice that, at least for UL delay measurements where TS 28.558 refers to TS 38.314, these measurements may be referred in TS 28.558 with slice granularity, however the corresponding reporting is done on a per DRB per slice basis and not per full (aggregated) S-NSSAI. We assume that DL delay measurements use the same DRB granularity as the UL delay measurements, which also corresponds to delay measurement signalling defined in TS 38.415 and TS 38.425.

Observation 1: Average packet delay metrics in TS 28.558 are measured per DRB per slice, and aggregation per S-NSSAI is not specified.

However, RAN3 also received at this meeting reply LS from SA5 [1] for Rel-18 UE-level UE Performance metrics, providing information relative to DL and UL throughput measurements defined in TS 28.558, e.g.:

- DRB.UEThpDl

- DRB.UEThpUl

According to [1], and despite the measurement names referring to DRB, the throughput measurements are aggregated to provide metrics at the UE level.

In Rel-18, RAN3 has had already discussions on how to obtain UE Performance metrics per UE granularity and a common understanding was captured in RAN3 #124, namely that “UE level performance is derived as the average value of per DRB level measurements of the UE.”. In our view, defining weighted average as an additional way to average UE Performance metrics is confusing. Instead, there should be a common way of calculating average UE Performance across Rel-18 and Rel-19.

Observation 2: There should be a common way of calculating average UE Performance across Rel-18 and Rel-19.

In addition, the group responsible to define measurements is SA5 and not RAN3. It is therefore reasonable to expect that SA5 shall define the missing per S-NSSAI granularity of UE Performance metrics. We think that those metrics should follow Rel-18 principles on averaging so that a common averaging method is used across Rel-18 and Rel-19.

Proposal 1: Common averaging methods should be used across Rel-18 and Rel-19 for UE Performance metrics.

Therefore, we believe that we must confirm with SA5 whether the UE Performance measurements are measured per S-NSSAI. If not, tthen we think that we should ask SA5 to define UE Performance metrics per S-NSSAI granularity. Those metrics can be defined by averaging of the per DRB level measurements of the UE.

Proposal 2: Send an LS to SA5 to request SA5 to define UE Performance metrics per S-NSSAI based on averaging of per DRB level measurements of the UE, if those are confirmed not to be currently defined in SA5 specifications.

A draft LS to SA5 is provided in R3-253585.

A UE that is requested to report slice-level UE Performance, reports UE Performance for all its active slices. A question that arises is on the behaviour when a UE performance cannot be calculated per slice at the target node during the reporting period.

Proposal 3: If UE at the target node has no active slices for a reporting period, the slice-level UE Performance report is skipped.

### 2.2.2	Some stage 3 issues

The BL CR for XnAP R3-252504 was updated after the last meeting by including the agreed TPs R3-252355, R3-252432, and R3-252448. The agreed TP in R3-252432 did not have any criticality assigned to the Slice Measurement Initiation Result IE in the DATA COLLECTION RESPONSE message.  In the agreed TP no criticality is assigned to this IE. After the meeting, the rapporteur assigned the criticality to “ignore”.

Since Slice Measurement Initiation Result IE is inside Cell Measurement Initiation Result List IE which has criticality reject, we believe that Slice Measurement Initiation Result IE should have the same criticality.

Proposal 4: Update the assigned criticality of Slice Measurement Initiation Result IE to “reject”.

Dimensioning of UE performance reporting per slice:

The current XnAP BLCR provides, for each of up to 16 UE reports, the following structure for UE performance reporting per slice (clause 9.2.3.x2 Slice UE Performance):

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice UE Performance |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAI UE Performance List |  | 1 |  | Indicates the UE performance per network slice (details are FFS) |
| >>S-NSSAI UE Performance Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Slice Based UE Performance | M |  | UE Performance 9.2.3.179 | [It is FFS how the slice based UE performance metrics are derived] [It is FFS whether and how the Slice Average Packet Loss DL IE is defined] |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

However, it should be considered that a given UE is, at a given point in time, served by a single PLMN. The loop on PLMN identity therefore doesn’t seem needed. When it comes to the loop per slice, the maximum number of slices supported per UE will depend on UE implementations. However, current network signalling allows for a maximum number of 256 PDU sessions per UE, and, as starting point for RAN3’s discussion, we propose to use this number also for maximum number of slices per UE for UE performance reporting.

Proposal 5: Restructure UE performance reporting per slice by removing the loop per PLMN ID and introducing maximum number of slices per UE for UE performance reporting equal to 256 (changes required in XnAP BLCR clause 9.2.3.x2 Slice UE Performance).

We provide a TP for BL CR to TS 38.423 in R3-253584.

# 3	Conclusion

In this paper we make the following observations and proposals:

Observation 1: Average packet delay metrics in TS 28.558 are measured per DRB per slice, and aggregation per S-NSSAI is not specified.

Observation 2: There should be a common way of calculating average UE Performance across Rel-18 and Rel-19.

Proposal 1: Common averaging methods should be used across Rel-18 and Rel-19 for UE Performance metrics.

Proposal 2: Send an LS to SA5 to request SA5 to define UE Performance metrics per S-NSSAI based on averaging of per DRB level measurements of the UE, if those are confirmed not to be currently defined in SA5 specifications.

Proposal 3: If UE at the target node has no active slices for a reporting period, the slice-level UE Performance report is skipped.

Proposal 4: Update the assigned criticality of Slice Measurement Initiation Result IE to “reject”.

Proposal 5: Restructure UE performance reporting per slice by removing the loop per PLMN ID and introducing maximum number of slices per UE for UE performance reporting equal to 256 (changes required in XnAP BLCR clause 9.2.3.x2 Slice UE Performance).

# References

[1]		R3-253023 - S5-252056, Reply LS on per-UE UE performance metrics, SA5