3GPP TSG-RAN WG3 Meeting #128	R3-253589

St Julian’s, Malta, 19 – 23 May, 2025

Agenda item:	11.4

Source:	Nokia

Title:	(TP to TS 38.423) Remaining issues for AI/ML Mobility Optimization in NR-DC

Document for:	Decision

# 1	Introduction

In RAN3 #126 we made the following agreement regarding the procedures to trigger the collection of UE Performance measurements for NR-DC:

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side.

Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

In addition, in RAN3 #127bis we made the following agreement:

The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked.

The following list of exit conditions to stop the collection of UE performance at SN side:

- the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE;

- the NR-DC with the S-NG-RAN node for the UE is released.

The following open points were also captured:

UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture?

Whether to introduce assistance information (predicted PSCell ID) from MN to SN?

Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

On a different topic of split architecture support for Rel-18 use cases we agreed that in split architecture the packet delay measured for UE Performance feedback sent from gNB-CU-UP to gNB-CU-CP is per DRB:

Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level.

We also took a WA that UE throughput in PDCP level can be considered for Rel-19:

WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.

In this contribution, we provide further views on the above topics.

# 2	Discussion

## 2.1 UE performance measurements in NR-DC

Over the last meetings, different options have been discussed on the granularity of UE Performance metrics exchanged over the interfaces for the different use cases discussed in NG-RAN AI/ML. We discussed the granularity of sending UE Performance metrics over Xn for AI/ML Slicing and for AI/ML Mobility Optimization in NR-DC scenarios. We also discussed the granularity of sending UE Performance metrics in split architecture for the support of the Rel-18 use cases and some discussions took place on the entity that measures UE performance. A WA was captured in split architecture support use case: WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.

We think that also for AI/ML Mobility Optimization use case UE Performance metrics need to be measured by the PDCP entity and the WA should be also valid for this scenario. This is because if the measurements are collected at PDCP layer, then the measurements can always be collected from a single layer and single node i.e., from PDCP Layer in Master Node in case of MN-Terminated MCG, SCG or Split bearer and from PDCP Layer in Secondary Node in case of SN-Terminated MCG, SCG or Split bearer. Master Node configures Data Collection initiation and reporting to the Secondary node only if there are one or more SN-Terminated bearers. Otherwise, MN can measure UE Performance measurements from its own PDCP layer.

In split architecture, it was agreed that Packet Delay is measured at PDCP layer for split architecture and sent from gNB-CU-UP to gNB-CU-CP. We believe that Packet Delay also in AI/ML Mobility Optimization for NR-DC is collected at the gNB-CU-UP.

Proposal 1: In AI/ML Mobility Optimization in NR-DC, collect the Average Packet Delay at the gNB-CU-UP.

While waiting for the reply LS from SA5 on the feasibility of measuring Average UE Throughput UL/DL and Packet Loss Rate UL/DL at PDCP, we think we should also port the above WA taken in the context of split architecture also for AI/ML Mobility Optimization scenario:

Proposal 2: We make the following WA: WA: In case of AI/ML Mobility Optimization in NR-DC, UE throughput UL/DL at PDCP level can be considered.

For NR-DC scenarios, we think that the full PDU session may not be terminated at a single node. It is possible that some DRBs of the PDU session are terminated in MN PDCP and some in SN PDCP. Therefore, we think that it is necessary to measure UE Performance metrics in NR-DC per DRB.

Proposal 3: For AI/ML Mobility Optimization in NR-DC report UE Performance metrics (UE throughput UL/DL and Average Packet Delay UL/DL) per DRB.

## 2.2 Assistance Information from MN to SN

One topic that was discussed in previous meetings was sending the predicted (or assumed) PSCell ID from MN to SN during Dual Connectivity. Next, we highlight the procedure with which a PSCell is selected. The SN selects the target PSCell among the reported cells by the UE, considering the radio quality, load of the reported cells, UE Capability, etc. The SN consequently further checks based on the band combination, the feature set combination to select one combination that maximizes the throughput. Thus, the PSCell selection is up to target SN and its internal logic.

Observation 1: PSCell selection is up to target SN and its internal logic.

The MN can get a list of visited PSCells for a UE from SCG UE History Information IE. However, it cannot know which were the reasons that led SN to choose those PSCells. For example, when establishing NR-DC for two different UE(s), SN can choose two different PSCell(s) because the actual selection algorithm in SN considers their bearer characteristics and UE Capabilities (e.g., supported band combinations) amongst other considerations (e.g. load). Thus, MN cannot accurately predict a PSCell since it is missing input on the internal logic that led SN into its PSCell decisions at each time.

Observation 2: An MN attempt to predict the PSCell ID chosen by the SN without considering the SN internal logic that resulted in the PSCell selection will result in inaccurate PSCell predictions.

Any attempt to predict by the MN the SN PSCell selection process would mean that that MN needs to be provided along with SCG UE History Information, all the associated criteria taken into consideration by the SN including load and deployment aspects or other implementation specific details. This, makes it very complex if not impossible for the MN to predict or even assume the PSCell choices by the SN.

Observation 3: It is very complex if not impossible for the MN to predict (or assume) the PSCell choices by the SN.

For the above reasons, the predicted PSCell ID does not provide assistance information that can help optimize NR-DC decisions. One related discussed issue was whether to enhance the UE performance IE in the DATA COLLECTION UPDATE message with the actual PSCell ID. Similar to the arguments above, since PSCell is dependent on SN implementation the PSCell ID would not provide useful information to the MN that can help evaluate the decision of triggering DC with the SN.

Observation 4: Enhancing the UE performance IE in the DATA COLLECTION UPDATE message with the actual PSCell ID would not provide the MN useful information that could help it evaluate the DC decision.

## 2.3 The effects of SCG Activation and Deactivation on Data Collection

In TS 38.423, we have captured the following interaction between the Data collection Reporting and Data Collection Reporting Initiation procedures considering the S-Node Addition trigger for NR-DC scenario:

While there is only a single procedure controlling the handover (Handover Preparation procedure), in Dual Connectivity there exist two independent processes, namely S-NG-RAN node Addition and SCG Activation/Deactivation (see TS 37.340, Clause 7.13). SCG activation can be requested by the MN, by the SN, and by the UE. SCG deactivation can be requested by the MN and by the SN. For SN-terminated bearers, this allows to establish an endpoint for data transfer in the SN (or the gNB-CU-UP of the SN) immediately as part of QoS flow setup or incoming handover without necessarily requiring the UE to actively monitor the SCG (e.g., PDCCH decoding) in periods without data. Therefore, it is possible to indicate whether the SCG resources are required to be activated or deactivated when S-NODE ADDITION REQUEST is sent from Master Node to Secondary Node. SCG can also be independently activated or deactivated by the SN. Only when the SCG resources are activated, data may actually be transferred over the SCG and corresponding UE Performance measurements can be collected. We therefore believe that UE Performance measurements for bearers handled over the SCG leg should only be collected when SCG resources are activated. Some examples on scenarios are listed next:

- Case #1: MN can add a secondary node together with an active SCG resources using XnAP: S-NODE ADDITION REQUEST message with SCG Activation Request=activate SCG. In this case the UP data flow and UE performance data collection can start as soon as the Secondary Node addition procedure is successful.

- Case #2: MN can add a secondary node with SCG resources de-activated using XnAP: S-NODE ADDITION REQUEST message with SCG Activation Request=de-activate SCG. In this case the UP data flow using the SCG resources is not started and hence any UE Performance data such as Throughput or packet delay collected in the Secondary Node would provide 0 measurement results.

- Case #3: After a successful Secondary Node addition with de-activated SCG resources, MN can subsequently activate the SCG resources using S-Node Modification Request (w/SCG Activation Request=activate SCG). After this, UP data flow starts using SCG resources and the UE Performance data collected after the activation only is meaningful for any evaluation in the Master Node.

- Case #4: After a successful Secondary Node addition with de-activated SCG resources, SN can subsequently activate the SCG resources using S-Node Modification Required (w/SCG Activation Request). After this, UP data flow starts using SCG resources and the UE Performance data collected after the activation only is meaningful for any evaluation in the Master Node.

So, we see from the above examples that data collection should not start after successful S-NG-RAN node addition but only when SCG is activated. In the last meeting, there were some proposals indicating the need for additional conditions as triggers to start data collection such as SCG activation. Accordingly, an FFS was captured in Chair notes:

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

Another alternative would be to keep the conditions to start the data collection of UE Performance unchanged but enable the SN to handle such scenarios by skipping the corresponding UE measurements. This second alternative would allow handling of this issue without additional triggers. We would like to note here that this issue is present irrespectively of whether the measurements are made in PDCP layer or not. If the secondary node continues the reporting under a deactivated SCG, it can only report zero UE Performance since there is no UP traffic. In that case, the MN cannot interpret whether a reported zero measurement is due to the lack of traffic or due to bad channel conditions.

Observation 5: In case of inactive SCG bearer, UE Performance Reporting by the SN will result in useless measurements, e.g., measurements where the reported UE Performance is zero which cannot be interpreted by the MN, namely whether a zero measurement is due to the lack of traffic or due to bad channel conditions.

Furthermore, if the SN uses the measurements collected during deactivated SCG for the averaging together with other samples in a reporting period it will impact the accuracy of the reported measurements.

Observation 6: If the SN uses the measurements collected during deactivated SCG for the averaging together with other samples in a reporting period it will impact the accuracy of the reported measurements.

Consider the example in Figure 1 which illustrates a scenario where SN is added but SCG is deactivated during SN addition and is activated later during the second reporting period. If SN reports periodically at T1 and T2 then it can only report measurements with 0 values since there does not exist any UP Traffic while SCG is deactivated. When SCG is activated again reporting can resume. The measurement at T3 may still not have enough samples since SCG is only activated in the middle of the reported period. If the SN determines that it cannot have enough samples for an accurate measurement then it may skip also T3 measurement and resume reporting at T4, T5, …

Figure 1 Example of Data Collection with SCG Deactivation

Observation 7: Measurement reporting from SN to MN for a given UE in NR-DC can be skipped in one or more reporting periods taking into account the amount of useful samples in a reporting period, i.e., samples collected after SCG activation.

Proposal 4: Update the text in TS 38.423 as follows:

UE Performance measurements for a given UE in NR-DC need not be sent from SN to MN in one or more reporting period(s) in case of absence of measurements.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Proposal 1: In AI/ML Mobility Optimization in NR-DC, collect the Average Packet Delay at the gNB-CU-UP.

Proposal 2: We make the following WA: WA: In case of AI/ML Mobility Optimization in NR-DC, UE throughput UL/DL at PDCP level can be considered.

Proposal 3: For AI/ML Mobility Optimization in NR-DC report UE Performance metrics (UE throughput UL/DL and Average Packet Delay UL/DL) per DRB.

Observation 1: PSCell selection is up to target SN and its internal logic.

Observation 2: An MN attempt to predict the PSCell ID chosen by the SN without considering the SN internal logic that resulted in the PSCell selection will result in inaccurate PSCell predictions.

Observation 3: It is very complex if not impossible for the MN to predict (or assume) the PSCell choices by the SN.

Observation 4: Enhancing the UE performance IE in the DATA COLLECTION UPDATE message with the actual PSCell ID would not provide the MN useful information that could help it evaluate the DC decision.

Observation 5: In case of inactive SCG bearer, UE Performance Reporting by the SN will result in useless measurements, e.g., measurements where the reported UE Performance is zero which cannot be interpreted by the MN, namely whether a zero measurement is due to the lack of traffic or due to bad channel conditions.

Observation 6: If the SN uses the measurements collected during deactivated SCG for the averaging together with other samples in a reporting period it will impact the accuracy of the reported measurements.

Observation 7: Measurement reporting from SN to MN for a given UE in NR-DC can be skipped in one or more reporting periods taking into account the amount of useful samples in a reporting period, i.e., samples collected after SCG activation.

Proposal 4: Update the text in TS 38.423 as follows:

The result of UE Performance collection for a given UE in NR-DC need not be sent from SN to MN in one or more reporting period(s) in case of absence of measurements.

We provide a TP to TS 38.423 in Annex A.

# Annex A			  TP for BL CR to TS 38.423

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Skipped >>>>>>>>>>>>>>>>>>>>

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	the time since S-NG-RAN node addition successfully completed is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell;

-	the NR-DC with the S-NG-RAN node for the UE is released.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message. The result of UE Performance collection for a given UE in NR-DC need not be sent from SN to MN in one or more reporting period(s) in case of absence of measurements.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>