3GPP TSG-RAN WG3 Meeting #128	R3-253590

St Julian’s, Malta, 19 – 23 May, 2025

Agenda item:	11.4

Source:	Nokia, Qualcomm

Title:	Replying to LS from SA5 on Continuous MDT

Document for:	Decision

# 1	Introduction

RAN3 sent an LS to SA5 in R3-250891 to ask SA5 to work a solution for Continuous MDT. In this meeting RAN3 received an incoming LS from SA5 that includes some additional questions for clarification from RAN3. Specifically, SA5 asked the following:

- For Measurement Continuity Problem:

First aspect from RAN3: “- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.”

Answer to Aspect 1 from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

Second aspect from RAN3: “- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.”

Answer to Aspect 2 from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

Third Aspect from RAN3: “- A solution should minimise impacts on the current Management Based MDT framework.”

Answer to Aspect 3 from SA5: SA5 agrees that the solution should minimise impacts on the current Management Based MDT framework.

- For Trace Correlation Problem:

Fourth Aspect from RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.”

Answer to Aspect 4 from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

In this contribution, we provide our views and additional answers to clarify the Continuous MDT topic to SA5.

# 2	Discussion

## 2.1 Configuration details as OAM Requirements

SA5 provided the following answer to RAN3:

Answer to Q1 from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

In this section we provide our views regarding configuration details as OAM requirements.

Continuous MDT corresponds to Logged MDT and Immediate MDT measurements from a given UE. Therefore, for Continuous MDT, a Logged MDT and an Immediate MDT configuration need to be linked so that they together define a Continuous MDT session. We think that there are two main categories of how to link these MDT configurations:

- OAM decides which configurations are linked

Linking MDT configurations in the OAM system can be done in different ways. One way is that the OAM combines the Logged and Immediate MDT configurations into a single configuration and allocates a single Trace Reference (TR) for this combination. Thus, it is always obvious to a gNB receiving a Continuous MDT Configuration from OAM which Immediate and Logged MDT Configurations correspond to a Continuous MDT session. We see some drawbacks with this solution:

- OAM does not know details regarding the gNB that will receive the configuration or the UE that will be configured subsequently by the gNB.

- Configuration flexibility is missing  since the pair of Logged and Immediate MDT configurations needs to be defined in advance by the OAM system.

- There is a security concern since a single Trace Reference points to Logged and Immediate MDT across RRC transitions and across connected mode mobility of a given UE. This would allow RAN to track a UE for a prolonged period of time.

Observation 1: We see the following issues if OAM allocates a single TR for both Immediate and Logged MDT Configurations:

- OAM does not know details regarding the gNB that will receive the configuration or the UE that will be configured subsequently by the gNB.

- The solution does not provide configuration flexibility since the pair of Logged and Immediate MDT configurations needs to be defined in advance by the OAM system.

- There is a security concern since a single Trace Reference points to Logged and Immediate MDT, and, combined with the allocation of a single TRSR by the gNB, can be used for tracking the UE across RRC transitions and across connected mode mobility.

Another way for OAM to link a Logged MDT and an Immediate MDT Configuration is to flag a TR corresponding to Logged MDT Configuration and a TR corresponding to Immediate MDT Configuration that they are for Continuous MDT. If in a given area of gNBs only a single Logged MDT TR and a single Immediate MDT TR corresponding to Continuous MDT are allowed, then this simple indication will be sufficient to link the two MDT configurations. A gNB receiving these two TRs can determine that it needs to select a UE for both of these TRs for Continuous MDT.

Observation 2: If OAM allocates one TR for Continuous Logged MDT and one TR for Continuous Immediate MDT we don’t see any concern regarding tracking a given UE, but the following issues remain:

- OAM does not know details regarding the gNB that will receive the configuration or the UE that will be configured subsequently by the gNB.

- Does not have configuration flexibility since:

  - Only a single Logged and Immediate MDT Configuration can be configured to a given gNB at any given time

  - The pair of Logged and Immediate MDT configurations needs to be defined in advance by the OAM system.

A third alternative in which the OAM creates the link between a Logged and an Immediate MDT configuration is by letting each MDT Configuration for Continuous MDT (Logged or Immediate MDT) point to a corresponding MDT Configuration (Immediate or Logged MDT) with which it needs to be linked for the purpose of continuity. This means that the MDT Configuration for Continuous MDT would include two TRs, the TR of the configuration to be applied to a UE (Logged or Immediate MDT) and the TR of the configuration with which the configured MDT Configuration needs to be linked. We think that this approach is more flexible since it allows multiple Continuous MDT sessions to be configured to a given gNB, following also legacy management-based MDT framework.

Observation 3:  It is more flexible to introduce two TRs in a given Continuous MDT Configuration (Logged or Immediate MDT), one TR pointing to the configuration to be applied to a UE and one TR pointing to the corresponding Continuous MDT Configuration that needs to be linked (Immediate or Logged MDT) during Continuous MDT since it allows multiple Continuous MDT sessions to be configured to a given gNB, following legacy management-based MDT methods.

However, we still think that creating the link between Immediate and Logged MDT for Continuous MDT by the OAM has the drawback that the configuration for MDT Continuity is selected in advance by the OAM system and OAM does not know the gNB that will receive the configuration. Even more importantly the OAM cannot take into consideration the UE to be configured subsequently by the gNB.

Observation 4: Introducing two TRs in a given Continuous MDT Configuration for Logged or Immediate MDT still has the following issues:

- OAM does not know details regarding the gNB that will receive the configuration or the UE that will be configured subsequently by the gNB.

- The solution does not provide configuration flexibility since the pair of Logged and Immediate MDT configurations needs to be defined in advance by the OAM system.

- gNB creates the link between Logged and Immediate MDT for Continuous MDT

We think that linking the MDT configurations by the gNB has merits. First, it allows OAM to configure multiple Continuous MDT Configurations for Logged or for Immediate MDT to a gNB. The gNB, can determine which Continuous Logged MDT configuration to correlate with which Continuous Immediate MDT Configuration. In this way, the gNB is enabled to select the proper Continuous MDT configurations, taking also into account the UE selected for Continuous MDT and their existing RRC Configurations, which would follow legacy MDT paradigm. E.g. the gNB can select optimal immediate MDT configurations for available UEs based on their RRC Configurations and combine it with the corresponding OAM-defined logged MDT configuration for continuous MDT.

Proposal 1: RAN3 includes as part of OAM requirement: OAM configuration enables the gNB to select the optimal immediate MDT configuration for available UEs, e.g. by taking into account UE capabilities and combine it with the corresponding OAM-defined logged MDT configuration for continuous MDT.

A possible solution allowing this enhanced flexibility could correspond to mapping a given logged MDT configuration to multiple immediate MDT configurations as follows:

- OAM provisions immediate MDT configurations for continuous MDT each of which includes its own TR (for the immediate MDT configuration) and additionally includes the TR of the logged MDT configuration to be linked with.

- The logged MDT configuration would include only a single TR (its own).

This solution seems to be simple while it also provides configuration flexibility since it allows a gNB to take into account the current RRC configuration of different UEs and use it for UE selection. We, therefore, propose that RAN3 includes it as part of OAM requirements.

Proposal 2: RAN3 includes as part of OAM requirements: OAM provisions immediate MDT configurations for continuous MDT each of which includes its own TR (for the immediate MDT configuration) and additionally the includes the TR of the corresponding logged MDT configuration to which it is linked.

When it comes to reporting of the continuous MDT measurements, the NG-RAN would then report logged MDT reports to the TCE as per today (using TR/TRSR for logged MDT). Trace records for immediate MDT would include both the immediate MDT TR/TRSR (in order to identify the immediate MDT configuration) as well as the logged MDT TR/TRSR (for correlation).

Proposal 3: RAN3 includes as part of OAM requirements: Trace records for immediate MDT include both the immediate MDT TR/TRSR (in order to identify the immediate MDT configuration) as well as the logged MDT TR/TRSR (for correlation).

A gNB could know which configurations are for Continuous MDT through the allocated range of TRs for example or through another flag or attribute introduced by OAM. This aspect is up to SA5 to determine.

Proposal 4: RAN3 includes as part of OAM requirements: The OAM configuration enables the gNB to determine that an MDT Configuration (Logged or Immediate MDT) is for Continuous MDT.

In our view, another important configuration requirement relates to determining the allowed Area Scope for Continuous MDT, namely the area in which a given Continuous MDT session may collect data. In management-based logged MDT the area scope may be very large and may span the whole PLMN. This mechanism has the important advantage of enabling MDT measurement collection from any idle mode UE without any bias linked to the UE mobility pattern. Absence of such bias is of course an important element also for Data Collection for AI/ML Training and therefore, also for continuous MDT measurement collection.

Proposal 5: RAN3 includes as part of OAM requirements: Continuous MDT uses the same Area Scope parameter definition as legacy MDT, and in case of absence of Area Scope parameter the Continuous MDT session is applicable within the entire PLMN.

However, the legacy MDT framework doesn’t require the OAM system to provision management-based logged MDT configuration in all gNBs that serve cells located within the Area Scope (e.g. the entire PLMN). In this framework different gNBs configured by OAM independently select UEs for management-based MDT. If the same configuration area and area scope is assumed for management-based MDT and for Continuous MDT the number of selected UEs may be extremely high. Given that Continuous MDT brings additional burden to a selected UE to provide measurements across RRC states, it is in our view necessary to limit configuring of an unnecessary number of UEs for continuous MDT by careful selection of the configuration area where Continuous MDT is configured. Besides, it is reasonable to assume that OAM would configure Continuous MDT over a limited area in order to collect training data or inference data pertaining to a specific area where an AI/ML Model needs to be trained.

Proposal 6: RAN3 includes as part of OAM requirements: OAM determines a Configuration Area for Continuous MDT to be an area within which NG-RAN nodes will select UEs for a given Continuous MDT job. The Continuous MDT solution should enable that the Configuration Area is possibly smaller than the area represented by the Area Scope.

## 2.2 Clarifications on the Measurement Continuity Problem across different RRC states

SA5 provided the following answer to RAN3:

Answer to Aspect 2 from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

The definition on continuous MDT provided by RAN3 to SA5 in R3-250891 is the following: “Continuous MDT is a functionality under discussion in RAN3 that consists of using Management Based MDT for collection of MDT measurements from the same UE while it moves between different NG-RAN nodes and while it changes RRC states.” In this section, we would like to provide some clarifications since the definition related to the measurement continuity problem may not be sufficiently clear. Specifically, we think that the following clarifications need to be provided to SA5:

- Measurements for Continuous MDT

It may be unclear to SA5 which are the measurements for which MDT Continuity is considered since Connected Mode measurements are by nature different from measurements performed in Idle Mode (Logged MDT measurements). The original use case on Continuous MDT by RAN3 was related to obtaining a continuous set of data (for a given measurement) across RRC transitions. Examples on UE location information and RSRP measurements in RRC Connected and RRC Idle state were given in the RAN3 discussions. However, in the LS that RAN3 sent to SA5 no indication on the measurements was provided in the Measurement Continuity problem. In our view, Continuous MDT does not make sense for MDT Configurations where a UE reports totally distinct measurements in RRC Idle and RRC Connected states (e.g., Average Delay measurement is provided in RRC Connected state and RSRP measurement is provided in RRC Idle state). Thus, Continuous MDT problem is relevant for MDT Configurations where there exists at least one UE measurement that is common across RRC Idle/RRC Inactive and RRC Connected states (e.g., RSRP).

Observation 5: Continuous MDT problem is only relevant for MDT Configurations where there exists at least one UE measurement that is common across RRC Idle/RRC Inactive and RRC Connected states (e.g., RSRP).

- Reporting Configuration and Triggers

Another aspect that needs to be clarified towards SA5 is the reporting or logging configuration that may be associated with different measurements. A given Immediate MDT measurement that is reported periodically can be reported with different periods (Report Interval) e.g., 120ms, 240ms, 640ms, etc. Similarly, a Logged MDT Configuration for the same measurement may have different logging durations or logging intervals supported. RAN3 never discussed whether the reporting or logging intervals across Immediate and Logged MDT Configurations need to be identical. In our view, all configurations with the same measurement but with different reporting or logging intervals shall be considered equivalent with respect to Continuous MDT.

Proposal 7: MDT configurations corresponding to the same measurement having possibly different reporting or logging parameters are equivalent with respect to Continuous MDT.

- Measurement Continuity Requirement

When continuous management-based MDT is configured to a gNB, the gNB will need to select the UE(s) that better match the received measurement configuration from OAM. In legacy management-based MDT a gNB selects UEs for immediate MDT and for logged MDT separately, after checking gNB capabilities and UE capabilities. In particular, a gNB receiving management-based Trace Activation from OAM verifies whether it supports the Trace function and configuration (all or part of) parameters, and accordingly, if supported, initiates suitable device(s) selection, based on their capabilities, by e.g., determining support of the requested measurements Mx. Selecting the UEs in this way allows OAM to receive “enough” measurements from different UEs, e.g., from UEs that have logging capability (which can provide logged MDT reports) and from UEs that don’t (and which may provide immediate MDT).

When it comes to continuous MDT the requirement is that the “same” UE provides both logged and immediate MDT measurements which means that continuous MDT is only applicable to UEs that have logging capability. In addition, if those UEs must provide all the measurements indicated in the MDT configuration this will possibly lead to a very limited set of measurement-providing UEs, which would subsequently be a source of bias in the data collection and limit usefulness of the continuous MDT feature. Therefore, it seems that in order to maintain a sufficient number of UEs providing measurements we should not mandate a UE to provide all measurements indicated in the Trace Activation. It should therefore be possible for the OAM system to indicate in the MDT configuration which measurements that are mandatory, and which measurements that can be performed optionally by the UE.

Proposal 8: In order to limit the Continuous MDT impacts to the UE and enable a wider applicability of the feature, in Continuous MDT a UE should not be mandated to provide all the measurements indicated in the Trace Reference to a gNB.

Proposal 9: RAN3 provides the following OAM requirement: The OAM system indicates in the MDT configuration which measurements are mandatory and which measurements can be performed optionally by the UE.

## 2.3 Minimizing impacts on the current Management-based MDT framework

A third aspect from RAN3 was related to minimizing impacts on the current Management-based MDT framework to which SA5 agreed:

Answer to Aspect 3 from SA5: SA5 agrees that the solution should minimise impacts on the current Management Based MDT framework.

Besides the impacts to the current Management-based MDT framework we think that Continuous MDT should also limit the impacts towards a UE, induced from unnecessary reporting or logging. This can be reflected also through our proposal 10.

Proposal 10: The Continuous MDT solution minimizes the impacts to the UE by limiting reporting from the UE to the NG-RAN when feasible.

## 2.4 Usage of TR and TRSR for the Trace Correlation problem

A fourth aspect relates to the Trace Correlation Problem, to enable correlation of MDT measurements generated by the same UE.

Answer to Aspect 4 from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

SA5 agreed to use at least TR and TRSR (which does not reveal UE identity) for the trace correlation problem. However, as also SA5 identified there is indeed an issue if we were to identify a UE through the TR and TRSR pair in that the TRSR may be reused among different gNBs for the same MDT job, and overlapping could occur. In our view, this issue can be resolved based on the Cell Traffic Trace functionality. A gNB sends Cell Traffic Trace to an AMF when it selects a UE for MDT, informing AMF of the allocated TR and TRSR. This means that the AMF knows the pair of TR and TRSR allocated by the gNB for a given MDT session to a given UE. In legacy MDT framework, the AMF forwards to the TCE the TR and TRSR pair, along with (optionally) the IMEI-TAC of the UE, depending on the anonymization level, which are subsequently combined by TCE together with the MDT records received from different gNBs.

Observation 6: In legacy MDT framework, Cell Traffic Trace mechanism is used to allow the TCE to determine that trace records originate from the same UE.

For Continuous MDT, AMF could create a UE identifier, not revealing UE identity, from a permanent UE identifier and send it to TCE for the Trace Correlation instead of IMEI-TAC. In this way, Trace Correlation could be performed by re-using legacy mechanisms where instead of IMEI-TAC a different correlation identifier is created by AMF.

Proposal 11: AMF creates a UE identifier, not revealing UE identity, from a permanent UE identifier and sends it to TCE for the Trace Correlation.

We also  provide a reply LS to SA5 based on the above in the Annex of this paper.

# 3	Conclusion

In this paper we make the following observations and proposals:

Observation 1: We see the following issues if OAM allocates a single TR for both Immediate and Logged MDT Configurations:

- OAM does not know details regarding the gNB that will receive the configuration or the UE that will be configured subsequently by the gNB.

- The solution does not provide configuration flexibility since the pair of Logged and Immediate MDT configurations needs to be defined in advance by the OAM system.

- There is a security concern since a single Trace Reference points to Logged and Immediate MDT, and, combined with the allocation of a single TRSR by the gNB, can be used for tracking the UE across RRC transitions and across connected mode mobility.

Observation 2: If OAM allocates one TR for Continuous Logged MDT and one TR for Continuous Immediate MDT we don’t see any concern regarding tracking a given UE, but the following issues remain:

- OAM does not know details regarding the gNB that will receive the configuration or the UE that will be configured subsequently by the gNB.

- Does not have configuration flexibility since:

  - Only a single Logged and Immediate MDT Configuration can be configured to a given gNB at any given time

  - The pair of Logged and Immediate MDT configurations needs to be defined in advance by the OAM system.

Observation 3:  It is more flexible to introduce two TRs in a given Continuous MDT Configuration (Logged or Immediate MDT), one TR pointing to the configuration to be applied to a UE and one TR pointing to the corresponding Continuous MDT Configuration that needs to be linked (Immediate or Logged MDT) during Continuous MDT since it allows multiple Continuous MDT sessions to be configured to a given gNB, following legacy management-based MDT methods.

Observation 4: Introducing two TRs in a given Continuous MDT Configuration for Logged or Immediate MDT still has the following issues:

- OAM does not know details regarding the gNB that will receive the configuration or the UE that will be configured subsequently by the gNB.

- The solution does not provide configuration flexibility since the pair of Logged and Immediate MDT configurations needs to be defined in advance by the OAM system.

Proposal 1: RAN3 includes as part of OAM requirement: OAM configuration enables the gNB to select the optimal immediate MDT configuration for available UEs, e.g. by taking into account UE capabilities and combine it with the corresponding OAM-defined logged MDT configuration for continuous MDT.

RAN3 includes as part of OAM requirements: OAM provisions immediate MDT configurations for continuous MDT each of which includes its own TR (for the immediate MDT configuration) and additionally the includes the TR of the corresponding logged MDT configuration to which it is linked.

Proposal 3: RAN3 includes as part of OAM requirements: Trace records for immediate MDT include both the immediate MDT TR/TRSR (in order to identify the immediate MDT configuration) as well as the logged MDT TR/TRSR (for correlation).

Proposal 4: RAN3 includes as part of OAM requirements: The OAM configuration enables the gNB to determine that an MDT Configuration (Logged or Immediate MDT) is for Continuous MDT.

Proposal 5: RAN3 includes as part of OAM requirements: Continuous MDT uses the same Area Scope parameter definition as legacy MDT, and in case of absence of Area Scope parameter the Continuous MDT session is applicable within the entire PLMN.

Proposal 6: RAN3 includes as part of OAM requirements: OAM determines a Configuration Area for Continuous MDT to be an area within which NG-RAN nodes will select UEs for a given Continuous MDT job. The Continuous MDT solution should enable that the Configuration Area is possibly smaller than the area represented by the Area Scope.

Observation 5: Continuous MDT problem is only relevant for MDT Configurations where there exists at least one UE measurement that is common across RRC Idle/RRC Inactive and RRC Connected states (e.g., RSRP).

Proposal 7: MDT configurations corresponding to the same measurement having possibly different reporting or logging parameters are equivalent with respect to Continuous MDT.

Proposal 8: In order to limit the Continuous MDT impacts to the UE and enable a wider applicability of the feature, in Continuous MDT a UE should not be mandated to provide all the measurements indicated in the Trace Reference to a gNB.

Proposal 9: RAN3 provides the following OAM requirement: The OAM system indicates in the MDT configuration which measurements are mandatory and which measurements can be performed optionally by the UE.

Proposal 10: The Continuous MDT solution minimizes the impacts to the UE by limiting reporting from the UE to the NG-RAN when feasible.

Observation 6: In legacy MDT framework, Cell Traffic Trace mechanism is used to allow the TCE to determine that trace records originate from the same UE.

Proposal 11: AMF creates a UE identifier, not revealing UE identity, from a permanent UE identifier and sends it to TCE for the Trace Correlation.