3GPP TSG-RAN WG3 Meeting #128	R3-253592

St Julian’s, Malta, 19 – 23 May, 2025

Agenda item:	20.2

Source:	Nokia, CATT, Xiaomi, Ericsson, CMCC

Title:	(TP to TS 38.300) Stage 2 for case 3a

Document for:	Text Proposal

# 1	Introduction

In this contribution, we provide our views with respect to general principles that we see applicable to case 3a and provide a corresponding TP for BL CR to TS 38.300 in the Annex of this paper.

# 2	Discussion

## 2.1	General Principles

RAN3 has agreed that for case 3a AI/ML Model Training and AI/ML Model Inference functions are located at the gNB.

Case 3a assumes the existence of an AI/ML Model in the gNB. We, therefore, see useful to agree for this case some high-level principles from TR 37.817 and from the AI/ML for NG-RAN work, that were considered valid during Rel-18 AI/ML for NG-RAN normative phase and which are also used for the ongoing Rel-19 work.

Specifically, even though in case 3a we provide support so that a gNB may train an AI/ML Model, in our view, the assumption in AI/ML for NG-RAN work that AI/ML Models and Algorithms are not in scope of 3GPP shall remain applicable.

Proposal 1: For case 3a, details of AI/ML Models and Algorithms are implementation specific and out of scope of 3GPP.

Another principle that we followed in Rel-18 NG-RAN AI/ML work is that an AI/ML Model used for Model Inference needs to have completed all the steps of Life Cycle Management, meaning AI/ML Model Training, AI/ML Model Validation and AI/ML Model Testing. Since in case 3a, a gNB provides an LMF with inferred measurements we think that the same principle is valid.

Proposal 2: For case 3a, an AI/ML model used for Model Inference must be initially trained, validated and tested before deployment.

Finally, another fundamental principle for Data Collection is related to respecting user data privacy and anonymization, which in our view shall be valid in this context also.

Proposal 3: For case 3a, user data privacy and anonymization need to be respected during AI/ML operation.

In RAN3 #126 we confirmed a working assumption that NG-RAN node performs monitoring metric calculation for its own model. Note that Model monitoring in TR 38.843 corresponds to Model Performance monitoring in RAN3 NG-RAN AI/ML discussions, which is the process enabling the monitoring of the performance of an AI/ML model. The outcome of Model Performance monitoring is provided by the Model Inference to the Model Training entity through Model Performance Feedback.

Proposal 4: Use the term Model Performance monitoring to refer to Model monitoring for better alignment with RAN3 terminology.

RAN3 has taken the agreement that an NG-RAN node performs monitoring metric calculation for its own model, but there is still an open issue to address related to whether the NG-RAN node also monitors its own Model Performance or whether it sends its calculated Model Performance metrics to an LMF to enable Model Performance monitoring at the LMF. Model Performance monitoring needs implementation-specific performance monitoring metrics and should therefore take place at a node knowing the “details” of the model, namely the model output, the ground truth labels and the Model Performance monitoring metrics. In AI/ML for NG-RAN we had agreed to an important principle that model-specific performance information is implementation-specific and is not exchanged over NG-RAN interfaces. In our view, this is another important principle that shall remain valid in this context as well.

Proposal 5: For case 3a, model-specific performance information is implementation-specific and is not exchanged over NG-RAN interfaces.

A design principle for the AI/ML for NG-RAN work has been that the procedures used for the support of AI/ML for NG-RAN need to be use case and data type agnostic, namely it should not be indicated whether the requested information is to be used for input, output or feedback. We believe that this principle is applicable also for AI/ML Positioning case 3a.

Proposal 6: For case 3a, signalling procedures used for the exchange of information between a gNB and an LMF are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

# 3	Conclusion

In this paper we make the following proposals:

Proposal 1: For case 3a, details of AI/ML Models and Algorithms are implementation specific and out of scope of 3GPP.

Proposal 2: For case 3a, an AI/ML model used for Model Inference must be initially trained, validated and tested before deployment.

Proposal 3: For case 3a, user data privacy and anonymization need to be respected during AI/ML operation.

Proposal 4: Use the term Model Performance monitoring to refer to Model monitoring for better alignment with RAN3 terminology.

Proposal 5: For case 3a, model-specific performance information is implementation-specific and is not exchanged over NG-RAN interfaces.

Proposal 6: For case 3a, signalling procedures used for the exchange of information between a gNB and an LMF are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

In case 3a, an AI/ML Model is in the gNB, so that a gNB provides intermediate features related to measurements to the LMF. According to our proposals 1 - 6, the framework handling this AI/ML model at the gNB for case 3a shares the same principles and assumptions as the framework for AI/ML for NG-RAN use cases, already described in TS 38.300, clause 16.20.2.

RAN2 in their running CR to TS 38.300 has introduced a section X.Y.4 related to AI/ML based Positioning. We think that  proposals 1 -7 can be reflected in stage 2 by stating in section X.Y.4 of TS 38.300 that for AI/ML assisted positioning with gNB-side AI/ML model the general principles described in clause 16.20.2 apply. This is shown in Annex A.

Proposal 7: Endorse the TP to BL CR for TS 38.300 provided in Annex A.

# Annex A -TP to TS 38.300

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

-----------------------------------------------------------Skip Unchanged-----------------------------------------------------------

## X.Y	Support of AI/ML for NR Air Interface

### X.Y.1	Overview

The objective of AI/ML for NR air interface is to improve network performance and user experience, through AI/ML-enabled enhancements to the following features: beam management, CSI prediction and positioning accuracy.

-----------------------------------------------------------Skip Unchanged-----------------------------------------------------------

### X.Y.4	AI/ML-based Positioning

AI/ML-based Positioning is described in TS 38.305 [42].

For the deployment of AI/ML assisted positioning with gNB-side AI/ML model, AI/ML Model Training and AI/ML Model Inference are both located in the gNB node. The other principles for support of AI/ML for NG-RAN described in clause 16.20.2 apply for AI/ML assisted positioning with gNB-side AI/ML model.

Editor’s note:	AI/ML Model Training in OAM for AI/ML assisted positioning with gNB-side AI/ML model is FFS. Text can be further improved, if needed.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>