3GPP TSG-RAN WG3 Meeting #128	R3-253599

Malta, MT, 19 – 23 May, 2025

Title:	(TP for LTM BLCR for TS38.473, TS38.300): Further discussion on inter-CU LTM procedure

Agenda Item:	13.2

Source:	Huawei

Document for:	Other

1	Introduction

In this contribution, we continue to discuss the basic procedures of inter-CU LTM and focus on the standalone case.

2	Discussion

2.1	Security key update

In subsequent inter-CU LTM, how to update the security key is one issue to be considered when UE switches among candidate cells. After several meetings discussion in both RAN2 and SA3, SA3 replied a LS in [1] to inform RAN2 and RAN3 that the NCC is transmitted via MAC CE. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide.

RAN3 made the following agreements related to the key distribution procedure:

RAN3 #127bis For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB. The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure. Whether to define additional information from CU to DU to deliver NCC value.

For the initial LTM, the key distribution from the serving gNB to other candidate gNBs is per cell, since it re-uses the existing Handover Request message. However, for the subsequent LTM, the new procedure LTM configuration update is used. RAN3 is discussing whether this new procedure is per cell or per gNB. Considering that one candidate gNB may provide several candidate cells, if one single message can include all the security information (e.g., the new keys of all candidate cells in a candidate gNB), the signalling overhead can be reduced.

Proposal 1: The LTM configuration update procedure is designed to transfer the new security keys in a per gNB basis, i.e., including the security information of all candidate cells in a candidate gNB.

In RAN2 #129bis meeting, RAN2 agreed that the NCC is sent to the UE via the LTM cell switch command MAC CE, and the NCC is only included when this cell switch required key update.

RAN2 #129bis NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

Since the LTM cell switch command MAC CE is triggered by the serving gNB-DU, this DU needs to know whether the current cell switch requires NCC or not. Therefore, the "Rel-19 set ID" (ltm-NoSecurityChangeID) of each candidate cells should be sent to the serving gNB-DU, and sent to other candidate gNB-DUs for subsequent LTM.

A similar stage 3 design for the LTM Reset Information IE can be reused. The Rel-19 Set ID information needs to send to the source gNB-DU and other candidate gNB-DUs via the UE Context Modification Request message over F1.

On Xn, this new IE can be transferred to the candidate gNBs via LTM Configuration Update message.

9.3.1.346 LTM Reset Information This IE contains the L2 reset configuration for the serving cell and LTM candidate cell(s). IE/Group Name; Presence; Range; IE type and reference; Semantics description. Serving Cell L2 Reset Configuration; O; OCTET STRING; Includes the ltm-ServingCellNoResetID contained in the LTM-Config IE, as defined in TS38.331 [8], for the current serving cell.. LTM L2 Reset Configuration List; 0..1. >LTM L2 Reset Configuration Item; 1.. <maxnoofLTMCells>. >>Cell ID; M; NR CGI 9.3.1.12. >>LTM L2 Reset Configuration; M; OCTET STRING; Includes the ltm-NoResetID IE as defined in TS38.331 [8], for the LTM candidate cell identified by the Cell ID IE..

Proposal 2a: To introduce a new IE to transfer the Rel-19 Set ID information to the source DU and candidate DU in the UE CONTEXT MODIFICATION REQEUST message on F1AP and to the candidate gNB  in the LTM CONFIGURATION UPDATE message on XnAP.

In the incoming LS from RAN2 in [3], it states that:

The term “R19 set ID” refers to the “LTM no security change ID” configured to the UE for serving cell and candidate cells separately. RAN2 assumes that even for intra-CU LTM, the network is allowed to configure different R19 set IDs for different cells (including serving and/or intra-CU candidate cells) if the network wants to trigger security key update. From UE perspective, security key update is performed when the R19 set IDs are different between the serving cell and the target cell, whether it is intra-CU or inter-CU LTM can be transparent to the UE.

The above content in the LS implies further network signalling requirements to support the security key update for intra-CU cells. A simple way is to allow the candidate CU to feedback the list of candidate cells that needs security key change to the source CU. And then the source CU may adjust the Rel-19 set ID assignment for the candidate cells and sync up them to all other candidate CUs via LTM Configuration Update message.

Proposal 2b: The candidate CU feedback the list of candidate cells that needs security key change to the source CU in HANDOVER REQEUST ACKNOLEDGE message.

2.2	Semi-persistent CSI-RS measurement

In last RAN3 meeting, RAN3 had the following agreements to support semi-persistent (SP) CSI-RS measurement.

RAN3 #127bis Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s). Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation. NEC: concern on the timing issue if reusing the existing XnAP message. Open issues on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2 progress.

RAN3 already agreed to introduce new Xn and F1 class 1 procedures to perform the SP CSI-RS activation and deactivation. The open issue is the granularity of the CSI-RS resource/resource set. According to the RAN1 and RAN2 progress, one SP CSI-RS resource set may include the CSI-RS resources from different candidate cells, and these candidate cells may be from different candidate gNB-CUs or different candidate gNB-DUs.

RRC Running CR in RAN2 #129bis email discussion LTM-NZP-CSI-RS-ResourceSet-r19 ::= SEQUENCE { ltm-CSI-RS-ResourceList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF NZP-CSI-RS-ResourceId, ltm-CandidateIdList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF LTM-CandidateId-r18, ... }

In Uu interface, one single MAC CE may activate SP CSI-RS measurement for multiple candidate cells. However, from RAN3 perspective, more than one F1/Xn signalling may be needed, because these candidate cells are from different gNB-DUs/gNB-CUs. Therefore, for the new introduced F1 messages (DU-CU CSI-RS COORDINATION REQUEST/CU-DU CSI-RS COORDINATION REQUEST) and Xn messages (CSI-RS COORDINATION REQUEST), CSI-RS resource IDs and the corresponding candidate configuration IDs should be added.

Proposal 3: To add the CSI-RS resource IDs and the corresponding configuration IDs in the CSI-RS Coordination related messages.

As for how the source gNB-DU can select suitable SP CSI-RS resource set IDs, the straightforward method is to let the source gNB-DU know the SSB and CSI-RS association of the candidate cell. So that it can determine the suitable CSI-RS resources by the SSB measurement results.

In general, the source gNB-DU can obtain the SSB and CSI-RS association in the LTM preparation phase. The reason is that in current RRC spec, the TCI-StateId (qcl-InfoPeriodicCSI-RS) which can be used to deduce the related SSB in the NZP-CSI-RS Resource IE will be present only for periodic CSI-RS. For SP CSI-RS, the source gNB-DU cannot deduce the CSI-RS and SSB mapping relationship.

NZP-CSI-RS-Resource ::= SEQUENCE { nzp-CSI-RS-ResourceId NZP-CSI-RS-ResourceId, resourceMapping CSI-RS-ResourceMapping, powerControlOffset INTEGER (-8..15), powerControlOffsetSS ENUMERATED{db-3, db0, db3, db6} OPTIONAL, -- Need R scramblingID ScramblingId, periodicityAndOffset CSI-ResourcePeriodicityAndOffset OPTIONAL, -- Cond PeriodicOrSemiPersistent qcl-InfoPeriodicCSI-RS TCI-StateId OPTIONAL, -- Cond Periodic ..., ----omitted---- } qcl-InfoPeriodicCSI-RS For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS. Refers to the TCI-State or dl-OrJoint-TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList or in dl-OrJointTCI-StateList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resource belongs to (see TS 38.214 [19], clause 5.2.2.3.1). In case NZP-CSI-RS-Resources is received as part of an LTM-Candidate IE, it refers to the TCI state identifier in CandidateTCI-State and is defined in ltm-DL-OrJointTCI-StateToAddModList within the LTM-Candidate IE.. Periodic; The field is optionally present, Need M, for periodic NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise..

Proposal 4: In LTM preparation phase, the source gNB-CU obtains the candidate cell’s SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKKNOLEDGE message and send the mapping information to the source gNB-DU via the UE Context Modification Request message.

Furthermore, RAN3 agreed that the source gNB is able to request the candidate gNB to feedback the CSI-RS resource configuration during LTM preparation. This will impact F1AP, as usually, the CSI-RS resource configuration is generated by the gNB-DU.

Proposal 5: To update UE Context Setup and UE Context Modification procedures to enable the gNB-CU request CSI-RS resource from the gNB-DU.

Moreover, in the incoming LS from RAN1 in [2], it states that semi-persistence CSI-RS resource is introduced for CSI acquisition.

Similarly to the semi-persistent CSI-RS resource supported for candidate cell L1-RSRP measurement for gNB scheduled reporting, RAN1 has also agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 As captured in the agreement, RAN1 sees the spec impact in RAN2 to define the MAC CE. Also, RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case.

In consequence, a separate request for CSI-RS resource for CSI acquisition is needed in LTM preparation phase. In response to the request for CSI-RS resource from the source gNB, the candidate gNB/candidate gNB-DU needs to provide the reserved CSI-RS resource for CSI acquisition separately.

Proposal 6: In LTM preparation phase, the source gNB requests the candidate gNB/gNB-DU to provide the CSI-RS resource for CSI acquisition. In response to the request, the candidate gNB/candidate gNB-DU provides the CSI-RS resource for CSI acquisition separately.

When the candidate gNB-DU provides CSI-RS resource configuration, it may provide both periodic CSI-RS resources and SP CSI-RS resources. Which type the CSI-RS resources belong to should be indicated in both F1AP and XnAP, so the source gNB-DU can know whether it needs to send the SP CSI-RS activation signalling or not for the CSI-RS resources.

Proposal 7: When a candidate gNB-DU/gNB provides CSI-RS resources, it  indicates the resource type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

2.3	UE association setup

The following agreements were made at previous RAN3 meetings.

RAN3 #125bis Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS. RAN3 #126 To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch. RAN3 #127bis For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

RAN3 agreed that, in the LTM Configuration Update message, the old target UE XnAP ID that the candidate gNB allocated for the first source gNB should be included in the LTM Configuration Update message. As for how the serving gNB knows the old target UE XnAP ID, the previous source gNB can send the old target UE XnAP IDs to the new serving gNB via Cell Switch Notification message.

Proposal 8: In Cell Switch Notification message, the source gNB sends the previously obtained old target UE XnAP IDs to the new serving gNB.

Moreover, there are two possible understandings on how to interpret the “old target UE XnAP ID”. We think that it’s better to further clarify the exact meaning of this critical IE.

One understanding is that the old target UE XnAP ID is the target UE XnAP ID allocated by the candidate gNB at the initial LTM preparation which is used to identify the UE association between the initial source gNB and the candidate gNB. The other understanding is that it is the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch, e.g., when receiving the LTM Configuration Update Request message from the new source gNB.

We think that the first understanding may be problematic if the initial source gNB is not a candidate gNB anymore. As the UE association between the candidate gNB and the initial source gNB may be released by the candidate gNB if it detects that the initial source gNB is released. Therefore, the “old target UE XnAP ID” should refer to the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch. Below figure illustrates the issue in detail.

Proposal 9: To clarify that the “old target UE XnAP ID” is the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch e.g., when receiving the LTM Configuration Update Request message from the new source gNB.

2.4	Early data forwarding data discard

RAN3 agreed to support early data forwarding for subsequent inter-CU LTM. There is one proposal to discuss how to deal with the data forwarded by early data forwarding procedure.

In legacy handover, the data forwarded to candidate gNB in early data forwarding will be released after handover procedure is completed, e.g. by reception of HANDCOVER CANCEL message. But for subsequent LTM, the data forwarded to the candidate gNB except target gNB in early data forwarding will be useless after the precedent LTM procedure is completed. And in the subsequent LTM procedure, the new source gNB may perform early data forwarding to the candidate too. For the candidate gNB, it is unclear whether and when it should flush the data received in the precedent LTM procedure. If the data received in precedent LTM procedure is not flushed, they will bring additional buffer requirement to the candidate gNB. Proposal 5: RAN3 to discuss whether and when a candidate gNB flush UE data that is received in early data forwarding after a LTM procedure.

The candidate gNB should discard the data after it determines that the UE is switched to another candidate gNB. One straightforward method is to use the LTM CONFIGURATION UPDATE message to let the candidate gNB know that the serving gNB is changed. After receiving the LTM CONFIGURATION UPDATE message, the candidate gNB determines that the UE has switched to another gNB, and it may discard the buffered data received from the original source gNB.

Proposal 10: After receiving the LTM CONFIGURATION UPDATE message, the candidate gNB discards the buffered data received from the original source gNB.

3	Conclusion

This document proposes:

Security key update

Proposal 1: The LTM configuration update procedure is designed to transfer the new security keys in a per gNB basis, i.e., including the security information of all candidate cells in a candidate gNB.

Proposal 2a: To introduce a new IE to transfer the Rel-19 Set ID information to the source DU and candidate DU in the UE CONTEXT MODIFICATION REQEUST message on F1AP and to the candidate gNB  in the LTM CONFIGURATION UPDATE message on XnAP.

Proposal 2b: The candidate CU feedback the list of candidate cells that needs security key change to the source CU in HANDOVER REQEUST ACKNOLEDGE message.

Semi-persistent CSI-RS measurement

Proposal 3: To add the CSI-RS resource IDs and the corresponding configuration IDs in the CSI-RS Coordination related messages.

Proposal 4: In LTM preparation phase, the source gNB-CU obtains the candidate cell’s SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKKNOLEDGE message and send the mapping information to the source gNB-DU via the UE Context Modification Request message.

Proposal 5: To update UE Context Setup and UE Context Modification procedures to enable the gNB-CU request CSI-RS resource from the gNB-DU.

Proposal 6: In LTM preparation phase, the source gNB requests the candidate gNB/gNB-DU to provide the CSI-RS resource for CSI acquisition. In response to the request, the candidate gNB/candidate gNB-DU provides the CSI-RS resource for CSI acquisition separately.

Proposal 7: When a candidate gNB-DU/gNB provides CSI-RS resources, it indicates the resource type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

UE association setup

Proposal 8: In Cell Switch Notification message, the source gNB sends the previously obtained old target UE XnAP IDs to the new serving gNB.

Proposal 9: To clarify that the “old target UE XnAP ID” is the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch e.g., when receiving the LTM Configuration Update Request message from the new source gNB.

Proposal 10: After receiving the LTM CONFIGURATION UPDATE message, the candidate gNB discards the buffered data received from the original source gNB.

The related TPs are provided in the Annex.

4	Reference

- S3-251124, Reply LS on security handling for inter-CU LTM in non-DC cases, SA3.

- R1-2503079, LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells.

- R2-2503012, LS on RAN2 agreements for security key handling in inter-CU LTM.

Annex – Text Proposal for LTM BLCR for TS 38.473

/******************Start of changes*******************************/

8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

<skip unchanged part>

If the DL LBT Failure Information Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the gNB- CU requests collection of DL LBT failure information for the analysis of the MRO events of the UE specified in TS 38.300 [6], and act as specified in TS 38.401 [4].

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for the UE’s Ranging and Sidelink Positioning service.

If the Request for CSI-RS Resource Configuration IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, include the CSI-RS Resource Configuration IE in the UE CONTEXT SETUP RESPONSE message.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

/*********************Next change***********************/

8.3.4	UE Context Modification (gNB-CU initiated)

8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

<skip unchanged part>

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

If the LTM Security Information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store it and take it into account for supporting the UE’s AS security continuation during an inter-CU LTM cell switch and act as specified in TS 38.401 [4] and TS 38.321 [16].

If the Request for CSI-RS Resource Configuration IE is contained in the UE CONTEXT MODIFICATION message, the gNB-DU shall, if supported, include the CSI-RS Resource Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

/*********************Next change***********************

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| Request for CSI-RS Resource Configuration L1 measurement | O |  | ENUMERATED (true, …) |  | - |  |
| Request for CSI-RS Resource Configuration CSI acquisition | O |  | ENUMERATED (true, …) |  | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |  |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Path Addition Information | O |  | 9.3.1.296 | This IE contains either the Indirect Path Addition IE or the N3C Indirect Path Addition IE. | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

/*********************Next change***********************/

9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell. | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >L1 Execution Condition List | O |  | 9.3.1.XXX | The detailed definition of this IE is FFS. | - |  |
| >CSI-RS Resource Configuration L1 measurement | O |  | 9.3.1.x1 |  | - |  |
| >CSI-RS Resource Configuration CSI acquisition | O |  | 9.3.1.x1 |  | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

/******************Next change*******************************/

9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| Request for CSI Resource Configuration L1 measurement | O |  | ENUMERATED (true, …) |  | - |  |
| Request for CSI Resource Configuration CSI acquisition | O |  | ENUMERATED (true, …) |  | - |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |
| LTM Security Information | O |  | 9.3.1.XX |  | YES | reject |
| LTM No Security Change Information | O |  | 9.3.1.x2 |  | YES | reject |

/*********************Next change***********************/

9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >L1 Execution Condition List | O |  | 9.3.1.XXX | The detailed definition of this IE is FFS. | - |  |
| >CSI-RS Resource Configuration L1 measurement | O |  | 9.3.1.x1 |  | - |  |
| >CSI-RS Resource Configuration CSI acquisition | O |  | 9.3.1.x1 |  | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

/******************Next change*******************************/

9.3.1.294	LTM Configuration ID Mapping List

This IE indicates the list of LTM cells associated with its configuration IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Configuration ID Mapping Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >LTM No Security Change ID | O |  | OCTET STRING | Corresponds to the ltm-NoSecurityChangeID IE, as defined in TS 38.331 [8]. | - |  |

/******************Next change*******************************/

9.2.2.x1	DU-CU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-DU to request the gNB-CU e.g. to activate/deactivate the SP CSI-RS transmissions from specific cells. (Detail is FFS)

Direction: gNB-DU  gNB-CU

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  |
| gNB-CU UE F1AP ID |  | M |  |  |  | 9.3.1.4 |  |  |  |
| gNB-DU UE F1AP ID |  | M |  |  |  | 9.3.1.5 |  |  |  |
| CSI-RS to be Activated List |  |  |  | 0..1 |  |  |  |  |  |
| >CSI-RS to be Activatedtem IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID |  | M |  |  |  | NR CGI 9.3.1.12 |  |  |  |
| >>SP CSI-RS Resource ID | M |  | FFS |  |
| CSI-RS to be Deactivated List |  |  |  | 0..1 |  |  |  |  |  |
| >CSI-RS to be Deactivated Item IEs |  |  |  | 1 .. <maxnoofCellList> |  |  |  |  |  |
| >>Candidate Cell ID |  | M |  |  |  | NR CGI 9.3.1.12 |  |  |  |
| >>SP CSI-RS Resource ID | M |  | FFS |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellList | Maximum no. of Cells in which the SP CSI-RS is to be activated or deactivated, the maximum value is 8. |

9.2.2.x2	DU-CU CSI-RS COORDINATION RESPONSE

This message is sent by the gNB-CU e.g. to inform the gNB-DU about the SP CSI-RS transmissions activation/deactivation result.

Direction: gNB-CU  gNB-DU

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  |
| gNB-CU UE F1AP ID |  | M |  |  |  | 9.3.1.4 |  |  |  |
| gNB-DU UE F1AP ID |  | M |  |  |  | 9.3.1.5 |  |  |  |
| CSI-RS Activation Result List |  | 0..1 |  |  |
| > CSI-RS Activation Result Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| CSI-RS Deactivation List |  | 0..1 |  |  |
| >CSI-RS Deactivation Result Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellList | Maximum no. of Cells in which the SP CSI-RS is activated or deactivated, the maximum value is 8. |

9.2.2.y1	CU-DU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-CU e.g. to request the gNB-DU to activate/deactivate the SP CSI-RS transmissions from specific cells. (Detail is FFS)

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |
| CSI-RS to be Activated List |  | 0..1 |  |  |
| >CSI-RS to be Activatedtem IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>SP CSI-RS Resource ID | M |  | FFS |  |
| CSI-RS to be Deactivated List |  | 0..1 |  |  |
| >CSI-RS to be Deactivated Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>SP CSI-RS Resource ID | M |  | FFS |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellList | Maximum no. of Cells in which the SP CSI-RS is to be activated or deactivated, the maximum value is 8. |

9.2.2.y2	CU-DU CSI-RS COORDINATION RESPONSE

This message is sent by the gNB-DU e.g. to inform the gNB-CU about the SP CSI-RS transmissions activation/deactivation result. (Detail is FFS)

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |
| CSI-RS Activation Result List |  | 0..1 |  |  |
| > CSI-RS Activation Result Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| CSI-RS Deactivation List |  | 0..1 |  |  |
| >CSI-RS Deactivation Result Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellList | Maximum no. of Cells in which the SP CSI-RS is activated or deactivated, the maximum value is 8. |

/******************Next change*******************************/

9.3.1.x1	CSI-RS Resource Configuration

This IE contains the CSI-RS resource configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI-RS Resource Configuration To AddModList | O |  | OCTET STRING | Contains the ltm-NZP-CSI-RS-ResourceToAddModList as defined in TS 38.331 []. |
| CSI-RS Resource Configuration To Release List | O |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceToReleaseList contained in the LTM-Config IE as defined in TS 38.331 []. |

/******************Next change*******************************/

9.3.1.x2	LTM No Security Change Information

This IE contains the LTM no security change configuration for the serving cell and LTM candidate cell(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Serving Cell No Security Change Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellNoSecurityChangeID contained in the LTM-Config IE, as defined in TS38.331 [8], for the current serving cell. |
| LTM No Security Change Configuration List |  | 0..1 |  |  |
| >LTM No Security Change Configuration Item IEs |  | 1.. <maxnoofLTMCells> |  |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >LTM No Security Change ID | M |  | OCTET STRING | Includes the ltm-NoSecurityChangeID IE, as defined in TS 38.331 [8]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

/******************End of changes*******************************/

Annex – Text Proposal for LTM BLCR for TS 38.300

Figure 9.2.3.5.2-1. Signalling procedure for LTM

The procedure for LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure LTM.

3.	The source gNB requests LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s). For inter-gNB LTM, the source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. For both intra and inter-gNB LTM, the source gNB may request the candidate cell(s)/gNB(s) to provide the CSI resource configuration .

4.	Admission Control may be performed by the candidate cells(s)/gNB(s).

5.	The candidate prepares and provides the LTM configuration(s) to the source gNB. For inter-gNB LTM, the candidate gNB(s) respond(s) (with HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. For both intra and inter-gNB LTM, the candidate may also include additional information related to the CSI configuration and early sync information upon request.

6.	The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI resource configuration.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI report configuration.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

NOTE :	Step 6 may also be triggered after step 14, or after step 17 by implementation for subsequent LTM.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to the candidate gNB(s).

10/11. Early DL and UL synchronization to some LTM candidate cell(s) may be performed. The source gNB may activate or deactivate the TCI states of the candidate LTM cells.  The UE may perform UL synchronization with LTM candidate cell(s), by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the source gNB. With a NW triggered UL synchronization, a PDCCH order is received from the source cell to trigger CFRA to a candidate cell, the UE performs early TA acquisition by sending preamble towards the indicated candidate cell. In order to minimize the data interruption on the source cell due to CFRA towards the indicated candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB instead.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

12a. The source gNB sends the CSI-RS COORDINATION message to the candidate gNB(s) to activate/deactive the semi-persistant CSI-RS resources.

13.	The source gNB determines to initiate LTM. L3 measurement can also be used to determine this step.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a TA command for the target cell. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

NOTE :	Up to implementation, data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 14.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 16 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs. This message may include the new security key(s) to be used with the UE.

21. The candidate gNB(s) responds to the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

NOTE: 	If early data forwarding is applied, the candiate gNB(s) may discard the buffered user data for the UE after receiving the LTM CONFIGURATION UPDATE message from the new source gNB.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

The steps 10-22 can be performed multiple times for subsequent LTM cell switch executions using the LTM candidate configuration(s) provided in step 8.

The procedure over the air interface described in Figure 9.2.3.5.2-1 is applicable to both intra-gNB-DU LTM , inter-gNB-DU LTM and inter-gNB-CU LTM. The overall LTM procedures over F1-C interface are captured in TS 38.401[4].