3GPP TSG-RAN WG3 Meeting #128	R3-253602

Malta, MT, 19 – 23 May, 2025

Title:	Discussion on SONMDT for Intra-NTN mobility

Agenda Item:	10.3.1

Source:	Huawei, CMCC, China Unicom

Document for:	Discussion

1	Introduction

This document continues to discuss the enhancements for SON and MDT for NTN on top of the progress of last meeting and incoming LS in [1] from RAN2.

2	Discussion

2.1 Area scope enhancement for MDT in NTN

RAN3 made the following agreements in #127 meeting:

RAN3 assumes both geographical area and the mapped cell ID as Area Scope of logged MDT for NTN over NGAP are feasible and confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN.

In the reply LS [1] from RAN2, it states that:

Furthermore, TS 38.300 captures the following standard texts for mapped cell ID:

- 16.14.5 NG-RAN signalling - […] The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported: - The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information; - The Cell Identity used for Paging Optimization in NG interface; - The Cell Identity used for Area of Interest; - The Cell Identity used for PWS. The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID. The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging. NOTE 1: The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID. - The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network. NOTE 2: A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions). The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

From above standard texts, it can be learned that the mapped cell ID are only used in several certain NGAP functions, like UE location information, PWS, etc. The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

Furthermore, as there is no mapped cell ID exchange between neighbour NG-RAN nodes over Xn interface, it may deduce that usually the OAM will configure the mapped cell IDs only related to the geographical areas that a NG-RAN node provides coverage.

Considering that only geographic area scope is configured to the UE in logged MDT, it is not feasible for a NG-RAN node to convert the mapped cell IDs of other NG-RAN nodes which is in the area scope received from AMF into geographical areas.

Observation: Additional OAM configurations are needed to enable a NG-RAN node to convert the mapped cell IDs of other NG-RAN nodes which is in the MDT area scope into geographical areas.

Proposal 1: To keep alignment with RAN2, RAN3 to only support the geographic area as the area scope of MDT in NTN.

Proposal 2: To keep alignment with RAN2, RAN3 to support reference location with radius and polygon-based area.

2.2 Logged MDT enhancement for NTN

The distance based event trigger for logged MDT was discussed at last meeting. One of the comments is to check the need of UE capability.

TS 37.320 captures the following UE capability related requirements:

For NR: - The UE indicates one capability bit for support for Logged MDT in RRC idle and inactive mode, to indicate that the UE supports logging of downlink pilot strength measurements, periodical logging and event-triggered logging. NOTE: Logged MDT suspension due to in-device coexistence detection is conditionally mandatory feature, i.e., it is mandatory supported if the UE supports Logged MDT in RRC idle and inactive mode.

From above quoted sentences, one may think that the existing UE capability bit for logged MDT can be reused also for the distance based event triggered logging.

Anyway, whether a new UE capability bit is needed or not is pure RAN2 realm and should be discussed and decided by RAN2.

Proposal 3: RAN3 to agree on the distance based event trigger for logged MDT in NTN and inform RAN2 and SA5 about the decision.

Proposal 4: To consult RAN2 on the UE capability issue for support of distance based event trigger.

The detailed justification of the need is provided in the annex.

Stage 3 design

The event trigger logged MDT configuration currently is defined as a CHOICE IE as specified in TS 38.413. If the new location based event trigger is introduced as a new branch in the Event Trigger Logged MDT Configuration IE, it means that the L1 event and the location event cannot be configured to the UE simultaneously. It will disable the UE to log measurements for the two event trigger types in a single logged MDT task which may impact signalling based logged MDT negatively.

Therefore, it is proposed to introduce the location based event trigger type as a separate IE, as implemented below.

#### 9.3.1.180 Event Trigger Logged MDT Configuration This IE defines the event trigger logged MDT configuration. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE Event trigger type; M. >Out-of-coverage. >>Out-of-Coverage Configuration; M; ENUMERATED (true, ...). >L1 Event. >>CHOICE L1 Event Threshold; M. >>>RSRP. >>>>Threshold RSRP; M; INTEGER (0..127); Corresponds to the RSRP-Range IE as defined in TS 38.331 [18].. >>>RSRQ. >>>>Threshold RSRQ; M; INTEGER (0..127); Corresponds to the RSRQ-Range IE as defined in TS 38.331 [18].. >>Hysteresis; M; INTEGER (0..30); This parameter is used within the entry and leave condition of an event triggered reporting condition.. >>Time to Trigger; M; ENUMERATED (ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120); Time during which specific criteria for the event needs to be met in order to trigger a measurement report.. Location Event Trigger Type; 0..1. >Threshold Distance; M; INTEGER(0.. 65535); The threshold distance from the reference location of the serving cell..

Proposal 5: RAN3 to agree that the distance based event trigger type can be configured simultaneously with the legacy event type.

3	Conclusion

This document proposes:

Observation: Additional OAM configurations are needed to enable a NG-RAN node to convert the mapped cell IDs of other NG-RAN nodes which is in the MDT area scope into geographical areas.

Proposal 1: To keep alignment with RAN2, RAN3 to only support the geographic area as the area scope of MDT in NTN.

Proposal 2: To keep alignment with RAN2, RAN3 to support reference location with radius and polygon-based area.

Proposal 3: RAN3 to agree on the distance based event trigger for logged MDT in NTN and inform RAN2 and SA5 about the decision.

Proposal 4: To consult RAN2 on the UE capability issue for support of distance based event trigger.

Proposal 5: RAN3 to agree that the distance based event trigger type can be configured simultaneously with the legacy event type.

The corresponding TPs are provided in [2] ~ [4].

The draft LS is provided in the annex.

4 	Reference

[1] R2-2503203	Relay LS on MDT for NTN

[2] R3-253603	(TP for MDT BLCR for TS38.413): Logged MDT enhancement for NTN

[3] R3-253604	(TP for MDT BLCR for TS38.423): Logged MDT enhancement for NTN

[4] R3-253605	(TP for MDT BLCR for TS37.320): Logged MDT enhancement for NTN

Annex – justification of distance based event trigger for logged MDT in NTN

Logged MDT is mainly applicable for UEs in RRC_IDLE/INACTIVE. The UE measurement in idle/inactive mode in NTN follows the network configurations, such as SMTCs, and assistance information provided in SIB19. Specifically, in NTN the UE can perform time-based and location-based measurements on neighbour cells. For example, the location based measurement will be performed by the UE in following criteria as specified in TS 38.304:

#### 5.2.4.2 Measurement rules for cell re-selection Following rules are used by the UE to limit needed measurements: If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ: - If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN quasi-Earth-fixed system and has obtained its location information: - If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements; - Else, the UE shall perform intra-frequency measurements; - else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving system and has obtained its location information: - if the distance between UE's location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements; - else, the UE shall perform intra-frequency measurements; - Else, the UE may not perform intra-frequency measurements; - Else, the UE shall perform intra-frequency measurements.

Similar measurement rules for inter-frequency and inter RAT frequency cell reselection can be found in the same section.

It can be shortly summarized that if the signal power/quality of the serving cell falls lower than the preconfigured thresholds, the measurement will be directly triggered. Else if the distance to the reference location of serving cell is larger than the preconfigured threshold, the measurement will be triggered.

According to TS 37.320 and TS 38.413, different to periodic logged MDT, event triggered logged MDT supports event trigger L1 Event to reduce the measurement data collected by logged MDT.

We think that it is beneficial from functional perspective to introduce a location based event trigger type for logged MDT in NTN cells so that the idle/inactive mode measurements can be collected when the UE distance to the reference location of serving cell is larger than a preconfigured threshold.

The main reason is that the legacy A2-like L1 event trigger type may not work so effectively. As the radio signal quality of NTN cells is expected quite good even in the border area due to the different near-far effect in TN and NTN as depicted in the Figure below.

Figure: near-far effect in TN and NTN

In order to enable the network to collect radio measurements from idle/inactive UEs at NTN cells border, it is proposed to introduce a location based event trigger type for logged MDT in NTN cells and inform RAN2 and SA5 to design the corresponding protocol.

Annex – DRAFT LS OUT

3GPP TSG-RAN WG3 Meeting #128	R3-25xxxx

Malta, MT, 19 – 23 May, 2025

Title:	[DRAFT] LS on SONMDT for NTN

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2, SA5

Cc:

Contact person: 	Zhang Hongzhuo

zhanghongzhuo@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

RAN3 discussed the enhancement to logged MDT for idle and inactive UEs in NTN. To collect radio measuremnts at the border of NTN cells, RAN3 agrees to introduce a new event trigger, i.e., distance based trigger event for logged MDT. RAN3 also thinks that this new trigger event can be configured to the UE together with the legacy events if any.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 and SA5 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#129	25 – 29 Aguest  2025 	Bangalore, India