3GPP TSG-RAN WG3 Meeting #128	R3-253604

Malta, MT, 19 – 23 May, 2025

Title:	(TP for MDT BLCR for TS38.423): MDT enhancement for NTN

Agenda Item:	10.3.1

Source:	Huawei, CMCC, China Unicom

Document for:	Other

1	Introduction

This document contains a TP for MDT BLCR for TS38.423 to implement the MDT enhancement for NTN.

Annex	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

9.2.3.126	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Immediate MDT and Trace, Logged MDT only, ...) |  | – |  |
| CHOICE Area Scope of MDT-NR | O |  |  |  | – |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT-NR |  | 1 .. <maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. | – |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1 |  |  | – |  |
| >>>TAI List for MDT Item |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >>>>TAC | M |  | 9.2.2.5 |  | – |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | ignore |
| >>CAG List for MDT |  |  | 9.2.3.191 |  | – |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | – | - |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>TAC | M |  | 9.2.2.5 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >SNPN Based MDT |  |  |  |  | YES | ignore |
| >> SNPN List for MDT |  | 1..<maxnoofMDTSNPNs> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >GeoArea Based MDT |  |  |  |  | YES | ignore |
| >>Geographical Area |  |  | 9.2.3.x |  |  |  |
| CHOICE MDT Mode | M |  |  |  | – |  |
| >Immediate MDT-NR |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [43]. First Bit = M1, Second Bit= M2, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, Seventh Bit = M6, Eighth Bit = M7. Value "1" indicates "activate" and value "0" indicates "do not activate". This version of the specification does not use bits 3. | – |  |
| >>M1 Configuration | C-ifM1 |  | 9.2.3.128 |  | – |  |
| >>M4 Configuration | C-ifM4 |  | 9.2.3.129 |  | – |  |
| >>M5 Configuration | C-ifM5 |  | 9.2.3.130 |  | – |  |
| >>MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [43]. First Bit = GNSS Other bits are reserved for future use and are ignored if received. Value "1" indicates "activate" and value "0" indicates "do not activate". The eNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to "1". | – |  |
| >>M6 Configuration | C-ifM6 |  | 9.2.3.131 |  | – |  |
| >>M7 Configuration | C-ifM7 |  | 9.2.3.132 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >Logged MDT-NR |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960 , ms61440, infinity, ...) | Corresponds to information provided in the LoggingInterval IE as defined in TS 38.331 [10]. The value "infinity" represents one shot logging, i.e., only one log per event in the logged MDT report. | – |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90, 120) | Corresponds to information provided in the LoggingDuration IE as defined in TS 38.331 [10]. Unit: [minute]. | – |  |
| >>CHOICE Report Type | M |  |  |  | – |  |
| >>>Periodical |  |  |  |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Logged Event Trigger Config | M |  | 9.2.3.137 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >>Area Scope of Neighbour Cells | O |  | 9.2.3.140 |  | – |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [10]. | – |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | – |  |
| PNI-NPN Area Scope of MDT |  | 0..1 |  |  | YES | Ignore |
| >CAG List for MDT | M |  | 9.2.3.191 |  | – |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.3.137	Logged Event Trigger Config

This IE configures with UE with specific events for triggering MDT configuration. Current specified event is based on out of coverage (OOC) detection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Event Type Trigger | M |  |  |  | – |  |
| >Out of Coverage |  |  |  |  |  |  |
| >>Out of Coverage Indication | M |  | ENUMERATED (true, …) |  |  |  |
| >L1 Event |  |  |  |  |  |  |
| >>CHOICE L1 Event Threshold | M |  |  |  |  |  |
| >>>RSRP |  |  |  |  |  |  |
| >>>>Threshold RSRP | M |  | INTEGER (0..127) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |
| >>>RSRQ |  |  |  |  |  |  |
| >>>>Threshold RSRQ | M |  | INTEGER (0..127) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |  |  |
| >>Hysteresis | M |  | INTEGER (0..30) | This parameter is used within the entry and leave condition of an event triggered reporting condition and corresponds to information provided in the Hysteresis IE as defined in TS 38.331 [10]. |  |  |
| >>Time to trigger | M |  | ENUMERATED (ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120) | Time during which specific criteria for the event needs to be met in order to trigger a measurement report. Corresponds to information provided in the TimeToTrigger IE as defined in TS 38.331 [10] |  |  |
| Distance Event Trigger Type |  | 0..1 |  |  | YES | ignore |
| >Threshold Distance | M |  | INTEGER(0.. 65535) | The threshold distance from the reference location of the serving cell to the UE. |  |  |

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

9.2.3.x Geographical Area List

This IE contains the geographical area list for MDT in NTN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Geographical Area Item IEs |  | 1..<maxnoofGeoAreas> |  |  |
| >CHOICE Geographical Area | M |  |  |  |
| >>Polygon based |  |  |  |  |
| >>> Polygon | M |  | OCTET STRING | Specified in TS 37.355 [x]. Indicates the polygon based area scope in MDT in NTN. |
| >>CircleArea based |  |  |  |  |
| >>>Reference location | M |  | OCTET STRING | An Ellipsoid-Point specified in TS 37.355 [x]. Indicates the reference location in MDT in NTN. |
| >>>Distance radius | e |  | INTEGER(0..65535) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofGeoAreas | Maximum no. of geographical areas. Value is 8. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>