3GPP TSG-RAN WG3 Meeting #128	R3-253630

Malta, MT, 19th – 23th May 2025

Agenda item:	 10.4

Source:		Samsung, Jio Platforms

Title:	(TP for SON BLCR for TS37.340) Rel-18 SON and MDT leftovers

Document for:		Discussion and Decision

1	Introduction

The following leftovers in Rel-18 SON/MDT have been agreed in the scope of Rel-19 SON/MDT WI.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

The contribution discussed MHI Enhancement for SCG Deactivation/Activation.

2	Discussion

RAN2 achieved the following agreement on MHI enhancement for SCG Deactivation/Activation:

Agreements - We will introduce a UE based solution for SCG act/deact MHI, as an optional UE feature. - UE reports the time the UE has spent in PSCell with SCG activated, TBD details, e.g. percentage of time or absolute time, etc.

Agreements: - UE reports the absolute time it has spent in PSCell with SCG activated in MHI.

In RAN3 126 meeting, it has been agreed to support a network-based solution for enhancing PSCell activation/deactivation. There are two remaining issues:

- Do we prefer an absolute time in active state or a relative time?

- Is additional information needed, e.g. Throughput threshold used for activating, Average throughput, Number of activations?

For MHI, RAN2 agreed “UE reports the absolute time it has spent in PSCell with SCG activated in MHI”. It is the duration spent in the PSCell with SCG activated while connected to a PCell. If the PSCell has moved between activated and deactivated multiple times, the accumulated duration is logged.

Similarly, the network can collect the time duration spent in PSCell with SCG active in UHI.

Proposal 1: The network can collect the time duration spent in the PSCell with SCG activated while connected to a PCell.

For the second issue, once the nodes e.g. MN have the PSCell information of activation/deactivation status and time duration spent in each status, it know whether the PSCell is in deactivation status for a long time without actual usage, the configuration of dual connectivity can be optimised, such as the necessity to configure the UE as DC, or choose which cell as the PSCell. Hence, the PSCell activation/deactivation status and time duration with SCG activated are enough without introducing additional information.

Observation 1: PSCell activation/deactivation status and time duration with SCG activated are enough for the optimization of DC configuration.

Proposal 2: There is no need to introduce additional information, e.g. Throughput threshold used for activating, Average throughput, Number of activations, etc.

The information of PSCell status is used to optimize the configuration of dual connectivity, such as the necessity to configure the UE as DC, or choose which cell as the PSCell. So the MN needs to know the information. the activation/deactivation decision is transferred via MN. Thus, MN also knows whether SCG is deactivated or not, MN can collect the time duration information. Based on the time duration information, the MN could understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

Observation 2: The MN could know the PSCell activation/deactivation status and time duration with SCG activated which are enough for the optimization of DC configuration.

In practice, the SN doesn’t need collect the time duration with SCG activated because it is the MN to trigger DC configuration. Even if the SN need to collect such information, the SN also knows the PSCell status change and corresponding time. The SN could store the status and time duration by itself.

Observation 3: The SN could know the PSCell activation/deactivation status and time duration with SCG activated by itself.

Based on the understanding so far, it seems only stage 2 impact is needed. If any scenario is identified that the PSCell status information should be transmitted between nodes, such as from MN to target SN, the stage 3 impact can be discussed accordingly.

Proposal 3: Agree the stage 2 description of network-based solution in Annex A.

3	Conclusion

This contribution discussed MHI Enhancement for SCG Deactivation/Activation. We have the following observation and proposals.

Proposal 1: The network can collect the time duration spent in the PSCell with SCG activated while connected to a PCell.

Observation 1: PSCell activation/deactivation status and time duration with SCG activated are enough for the optimization of DC configuration.

Proposal 2: There is no need to introduce additional information, e.g. Throughput threshold used for activating, Average throughput, Number of activations, etc.

Observation 2: The MN could know the PSCell activation/deactivation status and time duration with SCG activated which are enough for the optimization of DC configuration.

Observation 3: The SN could know the PSCell activation/deactivation status and time duration with SCG activated by itself.

Proposal 3: Agree the stage 2 description of network-based solution in Annex A.

Reference

[1] TS38.331

[2] TS36.331

[2] TS38.423

Annex A: TP for TS37.340 on MHI enhancement

## 13.3	SCG UE history information

The MN stores and correlates the UE History Information from MN and SN(s) as long as the UE stays in MR-DC, forwards UE History Information and optional UE History Information from the UE to its connected SNs. The resulting information is then used by SN for dual-connectivity operation. The SN is in charge of collecting SCG UE history information and providing the collected information to the MN.

If the UE stays in a PSCell for a duration exceeding the maximum value of the Time Stay parameter, the SN may store the PSCell information with consecutive entries using the same PSCell identity. The total stay time in this PSCell is the sum of stay time for all consecutive PSCell with the same identity.

The SN shall provide the collected SCG UE history information, if available, to the MN in the following procedures:

-	the SN Release, and SN initiated SN Change procedures

-	the MN initiated SN Modification procedure if requested by the MN in this procedure

-	the SN initiated SN modification procedure upon PSCell change if subscribed in the SN Addition procedure

When the target NG-RAN node receives the SCG UHI from the source NG-RAN node via Handover Request message for CHO, the target NG-RAN node updates the time UE stayed in cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target NG-RAN node. The updated value of the time UE stayed in the source PSCell is equal to the value received from the source NG-RAN node during the Handover Preparation plus the time from receiving Handover Request message from the source NG-RAN node to receiving RRC Reconfiguration Complete message from the UE.

When the target SN receives the SCG UHI from the MN via SN Addition Request message for CPC, the target SN updates the time UE stayed in the cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target SN. The updated value of the time UE stayed in the latest PSCell is equal to the value received from the MN via the SN Addition Request message plus the time from receiving SN Addition Request message from the MN to receiving SN Reconfiguration Complete from the MN.

The MN stores time duration spent in the PSCell with SCG activated while connected to a PCell and use it for future dual-connectivity configurations.