3GPP TSG RAN3 meeting #128   	         	                         R3-253633

19th – 23rd May 2025  Malta, MT

Agenda Item:	10.3.1

Source:	China Unicom

Title:	Discussion on SON/MDT enhancements for NTN mobility

Document for:	Discussion

Introduction

The Work Item on SON/MDT enhancement has been agreed in RAN#102 (RP-234038) with the following objectives:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

This paper discusses the SON/MDT enhancements for NTN mobility.

Discussion

Distance-based event trigger for logged MDT

In RAN3#127bis meeting, there is the following FFS:

Distance-based event trigger for logged MDT?

The UE in NTN network can support mobility in RRC IDLE/INACTIVE states, and can support CHO in RRC CONNECTED state.

For UE in RRC IDLE/INACTIVE states, the network will configure the reference location and the moving reference location to UE via SIB19. UE will execute the cell re-selection according to the reference location and the moving reference location configured by the network.

For UE in RRC CONNECTED state, the network can configure the location based CHO for UE, and the UE will execute the conditional handover when the condition configured by the network is satisfied.

Since the NTN cell range is very large which may cover different administrative areas, distance-based event trigger for logged MDT is necessary for NTN measurement. Moreover, distance-based event trigger for logged MDT can also avoid unnecessary measurement for UE in cell centre where can help UE for power saving.

Proposal 1: Distance-based event trigger for logged MDT should be introduced in R19.

In RAN3#126 meeting, there is the following agreement:

RAN3 assumes both geographical area and the mapped cell ID as Area Scope of logged MDT for NTN over NGAP are feasible and confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN.

For the geographical area definition, the MBS NTN can be reused for MDT NTN. RAN2 agreed the following proposal in R2-2410638:

Proposal 1: The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

The reference location configuration is critical for the UE to execute cell re-selection and handover, and how to configure the reference location and the distance threshold for UE and reference location relies on the network deployment for NTN network. When the reference location and the distance threshold are not configured properly, there will be coverage hole or too much overlap coverage, which will lead to UE handover fail or unnecessary measurements for UE. The reference location and distance threshold configuration need to be optimized according to the measurements based on the reference location.

Proposal 2: The reference location and distance threshold configuration need to be optimized for UE cell re-selection and CHO.

For the cell re-selection and CHO case, the distance between the reference location is the critical judgment parameters according the condition configured by the gNB. How to optimize the configuration for cell re-selection and CHO relies on the measurements of the reference signal when the cell re-selection and CHO happen. The MDT measurement can be used for the measurements of the reference signal for certain distance to the reference point, and the reference point can be the reference point configured or will be configured for the cell re-selection and CHO.

Proposal 3: RAN3 agreed to introduce a reference location plus a radius as the area scope for MDT in NG interface.

For the logged MDT areas, it can be further limited to a maximum distance or minimum distance, or it can limited to a  maximum distance plus a minimum distance. For the maximum distance, it can limit the MDT measurement area since the NTN cell maybe very large, and for the minimum distance, it can limited the unnecessary measurement if the NTN UE is at the centre of the cell, which is close to the reference point.

Proposal 4: A reference location plus a maximum radius and minimum radius can be considered for logged MDT of NTN.

Condition Handover Optimization for NTN network

Location-based CHO Optimization for NTN network

In RAN3#127 meeting, RAN3 had the following agreements:

Revert the WA: There is no need to report any other info in the RLF report for the location based CHO. For earth moving case, the accuracy and complexity issue of the network-based solution is acknowledged. - Option 1: to introduce the absolute UTC time of when RLF/HOF happens in the RLF report. - Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell (when RLF occurred). FFS on option 1 or option 2.

For the location-based CHO, the accuracy on the estimation of the time when RLF/HOF happens will limit the location-based CHO configuration, which is not as precise as the original granularity. It is better to introduce the measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell to indicate the accurate RLF/HOF time.

For the location-based CHO, the UE should measure the distance to the reference location of the serving cell and the target cell, and it will execute the handover after the distance threshold for the serving cell and the target cell are satisfied. The UE will execute the handover too early or too late, or even handover to the wrong cells because the handover opportunity is not appropriate. In this case, the location-based CHO configuration may need to be optimized. The handover too early or too late, or handover to the wrong cells may be detected by the radio link failure procedure, the UE can record the information about the CHO when the RLF happen, and the UE can report the CHO information to the gNB for CHO configuration optimization.

The location-based CHO condition is configured by the gNB according to the network planing of the NTN network, and the  measurement result of the reference signal of the serving cell, handover target cell and other neighbor cells can be used for the network to optimize the distance thresh from reference for serving cell and neighbor cells. When the location-based CHO happen, these information also need to be recorded, and  these information can be send to the gNB when the RLF happens.

Proposal 5: The UE can record the distance from the reference point when the location-based CHO happen and RLF occurred, and report the information to the gNB when the radio link failure happens.

Proposal 6: The UE can record the measurement result of the reference signal of the serving cell, handover target cell and other neighbor cells when the location-based CHO happen, and report  these information to the gNB when the radio link failure happens.

The location-based CHO will configure location-based CHO condition for the handover, and the UE will execute the handover during the configured condition. Even if the handover is successful, and there is no radio link failure happened after the CHO, the CHO may also need to be optimized. The CHO may happen near the location of distanceThreshFromReference1 or distanceThreshFromReference2, therefore the gNB also needs to get the CHO related information to further optimize the CHO configuration to avoid the potential handover failure. The condition for the UE to report the CHO related information can be configured by the gNB.

Proposal 7: The gNB can configure the CHO successful report condition for the UE to report the location-based CHO related information.

Proposal 8: The UE can record and report the location-based CHO related information to the gNB when the location-based CHO related condition is satisfied.

Conclusion

Proposal 1: Distance-based event trigger for logged MDT should be introduced in R19.

Proposal 2: The reference location and distance threshold configuration need to be optimized for UE cell re-selection and CHO.

Proposal 3: RAN3 agreed to introduce a reference location plus a radius as the area scope for MDT in NG interface.

Proposal 4: A reference location plus a maximum radius and minimum radius can be considered for logged MDT of NTN.

Proposal 5: The UE can record the distance from the reference point when the location-based CHO happen and RLF occurred, and report these information to the gNB when the radio link failure happens.

Proposal 6: The UE can record the measurement result of the reference signal of the serving cell, handover target cell and other neighbor cells when the location-based CHO happen, and report  these information to the gNB when the radio link failure happens.

Proposal 7: The gNB can configure the CHO successful report condition for the UE to report the location-based CHO related information.

Proposal 8: The UE can record and report the location-based CHO related information to the gNB when the location-based CHO related condition is satisfied.