3GPP TSG-RAN WG3 Meeting #128	R3-253659

Malta, MT, May 19th – 23rd, 2025

Agenda item:	13.2

Source: 	LG Electronics Inc.

Title: 	(TP for NR_Mob_Ph4 TS 38.423 and TS 38.473) Discussions on remaining issues related to Inter-CU LTM security

Document for:	Other

# Introduction

Regarding the Inter-CU LTM security, SA3 reached the following conclusions [1]:

SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

Based on this SA3’s compromise, RAN3-127bis reached the following agreements and informed SA3 [2]:

Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM.

The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure.

For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy.

The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB.

Whether to define additional information from CU to DU to deliver NCC value.

RAN2-129bis also progressed and inform their agreements to RAN3 [3]:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Regarding the security key handling in Rel-19 Inter-CU LTM, RAN2 made the following agreements at RAN2#129bis meeting:

Agreements: - For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE. - NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included. - NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG. - A list of sk-counters is linked to a Rel-19 set ID configured by the SN. - At RLF and reconfiguration with sync failure: - If RLF: - If the selected candidate cell has the same Rel-19 set ID as source (no security key change), the UE performs fast failure recovery (same as in Rel-18). - If reconfiguration failure (inter-CU LTM): - If the selected candidate cell has the same Rel-19 set ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 set IDs is allowed. - The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message.

The term “R19 set ID” refers to the “LTM no security change ID” configured to the UE for serving cell and candidate cells separately. RAN2 assumes that even for intra-CU LTM, the network is allowed to configure different R19 set IDs for different cells (including serving and/or intra-CU candidate cells) if the network wants to trigger security key update. From UE perspective, security key update is performed when the R19 set IDs are different between the serving cell and the target cell, whether it is intra-CU or inter-CU LTM can be transparent to the UE.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Considering these progresses, we further discuss the remaining aspects for Inter-CU LTM security design.

# Discussion

## Supporting NCC delivery to the UE via MAC CE

The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure.

Whether to define additional information from CU to DU to deliver NCC value.

First, regarding FFS, RAN2 agreed the following:

Agreements:

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

- NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

As RAN2 agreed, DU needs to know whether the selected target cell requires NCC delivery for updating the UE’s master key. To support this, as RAN2 agreed, DU needs to be configured with the Rel-19 set IDs assigned for each candidate cell (a.k.a. LTM no security change ID, defined by RAN2 to control the UE’s master key update process during Inter-CU LTM) [6]:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### –	LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...,

[[

ltm-NoSecurityChangeID-r19                     LTM-NoSecurityChangeId-r19                            OPTIONAL,    -- Need M

ltm-ExecutionCondition-r19                     SetupRelease {LTM-ExecutionConditionList-r19}         OPTIONAL     -- Need M

]]

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

LTM-NoSecurityChangeId-r19 ::= INTEGER (1..maxNrofLTM-Configs-plus1-r18)

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

LTM-Candidate field descriptions

ltmExecutionCondition This field can only be included within an ltm-Config associated with the MCG.

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the LTM candidate configuration contained in ltm-CandidateConfig.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

ltm-NoResetID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID.

ltm-NoSecurityChangeID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID.

ltm-UE-MeasuredTA-ID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID. This field is absent if tag2 is present for this LTM candidate configuration.

#### –	LTM-Config

The IE LTM-Config is used to provide LTM configurations.

LTM-Config information element

-- ASN1START

-- TAG-LTM-CONFIG-START

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        SetupRelease {ReferenceConfiguration-r18}                             OPTIONAL,   -- Cond NR-DC

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToAddModList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL,   -- Need N

attemptLTM-Switch-r18                 ENUMERATED {true}                                                    OPTIONAL, -- Cond LTM-MCG

ltm-ServingCellUE-MeasuredTA-ID-r18   INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

...,

[[

ltm-ServingCellNoSecurityChangeID-r19       LTM-NoSecurityChangeId-r19                                          OPTIONAL,   -- Need N

ltm-ServingCellExecutionCondition-r19       SetupRelease {LTM-ExecutionConditionList-r19}                       OPTIONAL    -- Need M

]]

}

-- TAG-LTM-CONFIG-STOP

-- ASN1STOP

LTM-Config field descriptions

ltm-ServingCellExecutionCondition This field can can only be included within an ltm-Config associated with the MCG.

| Conditional Presence | Explanation |
| --- | --- |
| LTM-MCG | This field is optional present for the MCG, Need R, if the UE is configured with at least an LTM candidate configuration associated to the MCG. Otherwise, the field absent. |
| NR-DC | If ltm-ConfigNRDC is configured, this field is optionally present, Need M, within ltm-ConfigNRDC and absent within ltm-Config. If ltm-ConfigNRDC is not configured, this field is optionally present, Need M, within ltm-Config. |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### –	VarLTM-ServingCellNoSecurityChange

The IE VarLTM-ServingCellNoSecurityChange is used to store the ID associated with the serving cell based on which the UE determines whether a security change is needed or not upon an LTM cell switch procedure and the list of sk-counter value that can be used when a security key change is performed at the SCG.

VarLTM-ServingCellNoSecurityChange UE variable

-- ASN1START

-- TAG-VARLTM-SEVINGCELLNOSECURITYCHANGE-START

VarLTM-ServingCellNoSecurityChangeID-r19 ::=     SEQUENCE {

ltm-ServingCellNoSecurityChangeID-r19           INTEGER (1..maxNrofLTM-Configs-plus1-r18)                                 OPTIONAL,

ltm-SK-Counters-r19                             SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfigLTM-r19    OPTIONAL

}

-- TAG-VARLTM-SEVINGCELLNOSECURITYCHANGE-STOP

-- ASN1STOP

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Following the UE's logic, when the serving DU initiating a PCell switch based on L1 measurements, the DU can compare the IDs of the current serving cell and the selected target cell, and determine whether to deliver the received NCC value to the UE (e.g. if equal, no need to deliver via CSC MAC CE; if different, deliver via CSC MAC CE).

Proposal 1: Configure the Rel-19 set IDs (introduced in RAN2’s RRC running CR [6]) to the DU, so that DU can compare the IDs of the current serving cell and the selected target cell during Inter-CU PCell switching, and determine whether NCC delivery (if received from CU) is required.

Proposal 2: Enhance the LTM Security Information IE captured in UE CTXT MOD REQ, to include the Rel-19 set IDs assigned for each candidate cell.

## Rel-19 Set ID assignment by CUs

For inter-CU LTM, the candidate cell(s) under different CUs shall be assigned with different Rel-19 set ID(s). The UE’s master key shall be updated when PCell switch happens across CUs. Moreover, as RAN2 observed in the LS sent to RAN3 [3], it is not precluded that NW assigns the different Rel-19 set IDs for different cells within the same CU:

The term “R19 set ID” refers to the “LTM no security change ID” configured to the UE for serving cell and candidate cells separately. RAN2 assumes that even for intra-CU LTM, the network is allowed to configure different R19 set IDs for different cells (including serving and/or intra-CU candidate cells) if the network wants to trigger security key update. From UE perspective, security key update is performed when the R19 set IDs are different between the serving cell and the target cell, whether it is intra-CU or inter-CU LTM can be transparent to the UE.

This coordination requires interaction between S-CU and C-CU(s) during Inter-CU LTM preparation phase. Among several possible approaches, we think that a straightforward way is to let S-CU determine the split of the Rel-19 set ID space across itself and the involved C-CU(s). S-CU is responsible for initiating Inter-CU LTM for the UE and selecting initial candidate cells, so it is well-positioned to manage and partition the overall ID space for the involved C-CUs. Each CU (S-CU or a C-CU) can then independently assign specific ID values to its own candidate cells, within the range allocated by S-CU.

S-CU can provide these ranges (or specific values) allocated to each C-CU via HO REQ or LTM Configuration Update. Each C-CU can assign Rel-19 set IDs to its own candidate cells and reply the assigned values back to S-CU. S-CU will then compile the final LTM configuration, including the assigned Rel-19 set IDs for all the candidate cells, for delivery to the UE.

Proposal 3: To prevent overlap of Rel-19 set ID value assignment across CUs, S-CU determine the split of Rel-19 set ID space across itself and the involved C-CU(s).

Proposal 4: During Inter-CU LTM preparation phase, S-CU provides these ranges (or specific values) allocated to each C-CU via HO REQ or LTM Configuration Update. Each C-CU assigns Rel-19 set IDs to its own candidate cells and reply the assigned values back to S-CU.

Moreover, when Inter-CU PCell switch happens, the final Rel-19 set IDs assigned for each and every candidate cells shall also be transferred to the new serving CU, so that the new serving CU can deliver these values to its DU for subsequent LTM operation. For this purpose, the Cell Switch Notification procedure can be enhanced to deliver such assigned Rel-19 set ID list.

Proposal 5: Enhance Cell Switch Notification to deliver the Rel-19 set IDs assigned for all the candidate cells to the new serving CU, so that the new serving CU can deliver these values to its DU for subsequent LTM operation.

## Supporting key distribution

Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM.

For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy.

The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB.

Regarding FFS, we believe that the LTM Configuration Update procedure should NOT be designed on a per-cell basis, since a new key is derived for each candidate cell, and the procedure may need to deliver multiple keys for multiple candidate cells belonging to a single CU. However, the current XnAP BL CR [4] still captured the stage-3 structure as per-cell basis:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### 9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |
| Early Sync Information Request | O |  | 9.2.1.xx3 |  | YES | ignore |
| Early Sync Information | O |  | 9.2.1.xx6 |  | YES | ignore |
| Early Sync Information for SUL | O |  | 9.2.1.xx6 |  | YES | Ignore |
| Data Forwarding Info | FFS |  |  |  | YES | ignore |
| Key NG-RAN Star | O |  | BIT STRING (256) | KNG-RAN* defined in TS 33.501 [28]. | YES | ignore |
| Security Information for LTM (FFS on the name) | O |  | 9.2.1.x12 |  | YES | ignore |

Editor’s note: The Data Forwarding Info IE is a list, the granularity is FFS.

#### 9.1.1.x4	 LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Response | O |  | 9.2.1.xx2 |  | YES | ignore |
| Early Sync Information Response | O |  | 9.2.1.xx4 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Given this procedure is now being specified from scratch, there is no reason to stick to a per-cell handling. Syncing up configurations may require updating for multiple candidate cells of a single CU simultaneously by one-shot. One example is the post-LTM-preparation step, where this procedure is utilized to finalize LTM configurations to the UE with each C-CU, based on the common information collected from all the admitted candidate cells. Any update or change to this common information (e.g. modification to a set of candidate cells) may require a holistic update of LTM configurations generated by each CU.

Therefore, the stage-3 message design for LTM Configuration Update is better to support updates related to multiple candidate cells by one shot, including the delivery of newly derived keys for each of them belonging to the recipient CU.

Proposal 6: Correct the stage-3 message design of the LTM Configuration Update procedure to support updates for multiple candidate cells simultaneously, including the delivery of newly derived keys for each candidate cell belonging to the recipient CU.

Moreover, in XnAP BL CR [4], the LTM Configuration Update procedure is currently written as being initiated by any CU. However, this should be corrected to ensure that it is triggered by the “source”. This correction aligns with the current 38.300 BL CR [5], for which it is initiated by the serving CU (i.e. S-CU) during LTM preparation. This also aligns with that a CU currently serving the UE is responsible of controlling configuration updates to potential target CUs and handling key distribution in a consistent and successive manner throughout LTM operations.

Proposal 7: Correct the LTM Configuration Update procedure to be initiated from the source toward the target (similar to the existing HO procedure). It is initiated from S-CU during the preparation phase, and also expected to be initiated from the CU who is currently serving the UE (i.e. “source”) during subsequent LTM for syncing up configurations and delivering newly derived keys.

The TPs for XnAP BL CR [4] and F1AP BL CR [7] can be found in Section 5 and Section 6, respectively.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

## Supporting NCC delivery to the UE via MAC CE

Proposal 1: Configure the Rel-19 set IDs (introduced in RAN2’s RRC running CR [6]) to the DU, so that DU can compare the IDs of the current serving cell and the selected target cell during Inter-CU PCell switching, and determine whether NCC delivery (if received from CU) is required.

Proposal 2: Enhance the LTM Security Information IE captured in UE CTXT MOD REQ, to include the Rel-19 set IDs assigned for each candidate cell.

## Rel-19 Set ID assignment by CUs

Proposal 3: To prevent overlap of Rel-19 set ID value assignment across CUs, S-CU determine the split of Rel-19 set ID space across itself and the involved C-CU(s).

Proposal 4: During Inter-CU LTM preparation phase, S-CU provides these ranges (or specific values) allocated to each C-CU via HO REQ or LTM Configuration Update. Each C-CU assigns Rel-19 set IDs to its own candidate cells and reply the assigned values back to S-CU.

Proposal 5: Enhance Cell Switch Notification to deliver the Rel-19 set IDs assigned for all the candidate cells to the new serving CU, so that the new serving CU can deliver these values to its DU for subsequent LTM operation.

Supporting key distribution

Proposal 6: Correct the stage-3 message design of the LTM Configuration Update procedure to support updates for multiple candidate cells simultaneously, including the delivery of newly derived keys for each candidate cell belonging to the recipient CU.

Proposal 7: Correct the LTM Configuration Update procedure to be initiated from the source toward the target (similar to the existing HO procedure). It is initiated from S-CU during the preparation phase, and also expected to be initiated from the CU who is currently serving the UE (i.e. “source”) during subsequent LTM for syncing up configurations and delivering newly derived keys.

The TPs for XnAP BL CR [4] and F1AP BL CR [7] can be found in Section 5 and Section 6, respectively.

# Reference

[1] R3-251525/S3-251124, Reply LS on security handling for inter-CU LTM in non-DC cases, SA3

[2] R3-252449, Reply LS for Reply LS on security handling for inter-CU LTM in non-DC cases, RAN3

[3] R3-253009/R2-2503012, LS on RAN2 agreements for security key handling in inter-CU LTM, RAN2

[4] R3-253089, “(BL CR to 38.423) Xn support for inter-CU LTM”, Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm

[5] R3-253086, “(BL CR to 38.300) Support for Inter-CU LTM”, Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung, Ofinno

[6] R2-2504125, “Running RRC CR for inter-CU and conditional LTM”, Ericsson

[7] R3-253149, “(BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM”, Huawei, Nokia, Samsung, Google, NEC, China Telecom, Ericsson, LG Electronics, CATT, Ofinno, ZTE, Lenovo, Qualcomm Incorporated

# TP for XnAP BL CR [4]

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.1.1	Messages for Basic Mobility Procedures

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. NOTE: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| >NR A2X UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| >LTE A2X UE PC5 Aggregate Maximum Bit Rate | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.2.3.108 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| >Conditional Handover Time Based Information | O |  |  | This IE only applies to NTN. | YES | reject |
| >>Handover Window Start | M |  | INTEGER (0..549755813887) | Corresponds to information provided in the t1-Threshold contained in the ReportConfigNR IE as defined in TS 38.331 [10] | – |  |
| >>Handover Window Duration | M |  | INTEGER (1..6000) | Corresponds to information provided in the duration contained in the condEventT1 contained in the ReportConfigNR IE as defined in TS 38.331 [10] | – |  |
| >Maximum Number of Conditional Reconfigurations to Prepare | O |  | INTEGER (1..8 , …) | Indicates that the target NG-RAN node may prepare for CHO with candidate PSCell(s) configuration and indicates the maximum number of conditional reconfigurations that the target NG-RAN node may prepare. | YES | reject |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| IAB Authorization Status | O |  | ENUMERATED(authorized, not authorized, ...) | This IE indicates the authorization status of the IAB node. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) | This IE indicates that information on DL LBT Failures occurring at the target gNB that results in mobility failure is requested | YES | ignore |
| Aerial UE Subscription Information | O |  | 9.2.3.175 |  | YES | ignore |
| NR A2X Services Authorized | O |  | 9.2.3.176 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.2.3.177 |  | YES | ignore |
| A2X PC5 QoS Parameters | O |  | 9.2.3.178 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.180 | This IE contains information about cells that a UE is predicted to be connected to. | YES | ignore |
| Data Collection ID | O |  | 9.2.3.184 |  | YES | ignore |
| Candidate Relay UE Info List | O |  | 9.2.3.188 |  | YES | reject |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC configuration information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| Mobile IAB Authorization Status | O |  | 9.2.2.105 |  | YES | reject |
| Ranging and Sidelink Positioning Services Information | O |  | 9.2.3.208 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| LTM Handover Information Request | O |  |  |  | YES | reject |
| >LTM Information Request | O |  | 9.2.1.xx1 |  | – |  |
| Early Sync Information Request | O |  | 9.2.1.xx3 |  | YES | ignore |
| LTM Available Security Change ID List | O |  | 9.2.1.xx13 |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. Ignored if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| >CHO-CPAC Information | O |  | 9.2.3.202 |  | YES | reject |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | ignore |
| PDU Set based Handling Indicator | O |  | 9.2.3.206 |  | YES | ignore |
| LTM Handover Information Response | O |  |  |  | YES | ignore |
| >LTM Information Response | O |  | 9.2.1.xx2 |  | – |  |
| >Security Change ID | O |  | INTEGER (1..9) | Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [8], for the admitted candidate cell. | YES | reject |
| Early Sync Information Response | O |  | 9.2.1.xx4 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |
| LTM Assigned Security Change ID List | O |  | 9.2.1.xx13 |  | YES | reject |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| LTM Candidate Cell List |  | 1 .. <maxnoofLTMCells> |  |  | YES | ignore |
| >Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| >AS Security Information | O |  | 9.2.3.50 |  | YES | reject |
| >LTM Information Request | O |  | 9.2.1.xx1 |  | - |  |
| >Early Sync Information Request | O |  | 9.2.1.xx3 |  | - |  |
| >Early Sync Information | O |  | 9.2.1.xx6 |  | - |  |
| >Early Sync Information for SUL | O |  | 9.2.1.xx6 |  | - |  |
| Data Forwarding Info | FFS |  |  |  | YES | ignore |
| LTM Available Security Change ID List | O |  | 9.2.1.xx13 |  | YES | reject |
| LTM UE Security Capabilities | O |  | 9.2.1.x12 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

Editor’s note: The Data Forwarding Info IE is a list, the granularity is FFS.

#### 9.1.1.x4	 LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: target NG-RAN node  source NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| LTM Candidate Cell List |  | 1 .. <maxnoofLTMCells> |  |  | YES | ignore |
| >Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| >LTM Information Response | O |  | 9.2.1.xx2 |  | - |  |
| >Early Sync Information Response | O |  | 9.2.1.xx4 |  | - |  |
| LTM Assigned Security Change ID List | O |  | 9.2.1.xx13 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

      - 9.2.1.x12	LTM UE Security Capabilities

This IE contains security related information for subsequent LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Security Capabilities | M |  |  |  |
| >NR Encryption Algorithms | M |  | BIT STRING {nea1-128(1), nea2-128(2), nea3-128(3)} (SIZE(16, …)) | Each position in the bitmap represents an encryption algorithm: "all bits equal to 0" – UE supports no other NR algorithm than NEA0, "second bit" – 128-NEA1, "third bit" – 128-NEA2, "fourth bit" – 128-NEA3, “fifth to eighth bit” correspond to bit 4 to bit 1 of octet 3 in the UE Security Capability IE defined in TS 24.501 [30], other bits reserved for future use. Value ‘1’ indicates support and value ‘0’ indicates no support of the algorithm. Algorithms are defined in TS 33.501 [28]. |
| >NR Integrity Protection Algorithms | M |  | BIT STRING {nia1-128(1), nia2-128(2), nia3-128(3)} (SIZE(16, …)) | Each position in the bitmap represents an integrity protection algorithm: "all bits equal to 0" – UE supports no other NR algorithm than NIA0, "second bit" – 128-NIA1, "third bit" – 128-NIA2, "fourth bit" – 128-NIA3, “fifth to eighth bit” correspond to bit 4 to bit 1 of octet 4 in the UE Security Capability IE defined in TS 24.501 [30], other bits reserved for future use. Value ‘1’ indicates support and value ‘0’ indicates no support of the algorithm. Algorithms are defined in TS 33.501 [28]. |
| User Plane Security Information List |  | 1 |  |  |
| >User Plane Information Item |  | 1 .. <maxnoof PDU sessions > |  |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  |
| >>Security Indication | O |  | 9.2.3.52 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256. |

#### 9.2.1.xx13	LTM Security Change ID List

This IE indicates the list of LTM security change IDs, optionally associated with candidate cells.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Security Change ID Item IEs |  | 1..< maxnoofLTMCellsPlusOne> |  |  |
| >Security Change ID | M |  | INTEGER (1..9) |  |
| >LTM Cell ID | O |  | NR CGI 9.2.2.7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCellsPlusOne | Maximum no. of Cells configured for LTM allowed towards one UE plus one, the maximum value is 9. |

# TP for F1AP BL CR [7]

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.3.1.XX	LTM Security Information

This IE contains the security related information for LTM candidate cell(s) to support the UE in generating the key material for AS security during an inter-CU LTM cell switch.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Next Hop Chaining Count | M |  | INTEGER (0..7) | Next Hop Chaining Count (NCC) defined in TS 33.501 [Y] |
| Security Change Serving Cell Configuration | O |  | OCTET STRING | Includes the ltm-NoSecurityChangeID IE as defined in TS 38.331 [8], for the current serving cell. |
| Security Change Candidate Cell Information List |  | 0..1 |  |  |
| >Security Change Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>Security Change Candidate Cell Configuration | M |  | OCTET STRING | Includes the ltm-NoSecurityChangeID IE as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |