3GPP TSG-RAN WG3 #128																			R3-253666

Malta, MT, 19th – 23rd May 2025

Agenda item:	21.3

Source:	China Telecom

Title:	Discussion on Rate Control for XR

Document for:	Discussion and Decision

1	Introduction

The revised WI on XR for NR Phase 3 WID was agreed in RAN#107 meeting [1]. One of objectives is to specify MAC layer XR rate control for uplink congestion adaption as following.

-	Specify uplink congestion signalling [RAN2, RAN3]:

- Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion

XR rate control was discussed in RAN2 and RAN3 simulatensouly. In RAN2#129bis meeting, the following agreements were made:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

In RAN3#127bis meeting, the following agreements were achieved:

- Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control.

- Introduce the same indication in XnAP to inform target gNB.

In addition, the following open issues need to be further discussed:

- FFS on whether CN may inform gNB the recommended UL rate for specific QoS flows.

This paper will provide our views on the remaining issues on UL rate control.

2	Discussion

Since XR services may involve multiple distinct traffic flows, and some of these flows may not support codec adjustment, it is necessary to introduce an indication mechanism to inform the NG-RAN node which QoS flow(s) are subject to uplink rate control. Regarding the issue of whether the core network should inform the recommended data rate, we believe that the NG-RAN node should be responsible for determining the appropriate rate. The UE may initiate end-to-end bitrate adaptation with its peer at the application layer or through IMS. Furthermore, in the existing Release 15 RAN-assisted codec adaptation mechanism, the CN does not provide any recommendation information to the NG-RAN. Therefore, there is no need to introduce a recommended uplink rate for specific QoS flows over the NGAP interface.

Proposal 1: no need to introduce the recommended UL rate for specific QoS flows over NGAP interface

In the RAN2#129bis meeting, RAN2 preferred to define per-QoS flow level XR rate control using a MAC CE. The QoS flow identifiers and corresponding rate recommendations may be included in this newly defined MAC CE. During the online discussion, some companies raised concerns about the impact on the F1 interface. The network configures which QoS flows are subject to rate control via RRC signaling, which is transmitted from the gNB-CU to the UE. However, the gNB-DU does not have access to the rate control information and therefore cannot generate the rate control MAC CEs.From the RAN3 perspective, the gNB-CU could provide at least the necessary QoS flow information related to rate control to the gNB-DU. Any additional required information will depend on the progress made in RAN2. Additionally, both the gNB-CU and gNB-DU could support XR rate monitoring or congestion monitoring at a per-QoS flow level. And how to support per-QoS flow rate monitoring in gNB-DU side is up to implementation. Technically, the gNB-CU could also provide the per-QoS flow level recommended rate to the gNB-DU. With this information, the gNB-DU would be able to generate the new MAC CE format for XR rate recommendation or receive the MAC CE format for XR data rate queries.

Proposal 2: the gNB-CU could provide at least the QoS flows information for rate control information to the gNB-DU.

Proposal 3: the gNB-CU could also provide the per QoS flow level recommended rate to the gNB-DU.

3	Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: no need to introduce the recommended UL rate for specific QoS flows over NGAP interface

Proposal 2: the gNB-CU could provide at least the QoS flows information for rate control information to the gNB-DU.

Proposal 3: the gNB-CU could also provide the per QoS flow level recommended rate to the gNB-DU.

4	Reference

- RP-250107, Revised WID: XR (eXtended Reality) for NR Phase 3