3GPP TSG-RAN WG3 Meeting #128	             R3-253669

St Julian, Malta, 19-23 May 2025

Agenda Item:	21.3

Source:	CMCC

Title:	Support of Exposure of Available Data Rate in Non-Homogeneous Deployment

Document for:	Discussions

# Introduction

The following revised WID on XR (eXtended Reality) for NR Phase 3 [1] was approved at RAN Plenary meeting #107. The discussions on highlighted topics are presented in this paper.

The Rel-19 XR Phase 3 objectives are as follows: - Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]: - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG. - Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3]: - PDU set based handling; - ECN marking; - Burst Arrival Time reporting, if needed; - PSI Discard coordination, if needed; NOTE: No RAN2 impact from above items. - Specify uplink congestion signalling [RAN2, RAN3]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion. - Support of PDU set based QoS handling enhancement [RAN3]: - Support of DL PDU Set marking without PDU Set QoS - Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER). - QoS Handling when Traffic Characteristics Change Dynamically [RAN3] - TTNB and burst size to be provided over GTP-U - Support of exposure of available data rate [RAN3] Note: Coordination with SA2, as needed - Specify Core requirements related to the above objectives as necessary [RAN4].

In this contribution, we mainly focus on the discussion of support of available data rate in non-homogeneous deployment.

# Discussions

In the previous RAN3 127Bis meeting [2], the following agreements are made:

Introduce thresholds and direction to trigger NG-RAN monitor available data rate in NGAP/XnAP/F1AP/E1AP. Introduce “available data rate” in TS 38.425 and TS 38.415. FFS on non-homogeneous deployment.

For non-homogeneous deployment, it is likely that one gNB supports available data rate reporting, while the other gNB does not support available data rate reporting. Thus, it is important to indicate whether available data rate handling is supported.

A new IE “Available Data Rate Handling Indicator” shall be included in the PDU Session Resource Setup Response (Transfer), PDU Session Resource Modify Response (Transfer), Path Switch Request (Transfer), Handover Request Acknowledge (Transfer) in NGAP.

A new IE “Available Data Rate Handling Indicator” shall be included in the Handover Request Acknowledge, S-NODE Addition Request Acknowledge, S-NODE Modify Request Acknowledge in XnAP.

# Conclusions

In this submission, we discussed the support of exposure of available data rate  in non-homogeneous deployment in RAN3. The following proposals were made:

- A new IE “Available Data Rate Handling Indicator” shall be included in the PDU Session Resource Setup Response (Transfer), PDU Session Resource Modify Response (Transfer), Path Switch Request (Transfer), Handover Request Acknowledge (Transfer) in NGAP.

A new IE “Available Data Rate Handling Indicator” shall be included in the Handover Request Acknowledge, S-NODE Addition Request Acknowledge, S-NODE Modify Request Acknowledge in XnAP.

# References

- RP-240107, Revised WID on XR (eXtended Reality) for NR Phase 3

- Draft_MeetingReport_RAN3#127-bis_v20250419