3GPP TSG-RAN WG3 Meeting #128	R3-253673

St. Julien's, Malta, 19 - 23 May, 2025

Title:	Discussion on Inventory Procedure

Agenda Item:	16.2

Source:	China Unicom

Document for:	Discussion

- Introduction

During RAN3#127bis meeting, the agreement about A-IoT Inventory procedure can be found as the following:

About Inventory Procedures: - WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - Include the AIOTF Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - The Inventory Request Transfer IE, also includes the following: - Device Identification for Paging (to enable paging for single device, a group of devices, all devices) - Requested Service Area Information (A-IoT Area list and/or Reader list) - Inventory Assistance Information (Approximates Number of Target A-IoT Devices). - It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

In this contribution, we discuss open issue part of A-IoT  inventory operation.

- Discussion

The Correlation Identifier IE would be used if multiple AIOTFs initiate inventory requests toward the same gNB via the AMF. If the Correlation Identifier were embedded within Transfer IEs, the gNB would need to decode the Transfer IE to extract it, which will complicate the session management. By placing the Correlation Identifier outside the Transfer IE, the gNB can efficiently track and correlate requests from different AIOTFs without additional processing overhead.

On the other hand, the Correlation Identifier IE is critical for enabling the AIOTF to associate inventory responses with their corresponding requests. If the Correlation Identifier IE is located outside the Inventory-related Transfer IEs, intermediate nodes like the AMF can directly identify this IE without needing to parse the Transfer IE content. This is particularly important in indirect communication scenarios, where the AMF is responsible for routing responses back to the correct AIOTF.

Proposal 1: Change the WA to an agreement, include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

As defined in SA2 specifications (e.g., TS 23.xyz-020), assistance information from the AIoTF (Ambient IoT Function) to the A-IoT RAN node is essential for enabling the RAN to perform service operations effectively. For inventory procedures, the RAN requires parameters to allocate radio resources optimally.

The inclusion of the approximate D2R message size allows the gNB to optimize uplink (UL) resource allocation. By estimating the length of device IDs or other payloads, the gNB can pre-allocate appropriately sized resources, minimizing resource fragmentation or shortages. For example, if the D2R message size is expected to be large (e.g., due to extended device identifiers or additional metadata), the gNB can allocate contiguous UL resources, improving spectral efficiency and reducing retransmission overhead.

Furthermore, refered to TR 38.769,  the importance and necessity of assistance information for inventory operations has been studied. If the gNB is unaware of the expected D2R message size, it may assign uniform UL grants based on default assumptions. This could lead to collisions or resource contention if multiple devices transmit messages exceeding the allocated grant size. Including the "Estimate of Expected D2R Message Size" enables dynamic resource tailoring, reducing collision risks and enhancing reliability.

Therefore, the Inventory Assistance Information should includes the “Estimate of Expected D2R Message Size” as the optional IE.

Proposal 2: The Inventory Assistance Information should includes the “Estimate of Expected D2R Message Size” as the optional IE.

- Conclusion

In this paper, the following proposals are made:

Proposal 1: Change the WA to an agreement, include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

Proposal 2: The Inventory Assistance Information should includes the “Estimate of Expected D2R Message Size” as the optional IE.

- Reference

[1] 3GPP TR 38.769 V19.0.0, Study on solutions for Ambient IoT (Internet of Things) in NR