3GPP TSG-RAN WG3 Meeting #128	R3-253685

Malta, MT, May 19 - 23, 2025

Title: 	Discussion on continuous MDT

Source: 	CMCC

Agenda item: 	11.4

Document Type:	Discussion

- Introduction

SA5 reply an LS to RAN3 for the continuous MDT, so in this proposal, we provide our consideration based on the agreements and reply LS.

.2	Discussion

In R19, it has been agreed to use the existing MDT framework as a baseline for data collection from UE. But enhance it to support the continuous collection of MDT trace for AI/ML training in OAM.

In the previous discussion, two problems need to be addressed to guarantee the continuous MDT collection and association for a given UE:

- Problem A (measurement continuity): how to ensure that the same UE is selected for collection of MDT measurements during sessions in the same RRC state, across different RRC states and across RRC_Connected mobility

- Problem B (trace correlation): how to ensure that the TCE receiving the Trace Record files containing MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection process from the same data source (UE)

In the reply LS, SA5 provides the answers for these two problems:

For Measurement Continuity Problem, regarding RAN3 agreements: RAN3: “- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.” Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements. RAN3: “- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.” Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”. RAN3: “- A solution should minimise impacts on the current Management Based MDT framework.” Answer from SA5: SA5 agrees that the solution should minimise impacts on the current Management Based MDT framework. For Trace Correlation Problem, RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.” Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

  - Problem A (measurement continuity)

- Problem A (measurement continuity): how to ensure that the same UE is selected for collection of MDT measurements during sessions in the same RRC state, across different RRC states and across RRC_Connected mobility.

Based on the reply LS from SA5, for problem A, SA5 clarified that it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT, but SA5 needs RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

In the previous discussion, the Measurement Continuity Problem is how to ensure that the same UE is selected for collection of MDT measurements during sessions in the same RRC state, across different RRC states and across RRC_Connected mobility. For the problem “Measurement Continuity Problem across different RRC states” in reply LS, we think it can be clarified as OAM needs to ensure the same UE is selected for continuous MDT collection for management based MDT.

Proposal 1: For the problem “Measurement Continuity Problem across different RRC states” in reply LS, RAN3 clarifies as OAM needs to ensure the same UE is selected for continuous MDT collection for management based MDT.

Then, SA5 also expects that RAN3 could provide the configuration details as OAM requirements. From the reply LS, SA5 agrees that at least TR and TRSR could be used for reports correlation. To minimize the impact, we think the TR and TRSR can also be used to identify Management based MDT for continuous MDT. For example, configure the same TR for continuous MDT, and same TRSR for the same UE. With UE mobility, the source gNB can also signal the TR and TRSR to the target gNB to indicate the UE is selected for continuous MDT and needs to be configured with the same TR and TRSR for the subsequent MDT measurement collection.

Proposal 2: RAN3 agrees that the same TR and TRSR could be configured to the UE for continuous MDT collection, and could be transferred to the target node to indicate the UE is selected for continuous MDT for UE mobility.

  - Problem B (trace correlation)

- Problem B (trace correlation): how to ensure that the TCE receiving the Trace Record files containing MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection process from the same data source (UE).

According to the LS reply from SA5 in R3-253024, SA5 agrees to use at least TR and TRSR (which does not reveal UE identity) for the correlation of the management-based MDT reports at TCE. But SA5 would like RAN3 to elaborate the details of how to avoid overlapping.

From our understanding, as discussed in problem A, the same TR and TRSR will be assigned to the same UE to indicate the UE is selected for continuous MDT. So, different UEs will have at least different TRSR. If the received logged and immediate MDT measurements have the same TR and TRSR, TCE will understand that these reports are received from the same UE and can be associate together for continuous MDT. Regarding the overlapping issue mentioned in SA5 reply LS, we think it might not be very clear what kind of problem it is. It’s proposed that SA5 provide some clarifications on what the overlapping issue is.

Observation 1: For continuous MDT, the same TR and TRSR will be assigned to the same UE even during the serving gNB changes, in this way, when TCE received logged and immediate MDT measurements with same TR and TRSR, it will understand these reports are from the same UE for continuous MDT.

Observation 2: It’s not clear what’s the overlapping issue mentioned in SA5 reply LS.

Proposal 3: It’s proposed SA5 provides some clarifications on what is the overlapping issue.

3	Summary

In this contribution, the following proposals are made:

Proposal 1: For the problem “Measurement Continuity Problem across different RRC states” in reply LS, RAN3 clarifies as OAM needs to ensure the same UE is selected for continuous MDT collection for management based MDT.

Proposal 2: RAN3 agrees that the same TR and TRSR could be configured to the UE for continuous MDT collection, and could be transferred to the target node to indicate the UE is selected for continuous MDT for UE mobility.

Observation 1: For continuous MDT, the same TR and TRSR will be assigned to the same UE even during the serving gNB changes, in this way, when TCE received logged and immediate MDT measurements with same TR and TRSR, it will understand these reports are from the same UE for continuous MDT.

Observation 2: It’s not clear what’s the overlapping issue mentioned in SA5 reply LS.

Proposal 3: It’s proposed SA5 provides some clarifications on what is the overlapping issue.

4. Reference

- RP-242385 WID Enhancements for Artificial Intelligence (AI) Machine Learning (ML) for NG-RAN (ZTE, NEC), 3GPP RAN#105

- R3-253024 Reply LS to LS on Continuous MDT