3GPP TSG-RAN WG3 Meeting #128	             R3-253692

Malta, MT, May 19th- May 23th, 2025

Agenda Item:	16.2

Source:	CMCC

Title:	(TP to TS 38.300) Discussion on RAN Architecture for Ambient IoT

Document for:	Other

# 1	Introduction

In RAN3#127bis meeting, the discussion on RAN architecture aspects for Ambient IoT has made following agreements and remaining FFSs.

Architecture aspects: - Turn the WA to agreement, i.e., Including AIOTF information containers in NGAP. - Enhancing the existing NGAP interface management procedures. - Introducing an AIoT indicator in the NGAP Setup Request message. The detail of this indicator can be further discussed. - Turn previous “WA: Introduce a NGAP Class 1 Inventory Request procedure.” and “WA: Introduce a NGAP Class 1 Command Request procedure” to agreements. - Define the Reader Index as INTEGER (1..65536, …). - Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. - The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB. - It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

In this contribution, we will provide our views on RAN architecture aspects for interface management and A-IoT reader selection mechanism.

# 2	Discussion

### 2.1	Interface Management

In both direct and indirect path deployment, excluding the A-IoT related procedures over NGAP, RAN3 agreed to enhance the NG interface management procedures between the gNB and the AMF. The NG interface management procedure includes NG Setup, RAN Configuration Update, NG Reset and Error Indication procedure. Inside the NG interface management procedure, some mandatory IEs used in NR may not be suitable for A-IoT services. To mitigate backward compatibility risks induced by large-scale modifications to the presence of mandatory IEs, RAN3 agreed to introduce an optional A-IoT indicator IE in the existing NG interface management procedure. Considered the A-IoT RAN node needs to satisfy the operator’s requirements to support both "A-IoT only" and "A-IoT and NR Uu" deployment, the design of A-IoT indicator IE should consider the two deployments mentioned above. The A-IoT indicator IE can be designed as an ENUMERATED type including "A-IoT only" and "A-IoT and NR Uu". Once this IE is set to "A-IoT only" in the NG Setup Request message, A-IoT CN shall ignore al inapplicable mandatory IEs using in NR.

Proposal 1: The A-IoT indicator IE can be designed as an ENUMERATED type including "A-IoT only" and "A-IoT and NR Uu".

Proposal 2: Once the A-IoT indicator IE is set to "A-IoT only" in the NG Setup Request message, A-IoT CN shall ignore al inapplicable mandatory IEs using in NR.

As one of the remaining issues in last meeting, it is possible that the cancellation of inventory or command operation triggered by A-IoT CN or requested by gNB at any time. To make the whole control plane procedures designed for inventory and command complete, the A-IoT release procedure needs to be introduced to indicate the cancellation of an ongoing A-IoT procedure. Similar to the existing UE Context Release procedure, the A-IoT CN can trigger the release of ongoing A-IoT service, while the gNB can request the A-IoT CN to release the ongoing A-IoT service due to gNB generated reasons. Accordingly, it is suggested to introduce a class-2 A-IoT Service Release Request procedure initiated by gNB and a class-1 A-IoT Service Release procedure initiated by A-IoT CN.

Proposal 3: Introduce a class-2 A-IoT Service Release Request procedure initiated by gNB and a class-1 A-IoT Service Release procedure initiated by A-IoT CN.

### 2.2 A-IoT Reader Selection

In previous meeting, RAN3 held extensive discussions regarding the A-IoT reader selection mechanism. Meanwhile, considering the different information carried in Inventory Request message, the conclusions made for several cases on reader selection are summarized as follows:

Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area. Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers. Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list. Turn the following WA to agreement: Upon receiving only the reader list in Inventory Request, the gNB selects the readers as indicated by the reader list? The gNB selects the readers taking the reader list provided by CN into account in some cases.

Regarding the case that gNB receives both the area and the reader list in Inventory Request, we think this scenario is unlikely to exist since it is impossible for A-IoT CN to provide both information at the same time in Inventory Request message which will make the gNB confused if the readers within indicated area and the reader list provided by CN is not consistent.

Proposal 4: The A-IoT CN does not provide both the area and the reader list in Inventory Request at the same time.

Regarding the case that gNB only receives the reader list in Inventory Request, the gNB does not have rights to select the readers indicated by the reader list, and it only follow the guidance from A-IoT CN. The reader list provided by the A-IoT CN is selected on account of the reader’s location configured by the A-IoT CN. The principle for A-IoT CN selecting readers is providing a coverage assurance mechanisms which ensure that readers can perfectly cover the entire service area.

Proposal 5: Upon receiving only the reader list in Inventory Request, the gNB should follow the reader list provided by the A-IoT CN.

### 2.3 A-IoT Area Information

In previous meeting, RAN3 introduced the concept of A-IoT Area and made following agreements related to the A-IoT Area.

AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list. AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN. Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB.

As we already agreed to use a new term “A-IoT area”, to facilitate management, each reader will only serve within one A-IoT area. The AIOTF would be preconfigured with the supported A-IoT area of a gNB and the mapping relation between new A-IoT area and geographical area by the OAM. Upon receiving the requested service area consisting of a list of “A-IoT Areas”, the gNB can quickly find the suitable reader within the A-IoT Area. The remaining issue is how to decode the new area. From our view, this A-IoT area can be encoded as a new A-IoT Area ID. The new A-IoT Area ID can better identify the A-IoT area and would be uniformly designed for the AIOTF and gNB to perform A-IoT service effectively. The new A-IoT Area ID can refer to the existing TAI design. If the A-IoT area is encoded as GNSS, the gNB needs to record the GNSS information for the readers and translate the A-IoT Area information to search readers within the requested area, which will increase the implementation complexity of gNBs.

Proposal 6: RAN3 agrees to support that the new A-IoT Area is encoded as new A-IoT Area ID, which can refer to the existing TAI design.

Considered the location of reader, we already agreed the AIOTF obtains the A-IoT RAN information via OAM configuration, and FFS on signalling support. Currently, the A-IoT RAN Information includes served reader list, supported A-IoT areas and reader location. From our view, the above A-IoT RAN information is relative stable and the OAM configuration is enough to configure that information to the AIOTF. We do not see the necessity to use NG signalling to transfer between AIOTF and gNB.

Proposal 7: RAN3 does not support to use NG signalling to transfer A-IoT RAN Information between A-IoTF and gNB.

# 3	Conclusion

Based on the discussions mentioned above, in this contribution we provide following observations and proposals on RAN architecture aspects for Ambient IoT:

Proposal 1: The A-IoT indicator IE can be designed as an ENUMERATED type including "A-IoT only" and "A-IoT and NR Uu".

Proposal 2: Once the A-IoT indicator IE is set to "A-IoT only" in the NG Setup Request message, A-IoT CN shall ignore al inapplicable mandatory IEs using in NR.

Proposal 3: Introduce a class-2 A-IoT Service Release Request procedure initiated by gNB and a class-1 A-IoT Service Release procedure initiated by A-IoT CN.

Proposal 4: The A-IoT CN does not provide both the area and the reader list in Inventory Request at the same time.

Proposal 5: Upon receiving only the reader list in Inventory Request, the gNB should follow the reader list provided by the A-IoT CN.

Proposal 6: RAN3 agrees to support that the new A-IoT Area is encoded as new A-IoT Area ID, which can refer to the existing TAI design.

Proposal 7: RAN3 does not support to use NG signalling to transfer A-IoT RAN Information between A-IoTF and gNB.

# 4	Reference

[1] RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TS 23.369 v0.3.0, Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19);

# 5	Text Proposal to TS 38.300 BL CR

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

2Rx XR UE: two antenna port XR UE as specified in TS 38.101-1 [18].

A2X communication: A communication to support A2X services leveraging PC5 reference points. A2X services are realized by various types of A2X applications, i.e. BRID or DAA.

Aerial UE communication: functionality enabling Aerial UE function, as defined in 16.18.

A-IoT CN node: The core network node which the gNB connects to 5GC for A-IoT operation. More specifically, it is the AIOTF in case the gNB directly connects with the AIOTF, and it includes both the AMF and AIOTF in case the gNB connects with the AIOTF via AMF.

Air to Ground network: An NG-RAN consisting of ground-based gNBs, which provide cell towers that send signals up to an aircraft's antenna(s) of onboard ATG terminal, with typical vertical altitude of around 10,000m and take-off/landing altitudes down to 3000m.

BH RLC channel: an RLC channel between two nodes, which is used to transport backhaul packets.

Boundary IAB-node: as defined in TS 38.401 [4].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 16.xx	Support of Ambient IoT

### 16.xx.x1	Introduction

### 16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	As specified in TS 23.369 [xx], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF).

NOTE 1:	It is not expected a deployment will deploy both direct connectivity and indirect connectivity.

-	In this version of the specification A-IoT specific information is transported between the gNB and the A-IoT CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas or a list of readers. The A-IoT Area is identified by the A-IoT Area Identity.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the readers supported by the gNB.

The AIOTF may also be aware of the reader’s location by means of OAM configuration.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) are targeted for the inventory, which can be a single A-IoT device, multiple A-IoT devices or all A-IoT devices.

Editor’s Note 3:	content of the A-IoT Device Identification Requested is FFS.

The Inventory Request message also includes Requested Service Area Information (A-IoT area list and/or reader list) and may include Inventory Assistance Information. The Assistance Information may provide an Approximate number of Target A-IoT devices, expected D2R message size.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note 4:	Specification of further parameters of the Inventory Request message (e.g., other assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices,

### 16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

NOTE 4: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure, and includes a CN A-IoT device NGAP ID for A-IoT device which has already assigned a RAN A-IoT device NGAP ID.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU targeting the A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size, command type.

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response message including the AIOTF Identifier and the Correlation ID received in the Command Request and the RAN/CN A-IoT device NGAP ID pair.

NOTE 5:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>