3GPP TSG-RAN WG3 Meeting #128	             R3-253694

Malta, MT, 19th – 23th May 2025

Agenda Item:	16.2

Source:	CMCC

Title:	Discussion on Command Procedure and Signaling

Document for:	Discussion

# 1	Introduction

In last meeting, the following agreements and remaining issues on Command Procedure and Signalling are captured in minutes.

About Command Procedures: - Introduce a follow-on command indication in Inventory Request Transfer IE, in case of Command after inventory. - For command related NGAP messages, the Correlation Identifier IE is the same one as in the related Inventory procedure. - WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure. - WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages. - Include the AIOTF Identifier IE outside of the Command related Transfer IEs in all the command related messages. - The Command Request Transfer IE, also includes the following: - An A-IoT NAS PDU - Command Assistance Information: (Estimate of Expected D2R Message Size) - It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…) - Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory. - In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device. - The Device Association is included in the Command Request Transfer IE and Command Response Transfer IE. - It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

In this contribution, we will continue to provide our views on command procedure and signalling between A-IoT RAN and A-IoT CN to support command operation.

# 2	Discussion

During last meeting, RAN3 agreed to introduce a NGAP Class 1 Command Request procedure. Majority of companies thought that the command procedure is a per single device procedure, this point of view can also be demonstrated in the current SA2 specification.

TR 23.369-030 ### 5.2.2 AIoT Command service Three types of Command service operations are supported: Read, Write and Disable. An AF uses the Command service Read operation to retrieve information from a specific AIoT device. An AF uses the Command service Write operation to configure information into a specific AIoT device. An AF uses the Command service Disable operation to permanently disable the capability of the AIoT device(s) to transmit RF signals

It can be seen that both SA and RAN reach a common understanding that the command operation can only be sent by the A-IoT CN for a single device. Based on this principle, each Command Request procedure can only apply to a single device. As per device procedure, there is no need to distinguish the COMMAND RESPONSE message and COMMAND REPORT message. The command results would be carried in the COMMAND RESPONSE message once the gNB retrieves the results from the A-IoT device(s). To allow AF providing multiple identical command operations towards multiple AIoT device(s), RAN3 needs to support AIOTF initiates parallel Command Request procedures for same command service operations towards multiple A-IoT devices to the gNB, but it is obviously unnecessary to aggregate the command service operations within one Command message. Therefore, we propose to turn the WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure. into agreement.

Proposal 1: It is proposed to turn the “WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure.” into agreement.

In addition, three command service operation types, i.e., read, write, and disable, are explicitly supported in SA2 specification. From our point of view, these service type indicators must be made visible to gNB through A-IoT CN exposure, as the command type enables critical gNB scheduling optimizations. With this information, gNB can implement service-specific resource allocation strategies. For read operation, gNB shall allocate uplink resources to retrieve the content from A-IoT device. For write or disable operation, gNB will only need provision minimal uplink resources for the success or failure acknowledgment signalling. The AIOTF can interpret service type provide by AF and send the corresponding command service type indicator to gNB via COMMAND REQUEST message.

Proposal 2: The command service type indicator (Read/Write/Disable) needs to be included in COMMAND REQUEST message.

Another issue worth reconfirming is the location of the Correlation Identifier IE in all the command related messages. Comparable to the same logic designed for Inventory related messages, the Correlation Identifier IE needs to be included in both inside and outside of the Command related Transfer IEs in all the Command related messages. The Correlation Identifier IE inside the Command Request Transfer IE is assigned by the AIOTF to denote and identify the A-IoT session within an AIOTF, and the Correlation Identifier IE outside the Command Request Transfer IE is used by AMF to identify the A-IoT session among parallel A-IoT command operations in the indirect deployment. Therefore, we propose to turn the previous WA to the agreement in this meeting.

Proposal 3: It is proposed to turn the “WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.” into agreement.

As the command operation discussed in this release is always after inventory operation, an A-IoT device association mechanism shall be established between the gNB and AIOTF to identify the A-IoT device which can be analogous to the AMF/RAN UE NGAP ID association in NG interface. During last meeting, RAN3 already reached the following agreement “Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.” It can be seen that it is not enough to identify the A-IoT device only using RAN NGAP Device ID since multiple gNB may assign the same RAN NGAP Device ID. Thus, another CN NGAP Device ID assigned by AIOTF is required to construct a NGAP Device ID pair accompanied with RAN NGAP Device ID. This construction eliminates redundant transmission of lengthy A-IoT device identifiers by implementing the following protocol sequence: The gNB can allocate a RAN NGAP Device ID within the Inventory Report Transfer IE in the INVENTORY REPORT message, while the AIOTF subsequently pairs with a unique CN NGAP Device ID within the Command Request Transfer IE in the COMMAND REQUEST message. Furthermore, to allow parallel Command Request procedures for different devices between gNB and AMF for same A-IoT Correlation Identifier in indirect deployment, the AMF needs to identify Command Request procedures for different devices. Hence, there will be another identifier used to distinguish the different devices with the same A-IoT Correlation Identifier. To avoid introducing extra identifier to signify the session for AIOTF, it is proposed to place the Device Association outside of the Command Related Transfer IEs for differentiation.

Proposal 4: It is proposed to agree to introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.

Proposal 5: It is proposed to place the Device Association outside of the Command Related Transfer IEs to differentiate parallel Command Request procedures for different devices between gNB and AMF with the same A-IoT Correlation Identifier.

# 3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on the procedure and procedure between A-IoT RAN and A-IoT CN to support command operation:

Proposal 1: It is proposed to turn the “WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure.” into agreement.

Proposal 2: The command service type indicator (Read/Write/Disable) needs to be included in COMMAND REQUEST message.

Proposal 3: It is proposed to turn the “WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.” into agreement.

Proposal 4: It is proposed to agree to introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.

Proposal 5: It is proposed to place the Device Association outside of the Command Related Transfer IEs to differentiate parallel Command Request procedures for different devices between gNB and AMF with the same A-IoT Correlation Identifier.

# 4	Reference

[1] RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TS 23.369 v0.3.0, Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19);