3GPP TSG-RAN WG3 Meeting #128	             R3-253695

Malta, MT, 19th – 23th May 2025

Agenda Item:	16.3

Source:	CMCC

Title:	Discussion on A-IoT device location reporting

Document type:	Discussion

# 1	Introduction

During last meeting, we agreed the following agreements on the topic of A-IoT device location reporting.

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report. AIoT RAN node sends the Inventory Report aggregated per Reader. “Global gNB ID + Reader index” is used to uniquely identify the Reader globally. Reader indexes and the gNB ID are mandatory included in Inventory Report Transfer IE. Continue to discuss the open issues....

In this contribution, we will continue to provide our views on A-IoT device location reporting.

# 2	Discussion

From the WID, the scope of RAN3 restricts the granularity for device location reporting at reader ID granularity. As the location of the A-IoT device, the AIOTF already be aware of the reader ID received through the existing A-IoT Inventory Report procedure over NGAP. Since the command is always initiated after the inventory and the reader within a gNB is quite stable, it is unnecessary for gNB to provide the A-IoT device’s location at reader ID granularity to AIoTF in Command messages.

Proposal 1: The gNB does not need to provide the A-IoT device’s location at reader ID granularity to AIoTF in Command messages.

Another question is whether it is possible to contain multiple Reader IDs for a single device in the Inventory Report message. From our point of view, the answer is no. Regard to single A-IoT device, it only responds to one reader at one time as RAN2 already reached a consensus that they will not specify the multi-reader scenario and they will not specify something specific for this case and rely on the implementation. From our view, RAN3 needs to follow RAN2’s conclusion and focus on the readers non-overlapping scenario. Thus, each device ID can only be associated to one reader ID in the Inventory Report message.

Proposal 2: Each device ID can only be associated to one reader ID in the Inventory Report message.

Currently, the A-IoT service defined in SA2 only supports Inventory service and Command service, there is currently no demand to introduce a new service directly obtaining the A-IoT device location. Thus, we think it is unnecessary to introduce a dedicated locating procedure for A-IoT device to solely report device ID.

Proposal 3: It is unnecessary to introduce a dedicated locating procedure for A-IoT device to solely report device ID.

Regarding the details of the location of reader, it can be GNSS information or refer to the existing TRP Position and TRP High Accuracy Access Position defined in TS 38.455 shown below. As SA5 already asked us to inform them further potential new OAM requirements, we can send a reply LS to SA5 in this meeting to inform our A-IoT new requirements on the location of reader.

Proposal 4: The details of the location of reader can be GNSS information or refer to the existing TRP Position and TRP High Accuracy Access Position defined in TS 38.455.

Proposal 5: Send a LS to SA5 to inform our A-IoT new requirements on the location of reader.

# 3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on A-IoT device location reporting:

Proposal 1: The gNB does not need to provide the A-IoT device’s location at reader ID granularity to AIoTF in Command messages.

Proposal 2: Each device ID can only be associated to one reader ID in the Inventory Report message.

Proposal 3: It is unnecessary to introduce a dedicated locating procedure for A-IoT device to solely report device ID.

Proposal 4: The details of the location of reader can be GNSS information or refer to the existing TRP Position and TRP High Accuracy Access Position defined in TS 38.455.

Proposal 5: Send a LS to SA5 to inform our A-IoT new requirements on the location of reader.

# 4	Reference

[1] RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TS 23.369 v0.3.0, Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19);