3GPP TSG-RAN WG3 Meeting #128                                                                  			R3-253698

Malta, MT, May 19th - May 23th, 2025

Agenda Item:	8.3

Source:	CMCC, Ericsson, ZTE, CATT

Title:	Discussion on UAV regulation

Document for:	Discussion

# Introduction

In previous meeting, RAN3 discussed the UAV regulation on Rel-18 UAV UEs. During RAN3#127 meeting, RAN3 sent an LS to SA2, asking further clarification on whether the CN needs to indicate derived altitude thresholds and reporting periodicity for the aerial UE’s altitude reporting. The content of LS is excerpted below.

RAN3 thanks SA2 for the reply LS on UAV regulation. RAN3 further discussed the topic based on the feature provided by SA2 regarding the applicability of the “NEF Assisted In-flight Monitoring” in clause 5.12.3 of TS 23.256 to implement a network-based solution, which may require a UAV UE to report its own flight status information, and the NG-RAN then reports the received information to the AMF over NG. It is RAN3 understanding that NG-RAN can provide the UAV UE altitude information to CN. The three kinds of report type, i.e., periodical, threshold-based, event-based, can be supported to report altitude information based on NG-RAN node implementation. With regards to the “node-level signalling” solution configuration in clause 5.16.2, RAN3 would like to ask for further feedback from SA2 on whether the CN needs to indicate derived altitude thresholds and reporting periodicity for the aerial UE's altitude reporting.

In this meeting, RAN3 receive the reply LS from SA2 and the reply is excerpted below:

SA2 Reply: SA2 thanks RAN3 for the LS on UAV regulation for further clarification. SA2 would like to provide the following reply. Question1: With regards to the “node-level signaling” solution configuration in clause 5.16.2, RAN3 would like to ask for further feedback from SA2 on whether the CN needs to indicate derived altitude thresholds and reporting periodicity for the aerial UE's altitude reporting. SA2 reply: For SA2 perspective, the Core Network may indicate the derived altitude thresholds and reporting periodicity for the aerial UE's altitude reporting. This aspect is further clarified in the attached CRs (#0199 and #0194 for TS 23.256). Additionally, SA2 has specified that USS can benefit from knowing when altitude was reported from the UE and would like to check whether RAN3 has reached the agreement on NG-RAN providing a timestamp for UAV UE’s altitude information (see Step 2b in clause 5.16.3 of TS 23.256). #---excerpt from clause 5.16.3 of TS 23.256--- 2b. After receiving a report with the altitude information from the UE, the NG-RAN node should, for instance, add a time stamp indicating when the altitude information has been received from the UE; the NG-RAN sends the UE's altitude information report in an N2 message to the AMF. #--- 2. Actions: To RAN3 group ACTION: SA2 asks RAN3 to consider the attached CRs and provide feedback on timestamping of UAV UE altitude information reports from RAN3 perspective.

From the reply LS, SA2 already provides its own reply to the RAN3 question and waits for the RAN3 feedback on the attitude towards introducing time stamp for aerial UEs.

# Discussion

As stated in the SA2 reply LS, the Core Network may indicate the derived altitude thresholds and reporting periodicity for the aerial UE's altitude reporting. In SA2 latest CR, it gives a clear clarification on how the CN configures above information to RAN via the Location Report Control procedure. As the attached CR #0199 further clarifies as follows:

4b. The AMF sends the corresponding N2 messages (LOCATION REPORTING CONTROL message, see clause 8.12.1.2 of TS 38.413 [24]) to all applicable NG-RAN node(s), including the derived altitude thresholds and optionally reporting periodicity.

SA2 already confirmed that the Core Network can indicate the derived altitude thresholds and optionally reporting periodicity. The corresponding description on SA2 CR needs to be reflected in the TS 38.413. Accordingly, a new Aerial UE Flight Information Reporting Control IE, including the derived higher and lower altitude thresholds, reporting periodicity and the aerial UE reporting reference ID, is introduced within LOCATION REPORTING CONTROL message. Additionally, a new event type for obtaining the aerial UE flight information also needs to be added in Location Reporting Request Type IE.

Some of the changes on TS 38.413 are excerpted below:

#### 9.2.11.1 LOCATION REPORTING CONTROL This message is used by the AMF to request the NG-RAN node to report the location of the UE. Direction: AMF NG-RAN node IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. AMF UE NGAP ID; M; 9.3.3.1; YES; reject. RAN UE NGAP ID; M; 9.3.3.2; YES; reject. Location Reporting Request Type; M; 9.3.1.65; YES; ignore. #### 9.3.1.65 Location Reporting Request Type This IE indicates the type of location request to be handled by the NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Event Type; M; ENUMERATED (direct, change of serving cell, UE presence in the area of interest, stop change of serving cell, stop UE presence in the area of interest, cancel location reporting for the UE, … change of serving cell and UE presence in the area of interest, report the Aerial UE flight information, cancel the Aerial UE flight information reporting); -. Report Area; M; ENUMERATED (cell, …); -. Area of Interest List; 0..1; -. >Area of Interest Item; 1..<maxnoofAoI>; -. >>Area of Interest; M; 9.3.1.66; -. >>Location Reporting Reference ID; M; 9.3.1.76; -. Location Reporting Reference ID to be Cancelled; C- ifEventTypeisStopUEPresinAoI; Location Reporting Reference ID 9.3.1.76; -. Additional Location Information; O; ENUMERATED (Include PSCell, ...); YES; ignore. Additional Cancelled Location Reporting Reference ID List; 0..1; YES; reject. >Additional Cancelled Location Reporting Reference ID Item; 1..<maxnoofAoIMinusOne>; -. >>Location Reporting Reference ID to be Cancelled; M; Location Reporting Reference ID 9.3.1.76; -. Aerial UE Flight Information Reporting Control; O; 9.3.1.yy; YES; ignore. #### 9.3.1.yy Aerial UE Flight Information Reporting Control This IE indicates Aerial UE flight information reporting information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Aerial UE Reporting Reference ID; M; Location Reporting Reference ID 9.3.1.76. Higher Altitude Threshold; M; Altitude 9.3.1.ZZ; Indicates the higher altitude threshold information for the Aerial UE reporting.. Lower Altitude Threshold; M; Altitude 9.3.1.ZZ; Indicates the lower altitude threshold information for the Aerial UE reporting.. Reporting Periodicity; O; ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, …}; Indicates the periodicity of the Aerial UE reporting..

Proposal 1: Introduce a new Aerial UE Flight Information Reporting Control IE, including the derived higher and lower altitude thresholds, reporting periodicity and the aerial UE reporting reference ID, within LOCATION REPORTING CONTROL message.

Proposal 2: Add a new event type for obtaining the aerial UE flight information in Location Reporting Request Type IE.

Besides, as discussed in February meeting, the existing RRC specification already supports the NG-RAN node to collect the altitude information of aerial UE. The Location Report procedure can be reused to perform altitude reporting, so the altitude information for aerial UE needs to be included in User Location Information IE in LOCATION REPORT message.

Additionally, SA2 wants to check with RAN3 whether there is an agreement on NG-RAN providing a timestamp for aerial UE’s altitude information. Currently, there is no conclusion made in RAN3 to introduce a timestamp to indicate when the aerial UE flight information is received, but this timestamp may be beneficial for USS to know the exact time when the altitude of aerial UE exceeds the configured threshold for UAV regulation purpose. Thus, both the altitude information and the corresponding time stamp can be included in a newly introduced Aerial UE Flight information Reporting IE in NR User Location Information IE.

Observation 1: The timestamp may be beneficial for USS to know the exact time when the altitude of aerial UE exceeds the configured threshold for UAV regulation purpose.

Proposal 3: Introduce a timestamp for aerial UE’s flight information reporting to indicate when the aerial UE flight information is received.

Proposal 4: Both the altitude information and the corresponding time stamp can be included in a newly introduced Aerial UE Flight information Reporting IE in NR User Location Information IE.

In addition, another CR from SA2 already agreed to introduce a new mechanism to instruct an aerial UE to stop the altitude reporting. As stated in CR#0194, regard to the 3GPP-defined signaling, the NGAP signaling needs to be enhanced to support the feature.

0. The UAV UE is requested to perform altitude information reporting as specified in clause 5.16.2, and the UAV UE performs its altitude information reporting as specified in clause 5.16.3. 1. In case the UAV UE fails to report the altitude information and/or the NG-RAN to receive the altitude information, the UAS NF/NEF or the USS may decide to stop/update the UAV UE’s altitude reporting. Option B: 3GPP-defined signalling. 2b. The NG-RAN node sends an N2 message(LOCATION REPORTING FAILURE INDICATION message, see clause 8.12.2.2 of TS 38.413 [24]) to the AMF with information about the configured altitude information reporting. … 7b-8b. The AMF sends the corresponding N2 message (LOCATION REPORTING CONTROL message, see clause 8.12.1.2 of TS 38.413 [24]) to the NG-RAN node, and the NG-RAN triggers the release of the configured altitude reporting for the UAV UE, thereby stop the altitude reporting, as specified in TS 38.331 [23]. NOTE X3: Aspects related to AMF informing NG-RAN regarding start/stop of UAV UE’s altitude reporting are specified in TS 38.413 [24].

It is noted that in case the aerial UE fails to report the altitude information, the NG-RAN node sends the LOCATION REPORTING FAILURE INDICATION message to the AMF including a failure indication to indicate the failure of aerial UE flight information reporting. After the 5GC knows the failure, the USS would be informed and make the decision on whether to stop UAV UE’s altitude reporting. If the USS decides to stop altitude reporting, the AMF would be informed and sends the LOCATION REPORTING CONTROL message to the NG-RAN to release of the configured altitude reporting for the aerial UE. Accordingly, besides the event type for obtaining the aerial UE flight information, the Location Reporting Request Type IE needs to add another new event type for cancellation of the flight information reporting for the aerial UE.

Proposal 5: Introduce a failure indication to indicate the failure of UAV UE flight information reporting in the LOCATION REPORTING FAILURE INDICATION message.

Proposal 6: Add a new event type for cancellation of the flight information reporting for the aerial UE in Location Reporting Request Type IE.

# Conclusions

In this contribution, we discuss the potential impact on RAN spec based on the reply LS from SA2 and provide our feedback on time stamp information of aerial UE flight information reports. The corresponding stage2 and stage3 CRs are provided in R3-25xxxx/R3-25xxxx/R3-25xxxx. The following observation and proposal are listed below:

Observation 1: The timestamp may be beneficial for USS to know the exact time when the altitude of aerial UE exceeds the configured threshold for UAV regulation purpose.

Proposal 1: Introduce a new Aerial UE Flight Information Reporting Control IE, including the derived higher and lower altitude thresholds, reporting periodicity and the aerial UE reporting reference ID, within LOCATION REPORTING CONTROL message.

Proposal 2: Add a new event type for obtaining the aerial UE flight information in Location Reporting Request Type IE.

Proposal 3: Introduce a timestamp for aerial UE’s flight information reporting to indicate when the aerial UE flight information is received.

Proposal 4: Both the altitude information and the corresponding time stamp can be included in a newly introduced Aerial UE Flight information Reporting IE in NR User Location Information IE.

Proposal 5: Introduce a failure indication to indicate the failure of UAV UE flight information reporting in the LOCATION REPORTING FAILURE INDICATION message.

Proposal 6: Add a new event type for cancellation of the flight information reporting for the aerial UE in Location Reporting Request Type IE.

# References

- R3-253020, Reply LS on clarification of UAV regulation from RAN3;

- 3GPP TS 23.256, Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking;

- 3GPP TS 38.413, NG-RAN; NG Application Protocol (NGAP);