3GPP TSG RAN WG3 Meeting #128	R3-253703

Malta, ML, 19 – 23 May, 2025

Agenda item:	19.2

Source: 	Huawei, China Telecom, China Unicom

Title: 	Further discussion on remaining open issues for SBFD

Document for:	Discussion

1. Introduction

In the RAN3#127-bis meeting, there were agreements about SBFD related information exchange, listed as follow:

Introduce NZP-CSI-RS resource configuration in Served Cell Information NR IE of XnAP and F1AP specifications, which refers to the NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE defined in TS 38.331. Introduce new procedure to provide the strongest DL beam information and the CLI-mitigation request over XnAP and F1AP. gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Neighbour Cell Information List IE in gNB-CU Configuration Update message. Exchange of SBFD configuration The SBFD time and frequency location configuration with cell granularity in Served Cell Information IE is introduced in F1AP F1 SETUP REQUEST message and F1AP gNB-DU CONFIGURATION UPDATE message The SBFD time and frequency location configuration with cell granularity in the Neighbour Cell Information List IE is introduced in F1AP gNB-CU CONFIGURATION UPDATE message Exchange of measurement configuration For exchange of NZP CSI-RS over F1, to put a new IE in Served Cell Information IE from gNB-DU to gNB-CU, and in Neighbour Cell Information List IE from gNB-CU to gNB-DU with reference to RAN2 definition. In order to transfer the SSB info from gNB-CU to gNB-DU, to introduce an explicit IE in the Neighbour Cell Information List IE. Exchange of measurement result & Exchange of CLI-mitigation request To introduce a class 2 procedure, with tentative message name “CLI MEASUREMENT UPDATE”, in which both the strongest DL beam information and the CLI-mitigation request could be included as two optional IEs. RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs in Served Cell Information NR IE.

In this contribution, we will further discuss the remaining issues on SBFD [1] related information exchange.

2. Discussion

Issue 1: The name of new procedure and new message

In the last meeting, the new class 2 procedure was agreed to indicate the strongest DL beam information and the CLI-mitigation request, while the name for the new procedure and message has not been final decided. In the current version of XnAP and F1AP BL CRs, the procedure is temporarily named as CLI Measurement Reporting procedure, the message is temporarily named as CLI MEASUREMENT UPDATE message. Similar as Notification Control Indication procedure, Access And Mobility Indication procedure, and RACH Indication procedure, the CLI MEASUREMENT is not trigged by additional matching Class 1 Initiation procedure. Thus, we think the new procedure can be named as CLI Measurement Indication procedure, the new message should be named as CLI MEASUREMENT INDICATION message.

Proposal 1: the new Class 2 procedure is named as CLI Measurement Indication procedure, the new message is named as CLI MEASUREMENT INDICATION message.

Issue 2: The detail of stage3 BL CR

In the current version of XnAP and F1AP BL CRs, the NZP CSI-RS Resource Indication(CRI) is encoded to integer with value from 1 to 64, which is descripted as Strongest DL NZP CSI-RS beam information, as following:

XnAP:

| >>NZP CSI-RS Resource Indication | O |  | INTEGER (1..64) | Strongest DL NZP CSI-RS beam information | – |  |
| --- | --- | --- | --- | --- | --- | --- |

F1AP:

| >>CRI | O |  | INTEGER (1..64) | Strongest DL NZP CSI-RS beam information |
| --- | --- | --- | --- | --- |

And for the NZP CSI-RS Resources Configuration, which is encoded to OCTET STRING including the NZP-CSI-RS-Resource IE in RRC specification, in which the value of NZP-CSI-RS-ResourceId is a integer from 0 to 191:

#### – NZP-CSI-RS-ResourceId The IE NZP-CSI-RS-ResourceId is used to identify one NZP-CSI-RS-Resource. NZP-CSI-RS-ResourceId information element -- ASN1START -- TAG-NZP-CSI-RS-RESOURCEID-START NZP-CSI-RS-ResourceId ::= INTEGER (0..maxNrofNZP-CSI-RS-Resources-1) -- TAG-NZP-CSI-RS-RESOURCEID-STOP -- ASN1STOP

As can be seen, it is unclear what the value of CRI means, there would be ambiguity whether the CRI value is NZP-CSI-RS-ResourceIndex-r18 (integer from 0 to 63) or nzp-CSI-RS-ResourceId (integer from 0 to 191).

Observation 1: There is ambiguity for what value of CRI IE represent in current XnAP and F1AP BL CRs.

According to the LS from RAN1 [5], the CSI-RS Resource Indicator should be the relative index of the CSI-RS resources within a set of resources:

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

Proposal 2: Update the description of NZP CSI-RS Resource Indication IE in XnAP CR and CRI IE in F1AP CR to clarify the value is the relative index of the CSI-RS resources within a set of resources.

In additional, in the current F1AP BL CR, the SSB configuration for one neighbour cell is encoded to include the MeasTimingList contained in the MeasurementTimingConfiguration message:

| >>SSB Resource Configuration |  |  | OCTET STRING | Includes the MeasTimingList contained in the MeasurementTimingConfiguration message as defined in 38.331 [10]. | - | - |
| --- | --- | --- | --- | --- | --- | --- |

However, the MeasTimingList IE would include multiple SSB configurations of all the cell of the neighbour cell, which will cause redundancy information from gNB-CU to gNB-DU, and the gNB-DU need to determine which SSB configuration is belong to the cell indicated by NR CGI IE in Neighbour Cell Information List Item IE.

MeasTimingList ::= SEQUENCE (SIZE (1..maxMeasFreqsMN)) OF MeasTiming MeasTiming ::= SEQUENCE { frequencyAndTiming SEQUENCE { carrierFreq ARFCN-ValueNR, ssbSubcarrierSpacing SubcarrierSpacing, ssb-MeasurementTimingConfiguration SSB-MTC, ss-RSSI-Measurement SS-RSSI-Measurement OPTIONAL } OPTIONAL, ..., [[ ssb-ToMeasure SSB-ToMeasure OPTIONAL, physCellId PhysCellId OPTIONAL ]] }

Thus, the SSB Resource Configuration IE should be updated to include the MeasTiming contained in the MeasurementTimingConfiguration message.

Proposal 3: Update the description of SSB Resource Configuration IE in F1AP CR to include the MeasTiming contained in the MeasurementTimingConfiguration message.

Other miscellaneous issues

During last meeting, a set of BL CRs were agreed, and BL CRs to 38.401 and 38.300 were also agreed to basically capture the general framework of Cross Link Interference (CLI) in SBFD, for which Cross Link Interference related information of cells could be exchanged between neighbour gNBs, here the CLI related information may include the strongest beam info and possibly the CLI mitigation request.

Observation 2: CLI related information may include the strongest beam info and possibly the CLI mitigation request.

While in BL CR to 38.401, the exchange of CLI information is restricted to the precondition of CLI (inference) being detected, which is not necessary.

Proposal 4: To remove the precondition of CLI (inference) being detected before exchanging the CLI related information.

Corresponding stage 2 and stage 3 TPs could be seen in [2] [3] [4].

3. Conclusion

In this contribution, we analyzed the specification impacts of gNB-gNB mitigation. The following observation and proposals were provided:

Proposal 1: the new Class 2 procedure is named as CLI Measurement Indication procedure, the new message is named as CLI MEASUREMENT INDICATION message.

Observation 1: There is ambiguity for what value of CRI IE represent in current XnAP and F1AP BL CRs.

Proposal 2: Update the description of NZP CSI-RS Resource Indication IE in XnAP CR and CRI IE in F1AP CR to clarify the value is the relative index of the CSI-RS resources within a set of resources.

Proposal 3: Update the description of SSB Resource Configuration IE in F1AP CR to include the MeasTiming contained in the MeasurementTimingConfiguration message.

Observation 2: CLI related information may include the strongest beam info and possibly the CLI mitigation request.

Proposal 4: To remove the precondition of CLI (inference) being detected before exchanging the CLI related information.

4. Reference

[1]. RP-241614, WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2]	R3-253704,	(TP to BL CR for 38.423 and 38.420) introduction of Evolution of NR duplex operation Sub-band full duplex (SBFD), Huawei

[3]	R3-253705,	(TP to BL CR for 38.401) introduction of Evolution of NR duplex operation Sub-band full duplex (SBFD), Huawei, China Telecom, China Unicom

[4]	R3-253706,	(TP to BL CR for 38.473 and 38.470) introduction of Evolution of NR duplex operation Sub-band full duplex (SBFD) [5]. R1-2407533, LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation, Huawei