3GPP TSG-RAN WG3 Meeting #128                                                        R3-253719

St Julian’s, Malta, 19-23 May 2025

Agenda item:	 11.3

Source:		Rakuten

Title:		Multiple CCO Issues in AI/ML-based Coverage and Capacity Optimization

Document for:		Discussion and Decision

# 1	Introduction

In the paper, we present some scenarios where multiple CCO issue can be predicted/detected in the participating gNBs and what are the options of handling them.

# 2	Discussion

# In the legacy CCO mechanism:

Step 1: The gNB-CU detects the CCO issue (i.e., either coverage issue or cell edge capacity issue) and affected cells/SSBs;

Step 2: The gNB-CU sends the CCO Assistance Information IE (within the GNB-CU CONFIGURATION UPDATE message over F1) containing the CCO issue type and affected cells/SSBs to the gNB-DU;

Step3: The gNB-DU uses the received CCO Assistance Information IE to determine the new coverage state (CCO state) for the affected gNB-DU’s cells/SSBs;

Step 4: The gNB-DU sends the Coverage Modification Notification IE (within the GNB-DU CONFIGURATION UPDATE message over F1) containing the new CCO state for the affected gNB-DU’s cells/SSBs to the gNB-CU;

Step 5: The gNB-CU sends the Coverage Modification List IE (within the NG-RAN NODE CONFIGURATION UPDATE message over Xn) containing the detected CCO issue type and the new CCO state for the affected gNB-DU’s cells/SSBs to the neighbour gNB to allow for the neighbour gNB to determine a new CCO state for its own cells/SSBs that best matches the new CCO state of the gNB-CU’s cells/SSBs resulting from the detected CCO issue.

For the AIML-based CCO mechanism:

-	Step 0: AIML training entity generates predicted CCO issue

-	Step 1: gNB-CU predicts the CCO issue.

-	Step 2: gNB-CU sends the predicted CCO issue to gNB-DU.

-	Step 3: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

-	Step 4: gNB-DU sends the future coverage status to gNB-CU.

-	Step 5: gNB-CU sends the detected CCO issue and the new CCO state for the affected gNB-DU’s cells/SSBs to neighbour gNBs (say gNB2) along with the timing information of the future CCO state.

- 	Step 7: gNB2-CU sends this information to gNB2-DU which decides its own coverage/capacity configuration (future CCO state) in response to gNB1-DU’s future CCO state

For the sake of discussion, we define participating entities comprising of the (source) gNB where the CCO issue has been detected/predicted, and the gNB(s) which are neighbouring to the source gNB which end up taking a reactionary measure by changing its CCO state in response to the CCO state change of the source gNB.

Since there is time lag between the prediction at the source CU of the CCO issue and the time at which CCO issue is predicted to manifest, there are chances that, in the interim, another CCO issue could get predicted or detected (legacy) by the same gNB, by the neighbouring gNB, or by a third gNB altogether.

Observation 1	It is possible that a new CCO issue may be detected/predicted at the source gNB or a neighbouring node or a third gNB between the time at which the CCO issue was predicted and the time at which CCO issue is predicted to manifest.

Some of these cases are listed below, it is not an exhaustive list, but we hope to lay down principles for addressing a conflict arising between predicted issues and detected (or another predicted) issue.

Note1: While, it is implementation dependent, we also assume that gNB1-DU might apply the future CCO state little before the predicted time of the CCO issue. This will avoid the CCO issue (for accurate predictions) from being detected also.

Note2: While, it is implementation dependent, we also assume that gNB2-DU might apply the future CCO state at the same (or similar) time as the time when gNB2-DU applies the future CCO state. However, we do not recommend strict synchronization between the two gNBs.

For simplicity of discussion, let’s assume the following notations:

t0 = current time

t0+p = the time at which the CCO issue is predicted to occur at CU1

t0+a = the time at which gNB1-DU and gNB2-DU apply their respective future CCO state

For added simplicity, we may assume t0+p and t0+a are close together.

With the above assumptions in place, following are some of the scenarios where conflicts could occur. In the examples., we have considered the co-existence of one predicted CCO issue, and one detected CCO issue, but the underlying principles are applicable for multiple predicted CCO issues and multiple detected CCO issues occurring within a short time to each other.

Case 1: gNB1 detects an issue ahead of the predicted CCO issue time (issue is detected between t0 and t0+p)

Case 2: gNB2 detects an issue ahead of the future CCO state time of gNB1 and gNB2 (issue is detected between t0 and t0+a)

Case 3: gNBx detects an issue ahead of the predicted CCO issue time (issue is detected between t0 and t0+p)

Our first principle relies on the fact that detected issue is more important than the predicted issue. For each case of detected CCO issue, we propose that the CCO state in response to the detected CCO issue overrides the actions in response to the predicted CCO state.

- We propose that the CCO state in response to the detected CCO issue overrides the actions in response to the predicted CCO state.

Multiple issues can be detected/predicted at different instances of time at the participating entities, or at gNBs which influence the participating entities. To effectively handle this, we propose the each gNB independently handles the CCO issues in the order in which it is either detected/predicted/notified. We feel this is the right approach so as to avoid signalling complexity of added synchronization.

Proposal 2	We propose that the issue be tackled in the order in which they are detected, predicted or notified to occur.

For Case 1, if gNB1-CU detects an issue between t0 and t0+p

In this case, we may assume that gNB1-DU has not yet applied the proposed configuration change which it had derived in response to the predicted CCO issue, however, that change might no longer be valid. The legacy steps may follow:

- gNB1-CU informs the gNB1-DU of the detected CCO issue.

- gNB1-DU determines and applies the new CCO state and informs gNB1-CU

- gNB1-CU informs gNB2-CU of currently detected CCO issue and the CCO state applied by the gNB1-DU in response to the CCO issue detected by gNB1-CU.

- gNB2-CU forwards the information received from the gNB1-CU to gNB2-DU

- gNB2-DU determines and applies the new CCO state in response to the latest information and informs the gNB2-CU

After this, gNB1-CU reviews the earlier prediction, and if it is still valid for time t0+p, then it takes no further action. This implies that gNB1-DU and gNB2-DU will proceed to apply their respective future CCO states at t0+a. However, we believe that this is an unlikely case. At the time of review if gNB1-CU changes its prediction, then:

- gNB1-CU informs gNB1-DU of the cancellation of the earlier prediction over F1.

- It also informs gNB2-DU of the same over Xn interface, which forwards it to gNB2-DU over F1.

- Both gNB1-DU and gNB2-DU cancel their future CCO states and inform the respective gNB-CUs.

For Case 2, if gNB2-CU detects an issue between t0 and t0+p

In this case, we may assume that gNB1-DU and gNB2-DU has not yet applied the proposed configuration changes for the CCO states which had been derived in response to the predicted CCO issue at gNB1. The legacy steps may follow:

- gNB2-CU informs the gNB2-DU of the detected CCO issue.

- gNB2-DU determines and applies the new CCO state and informs gNB2-CU

- gNB2-CU informs gNB1-CU of currently detected CCO issue and the CCO state applied by the gNB2-DU in response to the CCO issue detected by gNB2-CU.

- gNB1-CU forwards the information received from the gNB2-CU to gNB1-DU

- gNB1-DU determines and applies the new CCO state in response to the latest information and informs the gNB1-CU.

After this, gNB1-CU reviews the earlier prediction, and if it is still valid for time t0+p, then it takes no further action. This implies that gNB1-DU and gNB2-DU will proceed to apply their respective future CCO states at t0+a. However, we believe that this is an unlikely case. At the time of review if gNB1-CU changes its prediction, then:

- gNB1-CU informs gNB1-DU of the cancellation of the earlier prediction over F1.

- It also informs gNB2-DU of the same over Xn interface, which forwards it to gNB2-DU over F1.

- Both gNB1-DU and gNB2-DU cancel their future CCO states and inform the respective gNB-CUs.

For Case 3, if gNBx-CU detects an issue between t0 and t0+p

In this case, we may assume that gNB1-DU and gNB2-DU has not yet applied the proposed configuration changes for the CCO states which had been derived in response to the predicted CCO issue at gNB1. The legacy steps may follow between gNBx and the neighbouring gNBs.

Case 3a: If the neighbouring gNB in this case is gNB2, then the following steps may take place:

- gNBx-CU informs the gNBx-DU of the detected CCO issue.

- gNBx-DU determines and applies the new CCO state and informs gNBx-CU

- gNBx-CU informs gNB2-CU of currently detected CCO issue and the CCO state applied by the gNBx-DU in response to the CCO issue detected by gNBx-CU.

- gNB2-CU forwards the information received from the gNBx-CU to gNB2-DU

- gNB2-DU determines and applies the new CCO state in response to the latest information and informs the gNB2-CU.

Since there is a pending future CCO state action at gNB2-DU based on the prediction of CCO issue by gNB1-CU, we propose that gNB2-CU informs gNB1-CU of the change in the CCO state of gNB2-DU. After this, gNB1-CU reviews the earlier prediction, and if it is still valid for time t0+p, then it takes no further action. This implies that gNB1-DU and gNB2-DU will proceed to apply their respective future CCO states at t0+a. At the time of review. if gNB1-CU changes its prediction, then the steps are followed as if a new CCO issue has been predicted, and the older CCO issue has been cancelled.

- gNB1-CU informs gNB1-DU of the cancellation of the earlier prediction over F1.

- It also informs gNB2-DU of the same over Xn interface, which forwards it to gNB2-DU over F1.

- Additionally, the change in prediction is treated the same as a new prediction of CCO issue at gNB1.

Case 3a: If the neighbouring gNB in this case is gNB1, then the following steps may take place:

- gNBx-CU informs the gNBx-DU of the detected CCO issue.

- gNBx-DU determines and applies the new CCO state and informs gNBx-CU

- gNBx-CU informs gNB1-CU of currently detected CCO issue and the CCO state applied by the gNBx-DU in response to the CCO issue detected by gNBx-CU.

- gNB1-CU forwards the information received from the gNBx-CU to gNB1-DU

- gNB1-DU determines and applies the new CCO state in response to the latest information and informs the gNB1-CU.

After this, gNB1-CU reviews the earlier prediction, and if it is still valid for time t0+p, then it takes no further action. This implies that gNB1-DU and gNB2-DU will proceed to apply their respective future CCO states at t0+a. However, we believe that this is an unlikely case. At the time of review. if gNB1-CU changes its prediction, then the steps are followed as if a new CCO issue has been predicted, and the older CCO issue has been cancelled.

- gNB1-CU informs gNB1-DU of the cancellation of the earlier prediction over F1.

- It also informs gNB2-DU of the same over Xn interface, which forwards it to gNB2-DU over F1.

- Additionally, the change in prediction is treated the same as a new prediction of CCO issue at gNB1.

We’ve presented some scenarios where multiple CCO issues occur where one of them is a legacy CCO issue which is detected, and the other is a predicted CCO issue. Similar conflicting scenarios may occur for multiple detected CCO issue or multiple predicted CCO issue.

Overall, we propose that the issue be tackled in the order in which they are detected or predicted to occur. And if a CCO state has been changed between the time of prediction and the time of action at the source gNB, then the gNB needs to check whether the prediction is still valid. And if a CCO state has changed between the time of prediction and the time of action at the neighbouring gNB, then the neighbouring gNB may decide on how best to handle the subsequent issues. However, this still doesn’t ensure that a race condition will not result due to gNBs continuously changing the CCO state in response to multiple detected/prediction issue.

Proposal 3	RAN3 to study mechanisms to ensure convergence of the CCO states to avoid race conditions resulting from coexistence of multiple detected/predicted CCO issues.

Consider an example of multiple gNBs, where gNB1 and gNB3 are source gNBs where CCO issue gets predicted, and gNB2 is the target gNB for both. If gNB2 receives predicted CCO issues from multiple gNBs (1 and 3), and both these issues are within 60s (the maximum prediction time) of each other, then gNB2 might try to make a coordinated decision about its future CCO state (if a future CCO state has not yet been applied to the gNB2 in response to either of these predicted CCO issues).

At time t0, gNB1 sends the CCO issue and future CCO coverage to gNB2. And the issue/future CCO state is expected to occur t0+30. And at time t0+10, gNB3 sends its CCO issue and the future CCO coverage to gNB2. And the issue/future CCO state is expected to occur at t0+40.

In this case, we believe that whatever decision is made by gNB2-DU about the future CCO state at t0 should be reevaluated at t0+10 to check whether any of the cells/beams impacted as reported by gNB3 is affected by the proposed future CCO state of gNB2. If not, then each of these issues may be treated “one at a time” as discussed in the preceding section. Else, gNB2-DU may derive a new future CCO state at t0+10, which possibly addresses both the CCO issues reported by gNB1 and gNB3.

Not taking coordinated effort to address multiple predicted issues might result in a ping-pong behavior (non-convergence), if a target gNB needs to handle multiple predicted CCO issues which occur within a short timeframe. In the above case, gNB2-DU would output a future CCO state to be applied around t0+30 (in response to future CCO state of gNB1), and another future CCO state to be applied around t0+40 (in response to future CCO state of gNB3). This might result in the first issue recurring at gNB1.

In order to mitigate a ping-pong behaviour, we propose that a gNB receiving multiple CCO issues within a short timeframe makes a coordinated effort to change its CCO state so as to minimize its own CCO state changes and still address the CCO state changes of multiple source gNBs.

Proposal 4	We propose that a gNB receiving multiple CCO issues within a short timeframe makes a coordinated effort to change its CCO state so as to minimize its own CCO state changes and still address the CCO state changes of multiple source gNBs.

# 3	Conclusion

Observation 1	It is possible that a new CCO issue may be detected/predicted at the source gNB or a neighbouring node or a third gNB between the time at which the CCO issue was predicted and the time at which CCO issue is predicted to manifest.

Proposal 1	We propose that the CCO state in response to the detected CCO issue overrides the actions in response to the predicted CCO state.

Proposal 2	We propose that the issue be tackled in the order in which they are detected or predicted to occur.

Proposal 3	RAN3 to study mechanisms to ensure convergence of the CCO states to avoid race conditions resulting from coexistence of multiple detected/predicted CCO issues.

Proposal 4	We propose that a gNB receiving multiple CCO issues within a short timeframe makes a coordinated effort to change its CCO state so as to minimize its own CCO state changes and still address the CCO state changes of multiple source gNBs.

# Reference

[1] TR 38.743

[2] TS 38.473