3GPP TSG-RAN WG3 #128	R3-253907

St Julian’s, Malta, May 19-23, 2025

Agenda Item:	10.2

Source:	Ericsson, Lenovo, Nokia, Samsung

Title:	(TP for BL CR to 38.473 for SON) MRO enhancements for LTM

Document for:	Discussion and Decision

# 1	Introduction

This TP is meant to capture the following agreements for MRO for LTM made during RAN3#128:

BFR shortly after successful LTM cell switch to the wrong beam(Case 1)

Use DU-CU ACCESS AND MOBILITY INDICATION message for target DU send recovery beam information to the CU. Only one recovery beam information included.

Additional Information with recovery beam information from target DU to CU:

- CU UE F1AP ID

- Failure type (for case 1, case 2) in an implicit way or explicit way

- beam failure indicator in an implicit way or explicit way

Use ACCESS AND MOBILITY INDICATION message for CU forward recovery beam information to the source DU. Only one recovery beam information included.

Additional Information with recovery beam information from target CU to DU:

- Source DU UE F1AP ID

- Failure type (for case 1, case 2) in an implicit way or explicit way

- beam failure indicator in an implicit way or explicit way

RACHless LTM cell switch failure due to wrong beam (Case 2)

Network-based solution for the LTM cell switch failure due to wrong beam using the same signaling flow as for Case1.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization.

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization.

Target DU needs to send the reconnect/re-established/recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

Work on stage3 semantic description for C-RNTI IE in ACCESS AND MOBILITY INDICATION message.

RAN3 supports network-based solution for out dated TA.

Reusing the same signaling flow as case1 and case2.

TA acquisition Type signalling is not needed based on the network based solution of out dated TA.

# Annex:	 TP to SON BL CR for 38.473

First change

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Beam Failure Recovery Information IE, the gNB-DU shall, if supported, consider that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the LTM Cell Switch Failure due to Wrong Beam Information IE, the gNB-DU shall, if supported, consider that an LTM Cell Switch Failure due to Wrong Beam has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the TA Information IE (name is FFS), the gNB-DU may take it into account for optimisation of Timing Advance related parameters for the UE identified by the gNB-DU UE F1AP ID IE.

#### 8.11.1.3	Abnormal Conditions

Not applicable.

Next change

### 8.11.2	DU-CU Access and Mobility Indication

#### 8.11.2.1	General

This procedure is initiated by the gNB-DU to send the Access and Mobility related Information to the gNB-CU.

The procedure uses non-UE-associated signalling.

#### 8.11.2.2	Successful Operation

Figure 8.11.2.2-1: DU-CU Access and Mobility Indication procedure. Successful operation

The DU-CU Access and Mobility Indication procedure is initiated by DU-CU ACCESS AND MOBILITY INDICATION message sent from the gNB-DU to the gNB-CU.

If the DU-CU ACCESS AND MOBILITY INDICATION message contains the DL LBT Failure Information List IE, the gNB-CU shall take it into account for optimisation of mobility parameters.

Interaction with the Access and Mobility Indication procedure:

If the Beam Failure Recovery Information IE or the LTM Cell Switch Failure due to Wrong Beam Information IE or the TA Information IE (name is FFS) is included in the DU-CU ACCESS AND MOBILITY INDICATION message, the gNB-CU shall, if supported, forward this information to the source gNB-DU via the ACCESS AND MOBILITY INDICATION message including the Beam Failure Recovery Information IE, the LTM Cell Switch Failure due to Wrong Beam Information IE and the TA Information IE (name is FFS) respectively.

#### 8.11.2.3	Abnormal Conditions

Not applicable.

Next change

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >Recovery TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used at Beam Failure Recovery, as defined in TS 38.331 [8]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| LTM Cell Switch Failure due to Wrong Beam Information | O |  |  |  | YES | ignore |
| >Target Beam Information | M |  | FFS | FFS if it corresponds to the target beam index or TCI state ID. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| TA Information (name is FFS) | O |  |  |  | YES | ignore |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value used at successful Random Access, as defined in TS 38.213 [31]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

Next change

#### 9.2.10.2	DU-CU ACCESS AND MOBILITY INDICATION

This message is sent by the gNB-DU to provide access and mobility information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL LBT Failure Information List |  | 0..1 |  |  | YES | ignore |
| > DL LBT Failure Information Item |  | 1 .. <maxnoofLBTFailureInformation> |  |  | - |  |
| >>DL LBT Failure Information | M |  | 9.3.1.327 |  | - |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >Recovery TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used at Beam Failure Recovery, as defined in TS 38.331 [8]. | - |  |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| LTM Cell Switch Failure due to Wrong Beam Information | O |  |  |  | YES | ignore |
| >Target Beam Information | M |  | FFS | FFS if it corresponds to the target beam index or TCI state ID | - |  |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| TA Information (name is FFS) | O |  |  |  | YES | ignore |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value used at successful Random Access, as defined in TS 38.213 [31]. | - |  |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLBTFailureInformation | Maximum no. of UEs for which LBT Failure Information is provided, the maximum value is 64. |

Next change

### 9.4.4	PDU Definitions

<SNIP>

F1U-PathFailure,

LTMResetInformation,

CuBeamFailureRecoveryInformation,

DuBeamFailureRecoveryInformation

<SNIP>

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-CuBeamFailureRecoveryInformation,

id-DuBeamFailureRecoveryInformation,

<SNIP>

-- **************************************************************

--

#### --  Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Access And Mobility Indication

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { AccessAndMobilityIndicationIEs} },

...

}

AccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-RAReportList				CRITICALITY ignore	TYPE RAReportList			PRESENCE optional }|

{ ID id-RLFReportInformationList				CRITICALITY ignore	TYPE RLFReportInformationList				PRESENCE optional }|

{ ID id-SuccessfulHOReportInformationList		CRITICALITY ignore	TYPE SuccessfulHOReportInformationList	PRESENCE optional }|

{ ID id-SuccessfulPSCellChangeReportInformationList	CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportInformationList		PRESENCE optional }|

{ ID id-CuBeamFailureRecoveryInformation			CRITICALITY ignore 	TYPE CuBeamFailureRecoveryInformation  		PRESENCE optional },

...

}

<SNIP>

-- **************************************************************

--

#### --  DU-CU Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DU-CU Access And Mobility Indication

--

-- **************************************************************

DUCUAccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DUCUAccessAndMobilityIndicationIEs} },

...

}

DUCUAccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory }|

{ ID id-DLLBTFailureInformationList				CRITICALITY ignore	TYPE DLLBTFailureInformationList		PRESENCE optional}|

{ ID id-DuBeamFailureRecoveryInformation		CRITICALITY ignore	TYPE DuBeamFailureRecoveryInformation	PRESENCE optional },

...

}

Next change

### 9.4.5	Information Element Definitions

<SNIP>

CuBeamFailureRecoveryInformation	::= SEQUENCE {

recoveryTCIState									OCTET STRING,

lTMgNB-DU-ID										GNB-DU-ID,

iE-Extensions		ProtocolExtensionContainer { { CuBeamFailureRecoveryInformation-ExtIEs} } OPTIONAL,

...

}

CuBeamFailureRecoveryInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<SNIP>

DuBeamFailureRecoveryInformation	::= SEQUENCE {

recoveryTCIState									OCTET STRING,

lTMgNB-CU-ID										GNB-CU-ID,

iE-Extensions		ProtocolExtensionContainer { { DuBeamFailureRecoveryInformation-ExtIEs} } OPTIONAL,

...

}

DuBeamFailureRecoveryInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<SNIP>

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-CuBeamFailureRecoveryInformation					ProtocolIE-ID ::= xxx

id-DuBeamFailureRecoveryInformation					ProtocolIE-ID ::= xxy

End of changes