3GPP TSG-RAN WG3 Meeting #128	R3-253975

Malta, MT, 19 – 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1531 | rev | 6 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.473) Introduction of Network Energy Saving Enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio Platforms, CATT, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-06-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR introduces the necessary specification changes to support the Network Energy Saving Enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UL WUS Configuration Information under the Served Cell Information IE. Based on the RAN3-127bis agreements, - introduce the choice structure under the Served Cell Information IE. - introduce a new indication for NES paging adaptation in UE Paging Capability over F1. Capture the RAN3-127bis agreement, - Agreement 3: The NES gNB-CU sends the indication to NES gNB-DU. The NES gNB-DU MAY go to OD-SIB 1 operation up to gNB-DU decision. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Network Energy Saving Enhancements cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.2, 8.2.4.2, 8.2.5.2, 8.7.1.2, 9.2.1.10, 9.3.1.10, 9.3.1.270, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1436 TS 38.470 CR 0161 TS 38.300 draft CR TS 38.420 CR 0049 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial version: R3-247858 Rev1: R3-247898 Add co-sources. Rev2: R3-250056 Update against the latest specification. Change to a single author. Rev3: R3-251566 Resubmission. Rev4: R3-252499 Merge the agreed TPs in R3-252361 and R3-252466. Add the ASN.1. Rev5: R3-253104 Resubmission. Rev6: R3-253975 Merge the agreed TP in R3-253857. |  |  |  |  |  |  |  |  |

Change Begins

Editor’s Note: The procedure, message and the IE names are FFS.

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the F1 SETUP REQUEST message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the On-demand SIB1 IE is included and set to “Provision” in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information indicated in the od-SIB1-Config IE for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

If the On-demand SIB1 IE is included and set to “Stop provision” in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, stop the coordination of on-demand SIB1 transmission as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the On-demand SIB1 IE is included and set to “Provision” in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information indicated in the od-SIB1-Config IE for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

If the On-demand SIB1 IE is included and set to “Stop provision” in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, stop the coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the On-Demand SIB1 Cell IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider the cell indicated by the NR CGI IE for On-demand SIB1 operation.

Editor’s note: FFS how to describe the behaviour relative to the On-Demand SIB1 Indicator IE.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

## 8.7	Paging procedures

### 8.7.1	Paging

#### 8.7.1.1	General

The purpose of the Paging procedure is used to provide the paging information to enable the gNB-DU to page a UE. The procedure uses non-UE associated signalling.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: Paging procedure. Successful operation.

The gNB-CU initiates the procedure by sending a PAGING message.

The Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it to determine the final paging cycle for the UE.

The Paging Priority IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 23.501 [21].

At the reception of the PAGING message, the gNB-DU shall perform paging of the UE in cells which belong to cells as indicated in the Paging Cell List IE.

The Paging Origin IE may be included in the PAGING message, and if present the gNB-DU shall transfer it to the UE.

The RAN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The CN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information for RRC INACTIVE IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The Paging Cause IE may be included in the PAGING message. If present the gNB-DU shall, if supported, send it to UE according to TS 38.331 [8].

The PEIPS Assistance Information IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [6].

The RedCap Indication IE may be included in the UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging of the RedCap UE or the eRedCap UE.

The NES Paging Adaptation Indication IE may be included in the UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging adaptation to the UE.

The Last Used Cell Indication IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, consider the cell identified by the NR CGI IE as the last used cell of the paged UE, and use it as specified in TS 38.331 [8].

The Recommended SSBs List IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, use it to send the paging message over the indicated SSB beams.

The PEI Subgrouping Support Indication IE may be included in the Paging Cell Item IEs IE in the PAGING message, and if present the gNB-DU shall, if supported, consider that the cell identified by the NR CGI IE is supported by the UE to receive the paging early indication as described in TS 38.300 [6] and TS 38.304 [24].

The UE Paging Capability IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, take it into account when paging the UE.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| On-Demand SIB1 Cell |  | 0..1 |  |  | YES | ignore |
| >NR CGI | M |  | 9.3.1.12 |  | - |  |
| >On-Demand SIB1 Indicator | M |  | ENUMERATED (start, stop, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [8]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| CHOICE on-demand SIB1 | O |  |  |  | YES | ignore |
| >Provision |  |  |  |  |  |  |
| >>On-demand SIB1 Config [FFS] | M |  | Octet String | Includes the od-SIB1-Config contained in the SIBxx message as defined in TS 38.331 [8]. | - |  |
| >Stop provision |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.270	UE Paging Capability

This IE provides the UE Paging Capability information needed for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| INACTIVE State PO-Determination | O |  | ENUMERATED(supported,…) | Corresponds to the inactiveStatePO-Determination contained in the UERadioPagingInformation IE defined in TS 38.331 [8]. | - |  |
| RedCap Indication | O |  | ENUMERATED(true,…) | Indicates that the paged UE is a Redcap UE or an eRedCap UE. | YES | ignore |
| NES Paging Adaptation Indication [FFS] | O |  | ENUMERATED(supported,…) | Corresponds to the [FFS] contained in the UERadioPagingInformation IE defined in TS 38.331 [8]. | YES | ignore |

Editor’s Note: FFS on the name and semantics description, pending to RAN2’s update.

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Broadcast-MRBs-Transport-Request-Item,

TAInformation-List,

NonIntegerDRXCycle,

AggregatedPosSRSResourceSetList,

F1U-PathFailure,

LTMResetInformation,

OnDemand-SIB1-Cell

FROM F1AP-IEs

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-OnDemand-SIB1-Cell,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE

--

-- **************************************************************

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE	TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List				CRITICALITY reject	TYPE	Cells-to-be-Activated-List							PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List			CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List						PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Add-List		CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Add-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Remove-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Remove-List			PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Update-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Update-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Barred-List					CRITICALITY ignore	TYPE	Cells-to-be-Barred-List								PRESENCE optional	}|

{ ID id-Protected-EUTRA-Resources-List			CRITICALITY reject	TYPE	Protected-EUTRA-Resources-List					PRESENCE optional	}|

{ ID id-Neighbour-Cell-Information-List			CRITICALITY ignore	TYPE	Neighbour-Cell-Information-List					PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE	Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping			CRITICALITY reject	TYPE	UL-BH-Non-UP-Traffic-Mapping						PRESENCE optional	}|

{ ID id-BAPAddress								CRITICALITY ignore  TYPE 	BAPAddress												PRESENCE optional }|

{ ID id-CCO-Assistance-Information				CRITICALITY ignore	TYPE 	CCO-Assistance-Information							PRESENCE optional	}|

{ ID id-CellsForSON-List						CRITICALITY ignore	TYPE	CellsForSON-List										PRESENCE optional }|

{ ID id-gNB-CU-Name								CRITICALITY ignore	TYPE	GNB-CU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-CU-Name					CRITICALITY ignore	TYPE	Extended-GNB-CU-Name									PRESENCE optional	}|

{ ID id-Cells-Allowed-to-be-Deactivated-List			CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List		PRESENCE optional	}|

{ ID id-OnDemand-SIB1-Cell						CRITICALITY ignore	TYPE	OnDemand-SIB1-Cell		PRESENCE optional	}

,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-XR-Bcast-Information,

id-MaxDataBurstVolume,

id-BarringExemptionforEmerCallInfo,

id-SIB17bis-message,

id-ReportingIntervalIMs,

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-OnDemandSIB1,

id-NESPagingAdaptationIndication,

maxNRARFCN,

maxnoofErrors,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

NeighbourNR-CellsForSON-Item ::= SEQUENCE {

nRCGI											NRCGI,

nR-ModeInfoRel16								NR-ModeInfoRel16												OPTIONAL,

sSB-PositionsInBurst							SSB-PositionsInBurst											OPTIONAL,

nRPRACHConfig									NRPRACHConfig													OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { NeighbourNR-CellsForSON-Item-ExtIEs} }	OPTIONAL,

...

}

NeighbourNR-CellsForSON-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NESPagingAdaptationIndication ::= ENUMERATED {true, ...}

NGRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {NGRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL

}

NGRANAllocationAndRetentionPriority-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- O

OffsetToPointA	::= INTEGER (0..2199,...)

OnDemandPRS-Info ::= SEQUENCE {

onDemandPRSRequestAllowed				BIT STRING (SIZE (16)),

allowedResourceSetPeriodicityValues		BIT STRING (SIZE (24))	OPTIONAL,

allowedPRSBandwidthValues				BIT STRING (SIZE (64))	OPTIONAL,

allowedResourceRepetitionFactorValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedResourceNumberOfSymbolsValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedCombSizeValues					BIT STRING (SIZE (8))	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { OnDemandPRS-Info-ExtIEs} } OPTIONAL,

...

}

OnDemandPRS-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

OnDemandSIB1 ::= CHOICE {

provision						OndemandSIB1Config,

stopProvison					NULL,

choice-extension				ProtocolIE-SingleContainer { { OnDemandSIB1-ExtIEs} }

}

OnDemandSIB1-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

OnDemand-SIB1-Cell ::= CHOICE {

nRCGI							NRCGI,

on-demandSIBIndindicator		ENUMERATED{start, stop, ...},

choice-extension				ProtocolIE-SingleContainer { { OnDemand-SIB1-Cell-ExtIEs} }

}

OnDemand-SIB1-Cell-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

OndemandSIB1Config  ::= OCTET STRING

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Served-Cell-Information ::= SEQUENCE {

nRCGI							NRCGI,

nRPCI							NRPCI,

fiveGS-TAC						FiveGS-TAC			OPTIONAL,

configured-EPS-TAC				Configured-EPS-TAC 		OPTIONAL,

servedPLMNs						ServedPLMNs-List,

nR-Mode-Info					NR-Mode-Info,

measurementTimingConfiguration	OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { {Served-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-Cell-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RANAC							CRITICALITY ignore	EXTENSION RANAC							PRESENCE optional }|

{	ID id-ExtendedServedPLMNs-List		CRITICALITY ignore	EXTENSION ExtendedServedPLMNs-List	PRESENCE optional }|

{	ID id-Cell-Direction				CRITICALITY ignore	EXTENSION Cell-Direction				PRESENCE optional }|

{	ID id-BPLMN-ID-Info-List			CRITICALITY ignore	EXTENSION BPLMN-ID-Info-List			PRESENCE optional }|

{	ID id-Cell-Type						CRITICALITY ignore	EXTENSION CellType						PRESENCE optional}|

{	ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{	ID id-AggressorgNBSetID				CRITICALITY ignore	EXTENSION AggressorgNBSetID				PRESENCE optional}|

{	ID id-VictimgNBSetID				CRITICALITY ignore	EXTENSION VictimgNBSetID				PRESENCE optional}|

{	ID id-IAB-Info-IAB-DU				CRITICALITY ignore	EXTENSION IAB-Info-IAB-DU				PRESENCE optional}|

{	ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst			PRESENCE optional }|

{	ID id-NRPRACHConfig					CRITICALITY ignore	EXTENSION NRPRACHConfig					PRESENCE optional }|

{	ID id-SFN-Offset					CRITICALITY ignore	EXTENSION SFN-Offset					PRESENCE optional }|

{	ID id-NPNBroadcastInformation		CRITICALITY reject 	EXTENSION NPNBroadcastInformation		PRESENCE optional }|

{	ID id-Supported-MBS-FSA-ID-List				CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List						PRESENCE optional }|

{	ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information	PRESENCE optional }|

{	ID id-ERedcap-Bcast-Information		CRITICALITY ignore	EXTENSION ERedcap-Bcast-Information	PRESENCE optional }|

{	ID id-XR-Bcast-Information			CRITICALITY ignore	EXTENSION XR-Bcast-Information			PRESENCE optional }|

{	ID id-BarringExemptionforEmerCallInfo				CRITICALITY ignore	EXTENSION BarringExemptionforEmerCallInfo				PRESENCE optional }|

{	ID id-OnDemandSIB1					CRITICALITY ignore	EXTENSION	OnDemandSIB1				PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

UEPagingCapability ::= SEQUENCE {

iNACTIVEStatePODetermination						ENUMERATED {supported, ...} 	OPTIONAL,

iE-Extension				ProtocolExtensionContainer { { UEPagingCapability-ExtIEs} } 		OPTIONAL,

...

}

UEPagingCapability-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RedCapIndication		CRITICALITY ignore 	EXTENSION RedCapIndication		PRESENCE optional }|

{ 	ID id-NESPagingAdaptationIndication	CRITICALITY ignore	EXTENSION NESPagingAdaptationIndication	PRESENCE optional	},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-TagIDPointer										ProtocolIE-ID ::= 853

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-OnDemandSIB1										ProtocolIE-ID ::= aaa

id-NESPagingAdaptationIndication					ProtocolIE-ID ::= bbb

id-OnDemand-SIB1-Cell								ProtocolIE-ID ::= ccc

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Change Ends