3GPP TSG-RAN3 Meeting #129-bis	R3-256528

Prague, CZ, 13 - 17 Oct 2025

Agenda item:	13.2

Source:	Xiaomi

Title:	Discussion on ISAC Architecture and General Aspects

Document for:	Other

1	Introduction

In this contribution, we’d like to discuss the RAN architecture and general aspects of R20 5GA ISAC according to the following SID in [1] based on the discussion progress in SA2 and RAN1.

Study the procedures, signaling between RAN and CN to support ISAC [RAN3]

Study network architecture for RAN node-based mono-static sensing for UAV sensing target use cases [RAN3]

- Applicability to RAN node bistatic sensing may be considered as part of this network architecture without additional architecture impacts.

- No inter-RAN node coordination will be studied.

- Coordination with SA2 as necessary.

2	Discussion

2.1 	 RAN architecture background

SA2 also has an ongoing Rel-20 study on Architecture Enhancement to support Integrated Sensing and Communication (FS_Sensing_ARC, SP-250401). The RAN3 network architecture study are dependent on the outcomes of the system architecture discussions in SA2. For RAN3, the main focus should be on RAN-CN connectivity and interface protocol design. Based on the latest version of the SA2 TR [2], the following aspects are identified as potential impacts to RAN3 study.

| SA2 discussion | Details | Example figure |
| --- | --- | --- |
| RAN-SF connectivity options (two main RAN-SF connectivity options has been discussed and captured in SA2) | Direct connectivity between RAN node (sensing entity) and sensing function (e.g., sol#1, sol#3, sol#18, sol#21, sol#22, sol#25,sol#34, ...) | (refer to sol#21) |
|  | Indirect connectivity between RAN node (sensing entity) and sensing function (e.g., sol#1, sol#3, sol#5, sol#7, sol#18, sol#21, sol#22, sol#25,...) | (refer to sol#7) |
| Control and processing split of sensing function | Some solutions (e.g., sol#3, sol#25, sol#26, sol#29, sol#34, sol#37) proposed to split the sensing function to sensing control function and sensing processing function. Some solutions (e.g.,e.g., sol#3, sol#22...) also mentioned to establish a data tunnel to transfer the sensing data. | (refer to sol#3) |

RAN-SF connectivity options have a direct impact on RAN3 network architecture and interface design. For the RAN node monostatic use case, a direct architecture appears more straightforward, as the UE is not involved and thus AMF participation is unnecessary. Since this is the study item phase, it is ok to begin analysis with both connectivity options from RAN3 point of view, but the final decision needs to be tightly coordinated with SA2.

Observation 1, direct architecture is more straightforward to support RAN node monostaic sensing.

Observation 2, the down-selection between direct and indirect architecture need coordination with SA2.

Proposal 1,  RAN3 starts the study on both direct and indirect architectures in the first meeting.

Regarding the control and processing split of the sensing function, the motivation of splitting sensing function is that sensing measurement data may require dedicated processing functions, especially given the potentially large size and volume of such data. This necessitates consideration of a dedicated data tunnel for transmitting sensing measurement results from the sensing entity to the processing function. Regardless of whether the function is split, the primary impact on RAN3 is whether a dedicated tunnel should be established for sensing measurement reporting. Currently, RAN1 is discussing sensing measurements, and possible report types are outlined in their offline discussion in [R1-2506480]. However, it remains unclear which measurement types will be reported. Without the knowledge of what measurement metrics are needed, it is also unclear about the data size of sensing data transmitted between RAN and CN, hence unclear whether a dedicated data tunnel is required or not. From architecture perspective, RAN3 can initially treat the SF as a unified entity, with the need on sensing function split and the dedicated data tunnels to be revisited once SA2 and RAN1 reach some progresses on the sensing metrics and its data size.

- Spectrum of delay, and/or Doppler, and/or angle, i.e., the powers or the amplitudes/phases are reported for at least a range of samples of delay, and/or Doppler, and/or angle - Intermediate sensing results per detected path, Delay, Doppler, angle, and power per path - Intermediate sensing results per detected point - Option 1: Position, velocity, and power per point - Option 2: Range, velocity, angle, and power per point -Final sensing results per detected object/target - Option 1: Position and velocity - Option 2: Range, velocity, and angle

Besides measurement type, the performance metrics are also under RAN1 discussion, for the first RAN3 meeting, we think RAN3 can wait for RAN1 progress on both measurement type and performance metrics.

Observation 3, The motivation for SF split and dedicated data tunnels is mainly due to the potentially large volume of sensing data. However, the necessity for such mechanisms depends on the measurement types and data size which are under discussion in RAN1.

Proposal 2, RAN3 starts the architecture study by treating the SF as a unified entity, and further discuss the need of supporting interface between RAN-SF for SF function split based on the progress in SA2 and RAN1.

Split Architecture related

As split architecture is not widely deployed in commercial networks, we think the support of split architecture can be de-prioritized or not supported in R20.

Proposal 3: RAN3 should consider split architecture as a low priority or not support it in Release 20.

2.2 	Interface and protocol options

Based on the SA2’s architecture options as discussed in section 2.1, we summarized the following logical architecture and interface protocol options from RAN3 perspective.

|  | Direct connectivity | Indirect connectivity |
| --- | --- | --- |
| Logical arch |  |  |
| New protocol |  |  |
| Reusing existing protocol(enhanced NG) | And/or |  |

Similar to Ambient IoT architecture discussion, no matter which architecture option is adopted, the protocol stack can be a new protocol or enhancing existing protocols. For sensing data reporting, using GTP-U tunnel for sensing data reporting may also be considered, so we list both NG-C and NG-U for the enhanced NG option.

Proposal 4, RAN3 agree to study both direct and indirect interface between RAN node and SF, and study the potential protocol including new protocol and enhanced NG protocol.

Interface management for direct connectivity

In case direct connectivity is supported, RAN3 needs to consider how to manage the interface management between RAN node and SF. The following options can be further considered:

Option 1, in case of new protocol between RAN node and SF

- Option 1-1, management by implementation via OAM

- Option 1-2, Nx setup, configuration update (with limited impacts)

Option 2, enhanced NG-C

- Handle the not applicable IEs in NG setup procedures (similar to the enhancement in A-IoT direct arch)

Option 3, enhanced NG-U

- the GTP-U tunnel can be establish or released via signalling

Proposal 5, In case of direct connectivity, RAN3 discuss how to manage the interface connection between RAN node and SF based on different protocol options.

2.3 	General aspects to support sensing

We think it would be beneficial for RAN3 has some common understandings on the sensing related terminologies and functionalities from RAN node perspective, in this section, we discussed the definitions and functionalities of the sensing RAN node.

In our view, a sensing RAN node should support the following aspects:

-  A sensing RAN node serves one or multiple sensing units (i.e., sensing TRP).

- A sensing TRP supports transmitting and/or receiving the sensing RS.

- A sensing RAN node may have sensing data processioning capabilities (whether to have this functionality should be decided by RAN1 depending on data metrics to be reported to SF)

Proposal 6, RAN3 agrees the following definitions and functionalities of a sensing RAN node

-  A sensing RAN node serves one or multiple sensing units (i.e., sensing TRP).

- A sensing TRP supports transmitting and/or receiving the sensing RS.

In the Annex, we provide a TP to TR on architecture and general aspects.

Proposal 7, RAN3 agree the TP in Annex.

3	Conclusion

In this contribution, we discussed the RAN architecture and general aspects and have the following proposals.

Architecture

Observation 1, direct architecture is more straightforward to support RAN node monostaic sensing.

Observation 2, the down-selection between direct and indirect architecture need coordination with SA2.

Proposal 1,  RAN3 starts the study on both direct and indirect architectures in the first meeting.

Observation 3, The motivation for SF split and dedicated data tunnels is mainly due to the potentially large volume of sensing data. However, the necessity for such mechanisms depends on the measurement types and data size which are under discussion in RAN1.

Proposal 2, RAN3 starts the architecture study by treating the SF as a unified entity, and further discuss the need of supporting interface between RAN-SF for SF function split based on the progress in SA2 and RAN1.

Proposal 3: RAN3 should consider split architecture as a low priority or not support it in Release 20.

Interface and protocol options

Proposal 4, RAN3 agree to study both direct and indirect interface between RAN node and SF, and study the potential protocol including new protocol and enhanced NG protocol.

Proposal 5, In case of direct connectivity, RAN3 discuss how to manage the interface connection between RAN node and SF based on different protocol options.

General aspects to support sensing

Proposal 6, RAN3 agrees the following definitions and functionalities of a sensing RAN node

-  A sensing RAN node serves one or multiple sensing units (i.e., sensing TRP).

- A sensing TRP supports transmitting and/or receiving the sensing RS.

Proposal 7, RAN3 agree the TP in Annex.

4	Reference

[1] RP-252819, “Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, China Telecom

[2] 3GPP TR 23.700-14, “Study on Integrated Sensing and Communication”, Sep. 2025.

5	Annex (TP for TR on architecture and general aspect)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

x.1 	 RAN architecture

The logical RAN architecture support the following terminology and functionalities:

Sensing RAN Node:	Hosts certain functions for Sensing, serves one or multiple sensing units (i.e., sensing TRP)

Sensing TRP:			A sensing TRP supports transmitting and/or receiving the sensing RS.

Sensing Function:	Hosts certain functions for Sensing as of the functional in CN.

x.1.1	Direct Architecture

Figure x.1.1-1 depicts a logical system architecture for direct connectivity.

Figure x.1.1-1 Direct architecture

Editor’s Note x1, The Xx interface can be the a new interface or NG-C or NG-U.

Figure x.1.1-2 shows the Protocol stack for direct connectivity:

Figure x.1.1-2. Protocol Stack for direct architecture

Editor’s Note x2, The XxAP can be  new protocol application layer or NGAP, the transport layer could be SCTP or GTP-U.

x.1.2	Indirect Architecture

Figure x.1.2-1 depicts a logical system architecture for indirect connectivity.

Figure x.1.2-1 Indirect architecture

Figure x.1.2-2 shows the Protocol stack for indirect connectivity with new protocol layer above NGAP:

Figure x.1.2-2. Protocol Stack for indirect architecture with new protocol layer

Figure x.1.2-3 shows the Protocol stack for indirect connectivity with sensing info within NGAP:

Figure x.1.2-3. Protocol Stack for direct architecture with sensing info within NGAP

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>